User’s Guide to the Gainestown Package Thermal Model Rev 0.0 (Flotherm V6.1)

The Gainestown package Flotherm 6.1 compact model is created to assist OEMs in developing their system level thermal solutions.  It is strongly recommended that users read this user’s guide before using the model.

# COMPONENTS OF THE MODEL

The model is provided as a library file. The model consists of the following components:

- Package block: Simulates the thermal behavior of the Gainestown package region. This block includes substrates and socket regions of the package.

- IHS Assembly: This assembly consists of 5 blocks namely IHS1 through IHS5.  These 5 blocks represent the integrated heat spreader (IHS) on the processor.   The OEM cooling solution must be attached to the top surface of the IHS to ensure proper cooling of the Gainestown processor package.

- Package Power block: This is a planar heat source called Power.  In the model, a 1W power source is provided as an illustration. Users should refer to the EMTS and apply the correct package power dissipation to this source before using this model in their simulations.

## MODELING ASSUMPTIONS

- The entire heat dissipation from all the components within the package is removed through the top of the IHS. The package block is modeled as a non-conducting block.

- Natural convection and radiation inside the package are not considered, because of their minimal impact to heat sink design.

- Internal components of the package are not included in the model and are simulated using the package keep out block to provide its general system behavior.

## HOW TO INSERT THE PACKAGE MODEL INTO YOUR SYSTEM MODEL

- Load your system level project into Flotherm Version 6.1 and click on “External”.

- From the Tool Bar select “Libraries” and set the library type to “Assemblies”

- Click on “Import” and select the library file GAINESTOWN-TTV-OEM-MODEL.library from the directory where you saved it.  Click “OK”.  The package assembly is loaded and is called GAINESTOWN-TTV-OEM-MODEL.  Click “Dismiss”.

- In the Project Manager screen, right click “Root Assembly”, select “Location” and the select “Library”.  Choose the GAINESTOWN-TTV-OEM-MODEL library, which should appear in the list of libraries.  Now click “Load”.

- The Gainestown package assembly should now be part of your project.  You may have to translate and re-orient the assembly to fit your system configuration.

- If you have any questions regarding this procedure, please contact your customer support representative at Flomerics.

## DISCLAIMERS

- The Gainestown package model is intended to be used to evaluate the system thermal solutions only.  The dimensions and thermal properties used in the model are based on the current best estimates.  These dimensions and thermal properties are subject to change.  Please refer to the EMTS for specification of case temperature and total power dissipation as a function of frequency.

- Users should disregard the temperature distribution within the package itself.

- It is strongly recommended that users re-mesh the model according to the accuracy requirements of their system level model.

- No boundary conditions are set on this model.  Users are expected to set the appropriate boundary conditions on the system level thermal design.

## SPECIFICATIONS OF THE MODEL

The model is designed for external system level thermal evaluations only.  After the package model has been inserted into the system model, the surface temperature at the center of the top surface of the IHS must be maintained at or below the temperature specification provided in the EMTS.

## IF YOU HAVE QUESTIONS

Any questions on model usage or other general questions related to the use of Flotherm software, should be directed to Flomerics the maker of Flotherm.  Questions about future versions of the Gainestown package model should be directed to Intel FAE to get appropriate assistance.