EECS 311: Analog Circuits

| SYLLABUS |  |
| --- | --- |
| LECTURE 1 |  |
| Topic |  |
|  | Class introduction Example analog circuit topics |
| Summary of Lecture Content |  |
|  | Briefly discuss applications of analog circuits, and using VCCS (voltage controlled current source), VCVS, CCVS, and CCCS to model analog circuits. |
| Textbook Reading |  |
|  | Chapter 1 |
| LECTURE 2 |  |
| Topic |  |
|  | Review topics: Analog vs. digital signals KCL, KVL Amplifier introduction |
| Summary of Lecture Content |  |
|  | Look at different signal types (i.e. periodic/aperiodic, analog/digital, etc), and different ways of representing these signals. Reviewed KVL/KCL circuit analysis techniques. Before the end of class, begin discussion of amplifiers. Show that by using a transformer one can achieve a voltage gain at the expense of current throughput. A transformer cannot give power gain. |
| Textbook Reading |  |
|  | Chapter 11 |
| LECTURE 3 |  |
| Topic |  |
|  | Thevenin/Norton equivalents Source absorption Miller’s theorem Two-port networks Complex numbers & frequency Transfer function & Laplace transform Frequency response & Bode plots |
| Summary of Lecture Content |  |
|  | Begin by reviewing Thevenin and Norton equivalent circuits. Next, discuss source absorption, which allows the replacement of a dependent source with an impedance if the current or voltage it is dependent on is driving the source. Miller’s theorem allows the replacement of a floating impedance with 2 equivalent impendances to ground if the relating voltages are directly proportional to each other. Look at two-port networks. Review complex numbers and frequency response. |
| LECTURE 4 |  |
| Topic |  |
|  | Filters Op-amps (ideal) Inverting, non-inverting, and summing amplifiers |
| Summary of Lecture Content |  |
|  | Begin by looking at different types and filters and their transfer functions (bandpass, all pass, complicated transfer functions). Show how the Q factor affects the shape of the filter response. Also begin op-amps, but look at only the ideal characteristics. Ideal differential op-amps reject common-mode inputs and thus have great noise immunity. |
| Textbook Reading |  |
|  | Chapter 12, Section 1 - 7 |
| LECTURE 5 |  |
| Topic |  |
|  | Active filters |
| Summary of Lecture Content |  |
|  | Look at op-amp circuits with capacitors. A capacitor limits the frequency response of the circuit and leaves a highpass or lowpass filter. |
| LECTURE 6 |  |
| Topic |  |
|  | Op-amps (non-ideal characteristics) |
| Summary of Lecture Content |  |
|  | Look at the realistic, non-ideal characteristics of op-amps. These non-idealities include leakage currents, finite open-loop gain, finitie input and output resistances, finite bandwidth, offset voltages, and finite common-mode rejection. |
| LECTURE 7 |  |
| Topic |  |
|  | Continue op-amp non-idealities |
| Summary of Lecture Content |  |
|  | Continue discussion of non-ideal op-amp characteristics. Topics included differential amps (common-mode input resistance, differential-mode input resistance), frequency dependence (3dB freq, gain-bandwidth product, unity-gain freq, etc.), and slew rate. |
| LECTURE 8 |  |
| Topic |  |
|  | Multi-stage (cascade) amplifiers Op-amp modeling (VCVS) Distortion Switched-cap integrators |
| Summary of Lecture Content |  |
|  | By cascading amplifiers in series one can achieve a large gain. The high-frequency roll off is also affected. Op-amps can be modeled using a VCVS (voltage controlled voltage source). We looked at distortion such as clipping (voltage-limiting) and slewing (current-limiting). Resistors are not very well controlled in integrated circuits. Another method for implementing resistors is by using switched capacitances. |
| Homework |  |
|  | Homework 1: Chapter 1, Problem 26 Chapter 10, Problems 11, 23, 60 Chapter 11, Problem 21 |
| LECTURE 9 |  |
| Topic |  |
|  | Semiconductor properties MOSFET intro |
| Summary of Lecture Content |  |
|  | Examine the properties of semiconductors (electron/hole mobility, controlled resistivity through doping, etc.) that allow construction of devices such as diodes, BJTs, and MOSFETs. We also looked at the different currents present in a PN junction diode. Begin looking at MOSFETs and their different regions of operation (linear, saturation, and cutoff), and the drain current in each region. |
| Textbook Reading |  |
|  | Chapter 2 Chapter 4, Section 1 - 12 |
| LECTURE 10 |  |
| Topic |  |
|  | Diode equation Ideal diodes Zener effect (avalanche) Diode circuits (half- and full-wave rectifiers, bridge) Super diode (op-amp circuit) |
| Summary of Lecture Content |  |
|  | Look at the ideal diode as a circuit element and some of its applications. The zener effect causes a diode to have a large reverse current when the applied voltage is beyond the breakdown threshold. A diode and a capacitor can be used together in a circuit to create rectifiers (circuits that convert AC to DC). |
| Textbook Reading |  |
|  | Chapter 3, Section 1-8 |
| LECTURE 11 |  |
| Topic |  |
|  | Continue diode circuits (peak detectors, clampers, precision clampers, voltage doublers, voltage limiters) Basic diode physics Diode capacitance |
| Summary of Lecture Content |  |
|  | Look at diode physics in terms of a PN semiconductor junction, and also talk about parasitic capacitance of a diode. |
| LECTURE 12 |  |
| Topic |  |
|  | Start BJTs: Basic physics Operation regions Large signal model BJT Amplifiers |
| Summary of Lecture Content |  |
|  | BJTs (Bipolar Junction Transistors) allow the building of very important voltage or current amplifier circuits. Most often a BJT will be operating in the Forward Active region (rather than reverse active, saturation, or cutoff). A BJT can be thought of as two back-to-back PN junction diodes. |
| Textbook Reading |  |
|  | Chapter 5, Section 1 - 14 |
| LECTURE 13 |  |
| Topic |  |
|  | Small signal BJT model (hybrid-pi) Proper BJT biasing Common emitter circuits |
| Summary of Lecture Content |  |
|  | We can use simple circuit elements (a dependent source, resistors, capacitors) to model the behavior of a BJT in an amplifier circuit. Common emitter circuits are good amplifiers. |
| Textbook Reading |  |
|  | Chapter 13, Section 1 - 6 Chapter 14 |
| LECTURE 14 |  |
| Topic |  |
|  | Continue BJT amplifier circuits |
| Summary of Lecture Content |  |
|  | Show that common-base and common-collector configurations can be used for unity gain current or voltage follower circuits. |
| Homework |  |
|  | Homework 2: Chapter 11 Problems 59, 62, 89, 104, 108 |
| LECTURE 15 |  |
| Topic |  |
|  | Continue BJT amplifier circuits |
| LECTURE 16 |  |
| Topic |  |
|  | Quiz 1 |
| LECTURE 17 |  |
| Topic |  |
|  | Review Quiz 1 solutions |
| LECTURE 18 |  |
| Topic |  |
|  | BJT Capacitances Frequency dependent small-signal BJT model Review for Midterm Exam 1 |
| Summary of Lecture Content |  |
|  | Discuss the internal capacitances associated with BJTs, specifically the base cap, and the emitter junction cap. These caps give rise to the frequency dependent small-signal model, which limits the high-frequency response of a BJT. |
| Textbook Reading |  |
|  | Chapter 13, Section 9 - 10 Chapter 17, Section 1 - 9 |
| LECTURE 19 |  |
| Topic |  |
|  | Midterm Exam 1 |
| LECTURE 20 |  |
| Topic |  |
|  | BJT Capacitance Frequency response Open circuit time constant method (OCTC) |
| Summary of Lecture Content |  |
|  | The response of a BJT amplifier rolls off at some high frequency due to internal BJT capacitances. The OCTC method is an easy way to approximate the high corner frequency (making several assumptions). |
| LECTURE 21 |  |
| Topic |  |
|  | Review Midterm Exam 1 solutions Short circuit time constant method (SCTC) |
| Summary of Lecture Content |  |
|  | The SCTC is used for estimating the low corner frequency for a given circuit. |
| LECTURE 22 |  |
| Topic |  |
|  | Small signal analysis for MOSFET amplifiers |
| Summary of Lecture Content |  |
|  | Discuss derivation of small signal equations for MOSFETs and different biasing configurations for MOSFET amplifiers. |
| LECTURE 23 |  |
| Topic |  |
|  | Current mirrors |
| Summary of Lecture Content |  |
|  | Show that precision currents are frequently needed in integrated circuits, and that current mirrors are an effective and inexpensive way of accomplishing this. |
| Homework |  |
|  | Homework 3: Chapter 14 Problems 23, 27, 34, 56, 61, 68 |
| LECTURE 24 |  |
| Topic |  |
|  | Active loads with MOSFET current mirrors |
| LECTURE 25 |  |
| Topic |  |
|  | MOSFET parasitic capacitances |
| Summary of Lecture Content |  |
|  | Discuss the many different capacitances associated with MOSFETs: gate oxide cap, gate-to-source, gate-to-drain, etc. The values change depending upon which region the MOSFET is operating in (linear, saturation, cutoff). Discuss the effects of these capacitors on the frequency response of MOSFET circuits, and the unity gain cutoff frequency. |
| LECTURE 26 |  |
| Topic |  |
|  | Begin Multi-stage amplifiers |
| Summary of Lecture Content |  |
|  | Show how to combine several single-stage amplifiers to form an amplifier with better features (higher gain, more control over the bandwidth, high input resistance, low output resistance, etc.). Show how to either bias each stage individually and use AC coupling caps to block the DC components of each stage from the others, or build a DC coupled amplifier. |
| Textbook Reading |  |
|  | Chapter 15, Section 1 - 2, 5 - 6 Chapter 16, Section 2 - 3 |
| LECTURE 27 |  |
| Topic |  |
|  | Review Lab 5: single-stage amplifier design project (refer to attached 311-Lab5.pdf and 311-Lab5-Grading.pdf) Continue multi-stage amplifier discussion |
| LECTURE 28 |  |
| Topic |  |
|  | Differential Amplifiers |
| Summary of Lecture Content |  |
|  | Make a differential amplifier by taking the output as a difference of two signals. Differential amps have many advantages over single ended amps: Smaller footprint, no need for coupling/bypass caps, do not need to zero out the DC, less sensitivity to noise, etc. Design these amps to minimize common-mode gain and maximize differential-mode gain. |
| LECTURE 29 |  |
| Topic |  |
|  | Continue differential amplifier circuits |
| Homework |  |
|  | Homework 4: Chapter 15, Problem 167 Chapter 17, Problems 21, 22, 58 |
| LECTURE 30 |  |
| Topic |  |
|  | Continue differential amps |
| Summary of Lecture Content |  |
|  | Show how to make the shared emitter resistor as large as possible to maximize differential gain. Cover CMRR (common-mode rejection ratio) for differential amps. |
| LECTURE 31 |  |
| Topic |  |
|  | Midterm Exam 2 |
| LECTURE 32 |  |
| Topic |  |
|  | Review Midterm Exam 2 solutions |
| LECTURE 33 |  |
| Topic |  |
|  | Begin Current Sources |
| Summary of Lecture Content |  |
|  | Look at simple, 2-transistor current mirrors. A reference current in the first transistor induces an equal output current in the second transistor (the output current can be scaled with the device width). Can improve the accuracy of the mirror by adding additional transistors. Many different configurations are possible: simple, compound, wilson, cascode, widlar. |
| LECTURE 34 |  |
| Topic |  |
|  | Continue current source circuits |
| LECTURE 35 |  |
| Topic |  |
|  | Continue current source circuits |
| Summary of Lecture Content |  |
|  | For a given reference current, output current should be as close as possible to the reference. In DC terms, the current mirror should function satisfactorily for as small a voltage as possible. |
| LECTURE 36 |  |
| Topic |  |
|  | Current mirror design Different configurations: Widlar, Cascode, Wilson |
| Summary of Lecture Content |  |
|  | Finish discussion of current sources. Compare a normal cascode mirror with a low-voltage cascode source. Look at another structure that is independent of the supply voltage and is only dependent on N, the transistor size ratio. |
| LECTURE 37 |  |
| Topic |  |
|  | Active load current sources |
| Summary of Lecture Content |  |
|  | Show how to use current mirrors as active loads (resistors). This method is used often in integrated circuits, since actual resistors are bulky and difficult to control. |
| Homework |  |
|  | Homework 5: Chapter 15, Problems 50, 53, 61 Chapter 16, Problems 16, 34, 39 |
| LECTURE 38 |  |
| Topic |  |
|  | Course Review |
| Summary of Lecture Content |  |
|  | Op-amps - Theory, ideal, non-ideal, frequency response, applications Devices - diodes, FETs, BJTs Amplifiers - CB/CE/CC, CS/CD/CG, multi-stage, differential, frequency response, SCTC, OCTC, miller effect Current Sources - simple, compound, wilson, widlar, supply independent, low voltage, metrics |
| LECTURE 39 |  |
| Topic |  |
|  | Final Exam Review Problems |