EECS370: Introduction to Computer Organization

| SYLLABUS |  |
| --- | --- |
| LECTURE 1 |  |
| Topic |  |
|  | Introduction to class and to the history of computer architecture |
| Summary of Lecture Content |  |
|  | This topic introduces the class and professor to the students, and also explains the format of the course. The remainder of the time is spent introducing the concept of computational devices, showing how the concept has evolved. We begin by showing a picture of “Astrolavos” (an ancient cogwheel-driven device used to compute moon phases) and move on through progressively more modern computers, including the “Analytical Engine,” the ENIAC, and die-scans of modern Intel processors. |
| Textbook Reading |  |
|  | Chapter 1. |
| LECTURE 2 |  |
| Topic |  |
|  | Instruction Set Architectures: registers, memory addressing modes |
| Summary of Lecture Content |  |
|  | This topic, spanning four lectures, introduces the concept of instruction set architectures (ISAs). Basic processor components, such as ALUs and register files, are introduced and their purposes explained. Instruction set design is explored in terms of what instructions should be included, how they should be encoded, and the value of placing restrictions on instruction sets (such as creating a load-store architecture). Direct, indirect, register indirect, base + displacement, and PC-relative addressing modes are explained, and their relative advantages and disadvantages are shown with the use of examples. |
| Textbook Reading |  |
|  | Chapter 3 |
| Homework |  |
|  | Introduction to a programming assignment, dubbed ‘Project 1’. Project 1 consists of two major parts, an assembler and a simulator. The assembler takes a file containing assembly code in a textual format, and outputs a numeric representation of the instructions. The simulator then reads in the assembler’s output, parsing the numeric instructions and simulating the program accordingly. At the end of each cycle, the value of each register is outputted, along with the contents of the memory. This project forms the basis of the next two assigned projects. Project 1 This project is intended to help the students understand the instructions of a very simple assembly language and how to assemble programs into machine language. This project has three parts. In the first part, the student will write a program to take an assembly- language program and produce the corresponding machine language output. In the second part, the student will write a behavioral simulator for the resulting machine code. In the third part, the student will write a short assembly-language program to multiply two numbers. The assembly language is simple, limited to only 8 instructions. Following is the format for one of the instructions (add). Bits 24-22 represent the opcode (000) Bits 21-19 represent the first operand, register A Bits 18-16 represent the second operand, register B Bits 15-3 unused (should all be 0) Bits 2-0 represent the destination register. Action: The value stored in register A is added to the value stored in register B, and the result thereof is stored in the destination register. |
| LECTURE 3 |  |
| Topic |  |
|  | Instruction set architecture: LC2K3 and MIPS instruction sets, instruction encoding, number representation. |
| Summary of Lecture Content |  |
|  | Two ISAs are introduced: MIPS, and a fictional ISA known as LC2, which is a simplified ISA used in course projects. Binary instruction encoding methods are developed, looking at issues such as assigning opcodes to instructions, including immediate values, and using two’s complement to represent negative immediate values. |
| Textbook Reading |  |
|  | Appendix A, 3.1-3.4 |
| LECTURE 4 |  |
| Topic |  |
|  | Instruction set architecture: More MIPS assembly, function calling conventions, caller/callee save registers. |
| Summary of Lecture Content |  |
|  | This lecture explores some of the more complex MIPS instructions, such as conditional branches. The rest of the lecture is spent on function calling conventions, and introducing the concept of caller- vs. callee-saved registers. |
| Textbook Reading |  |
|  | Chapter 3.5-3.6, 3.10 |
| Homework |  |
|  | A traditional (non-programming) assignment, testing students’ understandings of ISAs. Students are asked to trace an assembly-language program’s execution, and explain its function. Another typical question would deal with placement of variables in an object file. |
| LECTURE 5 |  |
| Topic |  |
|  | Instruction set architecture: compilers, assemblers, linkers, object code format, Java Virtual Machine structures. |
| Summary of Lecture Content |  |
|  | Thus lecture covers the roles of the compiler, assembler, linker, and loader. Students are introduced to the format of an intermediate object file, including the workings of symbol and relocation tables. The Java VM is covered briefly, mainly to introduce a different (stack-based) style of execution. |
| Textbook Reading |  |
|  | Chapter: 3.9, 3.12 |
| LECTURE 6 |  |
| Topic |  |
|  | Basic Processor Design: Combinational Logic |
| Summary of Lecture Content |  |
|  | This lecture is a review of combinational logic. For some students this serves as a first introduction to combinational logic design. The basic logic gates are introduced, and circuit evaluation is explained. Additionally, some higher level abstractions such as MUXes and decoders are introduced, and at the end of the lecture timing diagrams are covered. |
| Textbook Reading |  |
|  | Appendix B.1-B.3 |
| LECTURE 7 |  |
| Topic |  |
|  | Basic Processor Design: Sequential Logic |
| Summary of Lecture Content |  |
|  | This lecture focuses on sequential logic, building upon the first lecture to introduce the concept of a clock, flip- flops, and latches. These sequential logic circuits are then developed into primitive components of a processor, such as a memory cell. |
| Textbook Reading |  |
|  | Appendix B.4-B.9 |
| Homework |  |
|  | See Attachment: 370-HW1-Sol |
| LECTURE 8 |  |
| Topic |  |
|  | Basic Processor Design: ALU Design |
| Summary of Lecture Content |  |
|  | This lecture deals exclusively with ALU design. Ripple carry and carry look- ahead adders are the focus of the lecture, but representation of floating point numbers is also covered. The IEEE 754 format is presented, and methods for adding and multiplying numbers in IEEE 754, as well as for converting between decimal and IEEE 754, are given. |
| Textbook Reading |  |
|  | Chapter 4 |
| Homework |  |
|  | There is one traditional homework assignment and one programming assignment for this topic. The homework assignment asks questions such as determining the number of bits of ROM required to implement a particular FSM, creating various logic circuits, and a few questions on the representation of numbers in IEEE 754 format. See Attachment: 370-HW2-Sol The programming assignment (‘Project 2’) asks the students to take their first project, and change it to emulate a finite state machine. A datapath for the fictious LC2 processor is presented, with a single bus connecting all components. The execution of instructions must therefore be broken down into indidual ‘states’, such as fetching the instruction, decoding the instruction, and possibly using multiple states for the execution of said instruction. ‘Project 2’ still functions as a simulator for the output of Project 1’s assembler. Project 2: This project is intended to help the student understand how procedure calls work in assembly language and to give the student experience designing and optimizing a finite-state machine for a simple computer. This project has two parts. In the first part, the student will write an assembly-language program to recursively compute combination(n,r) (i.e. n choose r). In the second part, the student will simulate the computer at a lower, more-detailed level. Essentially, this will mean describing a finite- state machine for the LC-2K in C. Project 1 described the behavior (architecture) of the LC-2K. In this project, students will describe an implementation of the LC-2K3. For this, one will need knowledge of the registers present in the LC-2K3. The register-level diagram of the LC-2K3 is posted on the class web site. This details the availability and interconnection of registers and functional components, such as the ALU and memory access mechanisms. |
| LECTURE 9 |  |
| Topic |  |
|  | Basic Processor Design: Finite State Machines |
| Summary of Lecture Content |  |
|  | This lecture ties the previous three lectures on Basic Processor Design together, using combinational and sequential logic to design a finite state machine. ROMs are introduced, and are used to hold the state transition information for the example FSM that is built up in lecture, demonstrating the control logic of a vending machine. |
| Textbook Reading |  |
|  | Appendix B.6 |
| LECTURE 10 |  |
| Topic |  |
|  | Single Cycle Data Path |
| Summary of Lecture Content |  |
|  | This lecture covers the single- cycle datapath of the fictitious LC2 processor. We use basic building blocks (MUXes, decoders, ALUs, register files) to construct a datapath, and show how instructions move through this datapath. A datapath capable of handling all instructions other than branches and jumps is given at the beginning of the lecture, followed by traces of adds, loads, and other basic instructions. We show how MUXes and control logic are used to control the basic components in the datapath, and then discuss the problems introduced by a conditional branch instruction with the current datapath. The implementation is then revised to handle the addition of instructions implementing jumps and conditional branches. |
| Textbook Reading |  |
|  | Chapter 5.1-5.3 |
| LECTURE 11 |  |
| Topic |  |
|  | EXAM 1 (See Attachments: 370-Exam1 and 370-Exam1-Sol) |
| LECTURE 12 |  |
| Topic |  |
|  | Multi-cycle datapath |
| Summary of Lecture Content |  |
|  | This lecture works the datapath of the first into a multi-cycle datapath. Various components are removed, and a FSM is built up to indicate a control module for our multi-cycle datapath. Instruction by instruction, the FSM’s ROM is incremented to provide control of the datapath’s ALU, register file, memory, and MUXes. At the end of the lecture, the control rom is sufficient for a FSM handling all instructions save for JALR (a jump instruction). The implementation of JALR as a sequence of control parameters is left to the student as an exercise. |
| Textbook Reading |  |
|  | Chapter 5.4 |
| LECTURE 13 |  |
| Topic |  |
|  | Introduction to Pipelining |
| Summary of Lecture Content |  |
|  | This lecture provides an introduction to pipelining. As an example, we use a drive-through service at a fast-food restaurant. We show how processes naturally decompose into stages, such as ordering, paying, picking up food. The main concept of the first lecture is that through pipelining, we can decrease our CPI and increase processor efficiency. We present a very basic pipelined version of the LC2’s datapath, and show the execution of a few instructions. |
| Textbook Reading |  |
|  | Chapter 2, 6.1-6.3 |
| Homework |  |
|  | ‘Project 3’ involves once again modifying ‘Project 1’, this time to represent a simulator of a processor with a pipelined datapath. The datapath is well-defined, and students should be able to draw upon the work done in lecture to develop logic for detecting and handling hazards. Project 3: This project is intended to help students understand in detail how a pipelined implementation works. The student will write a cycle-accurate behavioral simulator for a pipelined implementation of the LC-2, complete with data forwarding and simple branch prediction. The student's simulator must be pipelined. This means that the work of carrying out an instruction should be done in different stages of the pipeline as done in the textbook, and that the execution of multiple instructions should be overlapped. Multiple constraints are placed on the functioning of the simulator. For example, the instruction decode stage should be the ONLY stage that reads the register file; the other stages must get the register values from a pipeline register. A C-structure is given to students, which should hold the state of the processor at any given time. This structure includes fields for registers, instruction and data memory, as well as pipeline registers. The student's simulator must update a copy of the present state structure at each stage of the pipeline, and at the end of the cycle, overwrite the previous state with the new state. In so doing, students must account for both data and control hazards, and implement data forwarding where necessary. |
| LECTURE 14 |  |
| Topic |  |
|  | Pipelined Data Hazards and Forwarding |
| Summary of Lecture Content |  |
|  | This lecture introduces the concept of data hazards. We present multiple methods for handling data hazards, including avoidance, runtime detect and stall, and data forwarding. The disadvantages of avoidance and ‘detect and stall’ are explained, along with the workings of data forwarding. The rest of the lecture is spent modifying the LC2 datapath to incorporate data forwarding and hazard detection. |
| Textbook Reading |  |
|  | Chapter 6.4-6.5 |
| LECTURE 15 |  |
| Topic |  |
|  | Pipelining: Control Hazards |
| Summary of Lecture Content |  |
|  | This lecture on the topic focuses on control hazards. Once again, the downsides of avoidance and ‘detect and stall’ are shown, and the squash and speculate method is proposed as an alternative. The LC2 pipelined datapath developed in the second lecture is once again modified to handle control hazards, using a squash and speculate method. For simplicity, we assume a “predict not taken” strategy, but explain various other branch prediction methods. |
| Textbook Reading |  |
|  | Chapter 6.6-6.7 |
| Homework |  |
|  | This homework assignment involves drawing execution diagrams of programs on a pipelined datapath. Questions are also asked on performance of programs on a given pipelined datapath. See Attachment: 370-HW3 and 370-HW3-Sol (which gives the solutions) |
| LECTURE 16 |  |
| Topic |  |
|  | Summary of Pipelining: Pipelining Performance |
| Summary of Lecture Content |  |
|  | This lecture summarizes pipelining, providing a basic recap of how to handle data- and control-hazards. In addition, methods for determining the CPI of a pipelined datapath are presented. At the end of the lecture, a student should be capable of determining the CPI for a given program’s execution. |
| Textbook Reading |  |
|  | Review Chapters 2 and 6 |
| LECTURE 17 |  |
| Topic |  |
|  | Advanced Pipelining |
| Summary of Lecture Content |  |
|  | This lecture introduces advanced pipelining techniques, such as superscalar pipelining and out-of-order execution of instructions. New techniques for improving branch predictions are discussed, along with information about methods employed by current microprocessors. |
| Textbook Reading |  |
|  | Chapter 6.8 |
| LECTURE 18 |  |
| Topic |  |
|  | Introduction to Memory Systems and Caches |
| Summary of Lecture Content |  |
|  | This lecture introduces the concept of caching, and introduces terminoligy necessary for the following lectures. Specifically, content-addressable memory and how to break down addresses into a tag plus an offset are introduced. The lecture concludes with a discussion of replacement policies used in choosing what to evict from a cache. |
| Textbook Reading |  |
|  | Chapter 7.1-7.3, 7.5 |
| LECTURE 19 |  |
| Topic |  |
|  | Caches: Block size and stores |
| Summary of Lecture Content |  |
|  | This lecture introduces the concept of a block-size, taking advantage of spatial locality in data accesses. In order to observe the benefits of blocking, methods to evaluate a cache’s performance (in terms of hit rate and miss penalties) are also explained. The lecture concludes by examining the potential benefits of write-back vs. write-through caches. |
| LECTURE 20 |  |
| Topic |  |
|  | Caches: Set associativity |
| Summary of Lecture Content |  |
|  | This lecture focuses on improving cache performance. Direct-mapped caches are introduced, along with their drawbacks. This is primarily to provide a basis for the discussion of set-associative caches, which are the focus of the rest of the lecture. At the end of the lecture, the possibility of introducing caches into the LC2 architecture are discussed, along with the advantages and disadvantages of maintaining separate instruction and data caches. |
| LECTURE 21 |  |
| Topic |  |
|  | Cache Performance |
| Summary of Lecture Content |  |
|  | This lecture is focused exclusively on cache performance. Sample programs are given, and the contents of the cache after each instruction are shown. Additionally, example problems are solved where performance is evaluated given a certain hit ratio and miss penalty. |
| LECTURE 22 |  |
| Topic |  |
|  | EXAM 2 See Attachment: 370-Exam2 (and 370-Exam2-Sol for the solutions) |
| LECTURE 23 |  |
| Topic |  |
|  | Virtual Memory: Address translation, page tables |
| Summary of Lecture Content |  |
|  | This lecture introduces virtual memory, building upon the concepts of caching taught in the previous lectures. Virtual memory is introduced initially as treating main memory like a cache - requests for pages not currently in memory must be brought in from disk, and something might need to be evicted. We then introduce the concept of a page table, and extend this concept to include hierarchal page tables. The first lecture closes with the introduction of the Translation Lookaside Buffer (TLB), discussing its function and where in the processor pipeline the TLB should be placed. |
| Textbook Reading |  |
|  | Chapter 7.4 |
| Homework |  |
|  | A programming assignment, Project 4, asks students to integrate a cache into the “Project 1’ simulator. As data is requested by the assembly-language program, the student’s simulator will output messages whenever a page of memory is either brought into or evicted from the cache. The size of the cache, as well as the associativity and block-size, are specified at program startup. Project 4: The purpose of this project is to teach students about cache design and how a caching processor generates and services address references. In this project, students will simulate a CPU cache (unified instruction/data) and integrate the cache into a Project 1 (behavioral) simulator. As the processor simulator executes an assembly- language program, it will access instructions and data. These accesses will be serviced by the cache, which will transfer data to/from memory as needed. The central part of this project is to write a function that implements a flexible cache simulator. The cache function will be used by the processor simulation when the processor accesses addresses. The cache function should be able to simulate a variety of cache configurations. The set associativity, block size, and cache size will all be specified at runtime. Each address reference during program simulation should be passed to the cache simulator. The cache simulator keeps track of what blocks are currently in the cache and what state they are in (e.g. dirty, valid, etc.). To service the address reference, the cache simulator may need to write back a dirty cache block to memory, then it may need to read a block into the cache from memory. After these possible steps, the cache simulator should return the data to the processor (for read accesses) or write the data to the cache (for write accesses). Each of these data transfers will be logged by calling a provided function. |
| LECTURE 24 |  |
| Topic |  |
|  | Memory Systems: Virtual Memory |
| Summary of Lecture Content |  |
|  | This lecture discusses performance of VM systems, including page replacement policies such as LRU and clock. Caching is re-examined, in terms of whether it is best to use physically-addressed or virtually-addressed caches. We look at the pros and cons of each, examining how much work must be done on context switches, and how much time a normal access requires. The lecture concludes with a step-by-step example of how data gets paged in and out, followed by a brief introduction to complications introduced in multi-processor systems. |
| Textbook Reading |  |
|  | Chapter 8 |
| Homework |  |
|  | The homework assignment covers primarily virtual memory systems, asking students to show step by step the paging in and out of memory for a given program on a given hardware setup. |
| LECTURE |  |
| Topic |  |
|  | I/O |
| Summary of Lecture Content |  |
|  | This lecture is primarily intended as a topical overview of issues pertaining to I/O on a hardware level. The lecture begins by introducing the physical design of a hard disk, and explaining how the performance and capacity are affected by certain parameters such as density and the number of platters. The lecture moves on to discuss data transport via buses, using the Pentium III I/O system as an example. The end of the lecture briefly discusses methods of interfacing the I/O system with the processor, including polling and DMA. |
| Textbook Reading |  |
|  | Chapter 8 |
| FINAL EXAM See Attachment: 370-Exam3-Sol |  |