Homework 3

- Show that the total integrated noise current through the inductor is IN2 = kT/L

- Assume that M1 is saturated and m1=0. Write an expression for:

  - Small signal low frequency gain

  - Output noise voltage (i.e. v/√Hz)

  - Input referred noise voltage (i.e. v/√Hz). Consider only thermal noise sources and ignore all capacitances.

3. Considering only thermal noise and ignoring capacitances, write the expression for the noise voltage vn2 in terms of small signal parameters. (M1 and M2 form a current mirror, L1=L2, W2=MW1).

4. Referring to the figure of problem (3). Given VDD=5V, M=10, IREF=100µA, R=2k. What is the minimum noise vn2. Consider only thermal noise and remember that the circuit has to work as a current mirror.

5. The figure shows a transimpedance amplifier (often used as a preamp in fiber optic receivers). What is the output noise voltage vo2? What is the input referred noise current ii2? How is this related to the 3dB BW of the amplifier?