Homework 5

- Ignore the body effect. M1 and M2 are saturated. M1 and M2 have the same W and L.

  - For the small signal input vi what s the small signal low frequency gain (vy/vi)?

  - If VY=VO+VQ where VQ is the quiescent output voltage (i.e. due to VBIAS):

    - Write a power series relating Vo to Vi.

    - What are the input referred values of HD2, HD3, and IM3.

- M1 and M2 are saturated. Ignore body effect. IO = ID1 – ID2.

  - Write a power series relating IO to Vi.

  - What are HD2 and HD3?

- The figure shows a psuedo differential amplifier.

  - Write a power series relating Vo to Vi.

  - Write an expression for HD2, HD3, and IM3 (input referred).

  - Using the power series, write an expression for the small signal low frequency gain.