Final Exam

Exam duration: 2 hours, 4 questions

- Question 1 is comprised of 3 independent short problems (a, b and c).

    - Assume M1 in saturation, =0, ignore all capacitances. Write an expression for the low frequency small signal gain in terms of gm, RL, etc. (i.e. vo/vin)

    - R2=R3=R4=R5. A magic voltage 1.262V is produced at VREF giving a zero TC at 25oC. Also VBE2 = 0.7V at 25oC. What is the voltage at the opamp output at 25oC (i.e. node x)?

    - Write an expression for the total integrated noise voltage across the capacitor. Use temperature = T. Derivation is not required.

- Question 2 is comprised of 2 parts (i and ii).

    - Assume M1 in saturation, =0, M1 has length W, width L, mobility µ, oxide capacitance Cox. VQ is the quiescent output voltage.

      - Derive a power series relating Vo and Vi.

      - Write an expression for the input referred HD2, HD3

    - An amplifier is formed by cascading two amplifiers: A and B.

A can be represented as a power series

SOA = a1SiA + a2S2iA + a3S3iA + ……

B can be represented by a similar power series

SOB = b1SiB + b2S2iB + b3S3iB + ……

      - Write the first 3 terms of the power series of the overall amplifier.

      - Write an expression for the input referred HD2, HD3

- Question 3 is comprised of 5 parts (1,2,3,4 and 5).

vi2 and ii2 are uncorrelated input noise generators that represent the noise of the network. RS is the series resistance associated with the source vin. zin is the input impedance of the network. You can assume that the load resistor RL is noiseless.

      - Write a definition of Noise Factor F.

      - Show that the minimum value of F is achieved with

RS2 = vi2/ii2.

      - What value of RS gives the minimum values of the output noise, vo2?

      - If ii2=0 then:

        - What value of RS gives minimum F?

        - What value of RS gives minimum vo2?.

      - Why do 4i and 4ii differ?

The total inductive, capacitive, and resistive loads at node x are LT, CT, and RT.

What is the steady state oscillation frequency (rad/s)?

If Gmeff is the effective large signal transconductance of the differential pair, then write an expression for the steady state value of Gmeff.

Assuming that the steady state amplitude at node y is large enough that the differential pair functions as an ideal switch, then derive the expressions for the signal amplitude Vx at node x and Vy at node y.

Assume the oscillation x is a pure sinusoid.

Hint: Fourier series for square wave frequency ω (rad/s)

ω (t) = K(1/2 + 2/π (cos ω t + 1/3 cos 3 ω t + …))