Midterm Exam

Exam duration: 1.5 hours, 3 questions

- Question 1 is comprised of 3 independent short problems (a, b and c).

    - Assume =0. Ignore all capacitances and the body effect. Assume M1 and M2 are saturated. What is low frequency small signal voltage gain (i.e. vo/vin)?

    - What is the noise voltage  vo2 (i.e. v2/Hz)? Use temperature = T.

    - Write an expression for the total integrated noise voltage across the capacitor. Use temperature = T.

- D is an ideal diode (i.e. no series resistance). IBIAS is a DC bias current source iin is a small signal input current.

    - What is low frequency small signal gain (i.e. vo/iin)? Draw the small signal model.

    - What is the input referred noise current (i.e A2/Hz)? Assume that only the diode produces noise.

    - Under the same assumptions, what is the output noise voltage vo2 (i.e. v2/Hz)?

- Question 3 has two parts (a and b).

  - Assume a magic voltage of 1.28V gives a temperature independent referene voltage at 400oK. Also assume that at 400oK with m=1 and Ic=100µA, we get VBE=0.7V. Use kT/q=26mV @ 300oK.

    - Indicate with a sketch the correct signs on the input terminals of the opamp.

    - Calculate the values R1, R2 that give a temperature independent reference at 400oK (a current of 100µA should flow through Q1).

  - Ignore the body effect, assume =0 and M1 and M2 are saturated. M2 is 8 times wider than M1. Both have the same L. The opamp is ideal. Use kT/q=26mV @ 300oK. Assume a magic voltage of 1.26V, T=300oK with Ic=100uA, |VBE|=0.7V for m=1.

    - Calculate the values of resistances R3, R4 that give a temperature independent reference voltage at 300oK (a current of 100µA should flow through M2).