EECS 579: Digital System Testing

| SYLLABUS |  |
| --- | --- |
| LECTURE 1 |  |
| Topic |  |
|  | Class Introduction, Course Overview |
| Summary of Lecture Content |  |
|  | After reviewing the syllabus with the class, the reasons behind system failure were discussed, to explain why digital testing is neccesary. Some examples of system failure were given, including the 'Three-Mile Island' nuclear reactor meltdown, the Therac-25 radiation machine, the class-action suit against Toshiba over using flawed NEC floppy disk controller chips, and the Centronics microprocessor-controlled printers. Summing up the end of the first lecture was a overview of why testing is difficult, including increasing complexity of the chip, increasing clock frequency, and the different testing needs of ASICs and system-on-a-chip devices. Discussion of the |
| LECTURE 2 |  |
| Topic |  |
|  | Economics of Testing |
| Summary of Lecture Content |  |
|  | This lecture began with a discussion of the costs associated with testing. Three diffrent areas of cost were defined, and the components of each were discussed: test equipment, test software development, and design-for-test (DFT). The second part of the lecture focused on testing terminology, in particular the attributes of different types of testing: on-line or off-line testing, self-testing, and external testing. Two types of signal detection (voltage level testing and current level or IDDQ testing) were explained. The lecture concluded with a summary of testing goals and a brief explanation of each goal: high fault coverage, high fault resolution, efficient test generation procedures, short test application/response times, and built-in self-testing (BIST) capability. |
| Textbook Reading |  |
|  | All of Chapter 1, all of Chapter 3 in the Bushnell and Agrawal text. |
| Homework |  |
|  | Homework 1 assigned. Seven problems, first half on economics of testing, second half on basic logic-level fault modeling. See attachments 579-HW1.pdf and 579-HW1-Sol.pdf |
| LECTURE 3 |  |
| Topic |  |
|  | Testing Basics I: Fault types and SSL model |
| Summary of Lecture Content |  |
|  | This lecture began with an overview of the different modelling levels to consider when testing a circuit, and the components associated with each. It was noted that this course focuses on testing primarily at the gate (or logic) level. Next, the types of fault classifications were discussed: permanent, intermittent, and transient. A list of definitions for some logic-level fault models were given, and the Single-Stuck Line model (also referred to as SSL, single-stuck-at, or ssa) was discussed in depth. Examples consisted of a 2-input AND gate and a 2-input OR gate. First, it was shown that AND and NAND gates has the same minimal test sets, as well as OR and NOR gates. Building on that idea, a proof was derived in class to prove the theory that an n-input elementary gate has a minimal test set of n+1 tests. |
| Textbook Reading |  |
|  | All of chapter 4 in the Bushnell and Agrawal text. |
| LECTURE 4 |  |
| Topic |  |
|  | Testing Basics II: SSL Fault Model, continued |
| Summary of Lecture Content |  |
|  | The first topic of this lecture was a discussion of SSL faults in XOR/XNOR gates. A theorem was proved, showing that for an odd number of inputs, the minimal test set for an XOR/XNOR gate would be two tests. These two tests would form a minimal test set as long as they were bitwise-compliment input patterns. For an even number of inputs, the minimal test set would be three tests. Two of the tests would be bitwise-compliment input patterns - these two tests would detect all SSL faults on input lines, but only one SSL fault on the output line. A third test would complete the test set, if it detects the other SSL fault on the output line. Next, the problems of controllability and observability in a circuit were discussed. A 4-input, 4-gate logic circuit (with one output) was given to illustrate the idea of path sensitization. In class, the test patterns for the input line “a stuck-at-0” was derived through path sensitization. Then the completed fault table for the circuit was shown, so that fault equivalence and fault- table collapsing could be discussed. The lecture ended with a summary list of new terms (fault equivalence, redundant fault,complete test set, minimal test set, essential test, easy-detect fault, hard-detect fault) and their definitions. |
| Supplementary Material |  |
|  | See attachment: 579-Lecture_4.pdf for an example schematic used in class |
| LECTURE 5 |  |
| Topic |  |
|  | Testing Basics III |
| Summary of Lecture Content |  |
|  | The lecture started with a brief review of definitions given last time, followed with a discussion of the concept of ‘path sensitization’. ‘Controlling’ and ‘noncontrolling’ values for gates in a combinational circuit were explained. A brief example of sequential path sensitization was explored, using a serial adder consisting of 1 full-adder and one flip-flop. The concept emphasized in this example was how a fault might need to be propagated through the full adder circuit and into the flip-flop in order to be seen at an observable output, so that testing may take place over more than one clock cycle. Sequential circuit testing is explored more fully starting with lecture 11. Next, fault-equivalent classes in elementary logic gates were discussed. A short example circuit, using NAND gates, was used to illustrate how fault equivalence and fault collapsing could reduce the number of potential fault sites in a circuit. This example led directly into a definition/example of fault dominance. The last topic of the lecture was on fanout-free circuits, and their special properties. |
| LECTURE 6 |  |
| Topic |  |
|  | Multiple Stuck-Line (MSL) Model |
| Summary of Lecture Content |  |
|  | The lecture started with a recap of fanout-free circuits, and a brief discussion about using fanout-free circuits with the Checkpoint Theorem. Next, the topic of Multiple Stuck-Line (MSL) faults in a circuit was introduced, in relation to fanout-free circuits. Using a small fanout-free circuit, the idea of fault-masking was demonstrated. A theorem was proved, in class, showing that a minimal set of SSL tests will detect all MSL faults in a fanout-free circuit. This topic led directly to a discussion of two-level logic circuits. The rest of the lecture focused on this topic, particularly the feasibility of fault-masking in single/multiple output two-level circuits, and examples of how fault masking can occur. |
| Homework |  |
|  | Homework 2 assigned. Seven problems, covering fault equivalence, fault collapsing, fanout-free circuits, redundancy, and combinational ATPG algorithms. See attachments 579-HW2.pdf and 579-HW2_sol.pdf |
| LECTURE 7 |  |
| Topic |  |
|  | Combinational Test Generation I |
| Summary of Lecture Content |  |
|  | This lecture started out with a general overview of possible approaches to testing. Exhaustive testing was shown to be the worst method and not feasable for typical circuits. The Boolean Difference method was defined and explained with an example, and a general aglorithm for a Fault-Oriented Deterministic Method was given. Most of this lecture focused on the D-algorithm. D-notation was explained, and a summary of the D-algorithm was discussed, as well as vocabulary terms (propagation, justification, iteration) associated with it. The rest of the lecture was spent in going through an example circuit using the D-algorithm to detect different faults. See Attachment: Example 1 in 579-Lecture7_8_9.pdf |
| Textbook Reading |  |
|  | Chapter 7, Sections 7.1 - 7.5.1 in the Bushnell and Agrawal text |
| LECTURE 8 |  |
| Topic |  |
|  | Combinational Test Generation II |
| Summary of Lecture Content |  |
|  | This lecture began with a brief review of the D-Algorithm example from the previous lecture. Two more examples were analyzed using the D-Algorithm, more complicated circuits that helped illustrate the concept of backtracking. Much of the class time was spent on these two examples. Next, PODEM was introduced and discussed. The steps of the basic PODEM algorithm were defined, and the difference between backtracking and backtracing was strongly emphasized. Near the end of the class, an example of PODEM was begun (to be finished in the next lecture). |
| Textbook Reading |  |
|  | Section 7.5.2 of the Bushnell and Agrawal text |
| LECTURE 9 |  |
| Topic |  |
|  | Combinational Test Generation III |
| Summary of Lecture Content |  |
|  | The first topic of this lecture was a recap of the steps of the D-algorithm, as well as the steps of PODEM. The first example from the last class lecture was finished, illustrating some of the differences between D-algorithm and PODEM. The rest of the class was spent on carefully going through two more examples of PODEM, stressing the proper terminology and notation to write out each PODEM solution correctly. |
| LECTURE 10 |  |
| Topic |  |
|  | Test Generation IV |
| Summary of Lecture Content |  |
|  | This lecture began with the topic of the FAN ATPG algorithm. The differences between FAN and PODEM were discussed, as well as the different strategies employed by FAN that make it heuristically faster than PODEM. Several small examples were presented and analyzed to illustrate the concepts of unique path assignment, head lines, bound lines, and multiple backtracing. Next, the Vector-PODEM (VPODEM) algorithm was briefly discussed. An example of applying a hierarchical model (VPODEM) was shown using a 74181 ALU/function generator. Other Combinational ATPG algorithms were presented and discussed, including SOCRATES, Critical Path, and Random test generation. The lecture concluded by defining quality measures that can be used to compare the quality of different ATPG algorithms, such as fault coverage, fault efficiency, and test generation time. |
| Textbook Reading |  |
|  | Rest of chapter 7 in the Bushnell and Agrawal text (section 7.5.4 is recommended, but not required). |
| LECTURE 11 |  |
| Topic |  |
|  | Sequential Circuit Testing I |
| Summary of Lecture Content |  |
|  | This lecture began with a discussion of test set compaction techniques. The differences between simulation-based compaction, static compaction, and dynamic compaction were explained, and a brief example of a test set compaction problem was given. Sequential circuit testing was introduced, and some general approaches were defined and discussed. Important concepts introduced include the idea of a test as a sequence of input vectors, as well as the difference between synchronous and asynchronous circuits. The state-table method of generating a sequence of test vectors was illustrated with an example. The idea of having a known initial state vs. an unknown initial state was discussed, and the diagnostic tree method of generating test vectors was illustrated for both cases. The term project was also discussed, and a handout detailing the requirements of the project was given in class. |
| Textbook Reading |  |
|  | Section 8.1 in Bushnell and Agrawal |
| Homework |  |
|  | Homework 3 assigned. Six problems, covering Boolean Difference, PODEM, FAN, state-table method, and sequential ATPG. See attachments 579-HW3.pdf and 579-HW3_sol.pdf A proposal for the final project was assigned, due in 2 weeks. See attachment: 579-Project.pdf |
| LECTURE 12 |  |
| Topic |  |
|  | Sequential Circuit Testing II |
| Summary of Lecture Content |  |
|  | This lecture started with the topic of machine identification and sequences. An example was given and analyzed in class to show how to construct a checking sequence for an n-state sequential machine. The example also illustrated the concepts of a synchronizing sequence, homing sequence, and distinguishing sequence. This example led directly into time-frame expansion for sequential ATPG.; the instructor emphasized that synchronizing sequences were necessary for time-frame expansion, since a key assumption is that the initial state of the circuit is known. A small circuit example was analyzed, (two flipflops and five logic gates), and a 3-cycle test sequence was derived. For the second example, a 1-bit serial adder was analyzed. The lecture concluded with a summary of the advantages and disadvantages of the time-frame expansion approach. |
| Textbook Reading |  |
|  | Chapter 8.2.1 through 8.2.5 in Bushnell and Agrawal text. |
| LECTURE 13 |  |
| Topic |  |
|  | Sequential Circuit Testing III |
| Summary of Lecture Content |  |
|  | The first topic discussed in this lecture was a definition of a 9-valued algebra, and an explaination of why Roth’s 5-valued algebra is insufficient for sequential testing. Next, cycle-free circuits were defined and discussed, and their testability advantages were summarized. The rest of the lecture focused on simulation-based sequential ATPG algorithms. A flowchart was shown that explained the process of using a fault simulator to generate/apply test vectors. The CONTEST algorithm was discussed in detail, including cost functions and the objectives of each phase of the algorithm. The Genetic algorithm and high-level test generation were overviewed near the end of the lecture - the key ideas were summarized and compared with CONTEST. |
| Textbook Reading |  |
|  | Finish all of Chapter 8 in Bushnell and Agrawal text |
| LECTURE 14 |  |
| Topic |  |
|  | Midterm Review Session |
| LECTURE 15 |  |
| Topic |  |
|  | Midterm Exam A sample midterm exam from 1998 is included as an attachment, see 579-1998midterm.pdf |
| LECTURE 16 |  |
| Topic |  |
|  | Processor Testing I |
| Summary of Lecture Content |  |
|  | This lecture focused on the concept of testing microprocessor and microcontroller circuits. A simplistic block diagram was shown, to illustrate how a microcontroller contained different components such as a central processing unit (CPU), memory, and input/output transfer control units. The problems associated with testing microcontrollers were discussed (no gate level implementation details, no fault models at this level of abstraction, the use of heuristic or ad-hoc methods). Next, the focus shifted to testing components of the microcontroller, starting with the CPU. A general overview of basic 5-stage CPU behavior was given for students who have not had architecture courses. The Intel 8085, an 8-bit microprocessor, was discussed as an example. The heuristic testing method used by Intel to test this chip was presented, and discussed in detail. The lecture concluded with a summary of the problems and approaches to CPU testing. Emphasis was given to the idea that test selection for a CPU depends on the instruction set of the CPU, as well as the organization of the CPU. It was also stressed that coverage may be hard to determine, incomplete, and thorough testing of the CPU this way is slow. |
| LECTURE 17 |  |
| Topic |  |
|  | Processor Testing II |
| Summary of Lecture Content |  |
|  | This lecture continued with the topic of CPU testing by introducing a formal model, the graph-based fault model. An example graph was shown for the Motorola 68000 microprocessor. The rest of the lecture focused on memory testing. A basic hierarchy of memory levels in a computer was explained, from a register file in a processor, through caches, to the secondary memory (hard drive). The differences between RAM and ROM were discussed, and the organization of RAM memory was explained. Next, the differences between SRAM and DRAM were discussed. After a brief review of the types of faults that could occur in an IC (particularly in memory), RAM testing methodes were discussed. The three basic tests discussed include the checkerboard test, march tests, and galloping tests. An example of a march test (MATS+ algorithm) was discussed in class. The lecture concluded with a discussion of the types of functional memory faults were discussed, and how to represent these faults in a state diagram. |
| Textbook Reading |  |
|  | Chapter 9 of Bushnell and Agrawal |
| LECTURE 18 |  |
| Topic |  |
|  | Design For Test I |
| Summary of Lecture Content |  |
|  | This lecture begain with a discussion of circuits classified as iterative logic arrays (ILAs) and how fault models can be efficiently applied. An example of a ripple-carry adder(four full-adders) was used to illustrate how certain test patterns to one cell can be applied to all cells simultaneously. The concept of C-testability was discussed, and illustrated with an example of a gate array composed of three-input AND gates. Next, these ideas were tied into the concept of Design for Testability (DTF). Testability goals were again reviewed, and the effectiveness of testability measures were discussed, particularly the ability to control or observe certain nodes in a circuit. The SCOAP algorithm was introduced as a solution. The basic steps of SCOAP were discussed, but an example was not analyzed in class. The lecture ended with a comparison of how effective certain testing methods were, for testability parameters such as fault coverage, test length, and test generation effort. |
| Textbook Reading |  |
|  | All of Chapter 6, Chapter 14 section 14.1 in Bushnell and Agrawal text |
| Homework |  |
|  | Homework 4 assigned. Five problems, covering exhaustive/pseudoexhaustive testing, cell fault method, memory tests, SCOAP, and C-testability. See attachments 579-HW4.pdf and 579-HW4_sol.pdf |
| LECTURE 19 |  |
| Topic |  |
|  | Design For Test II |
| Summary of Lecture Content |  |
|  | This lecture began with a summary of DFT approaches, and a comparison of Ad Hoc methods of design vs. systematic methods. Test point insertion was discussed, as well as what sites in a circuit would be control-point or observation-point sites. Control/oservation points in memory circuits were also discussed. The concept of fault-tolerant design was stressed as a way to increase testability of a circuit. An example of an n-bit counter was shown, to illustrate the usage of circuit restructuring to insert control points. Timing control was also briefly discussed, to emphasize the importance of avoiding asynchronous timing and making the clock both observable and controllable during testing. Next, XOR-based designs and Reed-Muller circuits were discussed. An example was illustrated with the function F(x,y) = x + y being converted into Reed-Muller canonical form. The topic of Scan design was introduced and discussed for the rest of the lecture. Scan chains were expalined, as was the general algorithm for testing with scan design. Several variants were overviewed, including level-sensitive scan design (LLSD), random access scan, partial scan, I-path testing, and boundary scan. |
| Textbook Reading |  |
|  | Rest of Chapter 14 in Bushnell and Agrawal text. |
| LECTURE 20 |  |
| Topic |  |
|  | Built-In Self Test I |
| Summary of Lecture Content |  |
|  | This lecture focused mainly on Built-in selft-test (BIST) techniques. A block diagram was shown to explain how a processor circuit would be partitioned into subcircuits for BIST, and the economic advantages of BIST were discussed. Concurrent or “on line” BIST was discussed, particularly the self-checking ability of the circuit based on error-detecting/correcting codes. The use of both parity codes and arithmetic codes were explained, as well as the definition of a circuit that is self-testing and fault secure. Next, nonconcurrent or “offline” BIST was discussed. Both random and deterministic types of test generators were discussed, as was the purpose of a response monitor. A form of offline BIST, compression-based BIST, was discussed. Concepts emphasized include the signature of test responses produced by compression functions such as one’s counting or transition counting. |
| Textbook Reading |  |
|  | Chapter 15, sections 15.1 through 15.2.4 in the Bushnell and Agrawal text |
| Homework |  |
|  | Homework 5 assigned. Five problems, covering scan design, partial scan, ATE data formats, LFSR, and compression testing. See attachments 579-HW5.pdf and 579-HW5_sol.pdf |
| LECTURE 21 |  |
| Topic |  |
|  | HP 82000 tester and tour of clean room |
| Summary of Lecture Content |  |
|  | The class took a tour of the clean room facilities and examined the HP 82000 tester. |
| LECTURE 22 |  |
| Topic |  |
|  | Built-In Self Test II |
| Summary of Lecture Content |  |
|  | This lecture began with a brief review of compression-based BIST. Both one’s counting and transistion counting were explored in more detail, and compared to each other. Emphasis was given to the idea that transition counting relies on the order in which the test vectors are applied, while one’s counting does not. The use of custom BIST was discussed, in particular the use of different LFSR types to customize test generation as well as response monitors/compactors. Bus-based BIST architectures were briefly discussed, but the bulk of the discussion was about modifiying LFSRs based on the use of XOR gates between registers in the LFSR. Generating functions were discussed, and an example was analyzed that showed how the feedback structure of an LFSR corresponded to a characteristic polynomial, and how this characteristic polynomial related to the number and types of errors that the LFSR could detect. The lecture concluded with a summary of the properties of an LFSR, as well as LFSR-based BIST. |
| Textbook Reading |  |
|  | Rest of Chapter 15 in the Bushnell and Agrawal text |
| LECTURE 23 |  |
| Topic |  |
|  | Built-In Self Test III |
| Summary of Lecture Content |  |
|  | This lecture began with a short overview of BILBO (Built-in logic block observation). From there, ways to implement BIST into a circuit design were discussed. Implementing BIST by regularization was illustrated with the 74X381, showing the almost-regular schematic, then showing a ‘regularized’ version. A tree circuit (74X154) was used to show that regularization of a circuit reduces the number of test required to detect all SSL faults in the circuit. RAM circuits were discussed; specifically, the problems encountered in testing RAM were reviewed, and BIST approaches were discussed as solutions, including the idea of modifying RAM to make it self-testing. Commercial BIST approaches were discussed, including STUMPS and the circular self-test path (CSTP) approach. Finally, a case study of the Bosch AE11 Microcontroller was given. The self-testing features implemented in the AE11 were strongly emphasized, as was the approach to CPU, I/O, and RAM testing on the chip. Finally, the performance of these testing circuits was evaluated, as well as the cost, in terms of chip area and performance loss. |
| LECTURE 24 |  |
| Topic |  |
|  | Logic and Fault Simulation |
| Summary of Lecture Content |  |
|  | This lecture focused on different types simulation. First, the terms fault simulation and emulation were defined and compared. Levels of simulation were defined, and logic (gate) level simulation was explored in detail. A small circuit example was shown, consisting of one flip-flop, one two-input NAND, and one inverter. Compiled-code simulation was discussed, and an example of a two-input NAND gate was shown represented in compiled-code format. Next, event-driven simulation was discussed, along with event scheduling. It was emphasized that time-driven simulation was very different from event-driven simulation. The applications of fault simulation were discussed, as was the deductive method of fault simulation. An example circuit was analyzed in class to show how all faults in a circuit can be simulated in one pass by using this method. Finally, emaulation and its approaches were quickly discussed. |
| Textbook Reading |  |
|  | Chapter 5 in the Bushnell and Agrawal text |
| LECTURE 25 |  |
| Topic |  |
|  | SOC Testing |
| Summary of Lecture Content |  |
|  | This lecture began by defining the three main types of core (IP) circuits: soft core, firm core, and hard core. Some examples of core circuits were listed, and block diagrams were shown for a GSM wireless communications chip and an MPEG2 decoder. SOC testing was discussed, particularly the role of a test wrapper. Two approaches for realization-Independent ATPG was discussed, cell fault testing and universal test sets. An example circuit, implementing the function z = ab’ + ac’ + bcd was analyzed in class to illustrate the concepts of unate and binate variables, and also to show how to derive a universal test set. Next, realization-independent block tests were discussed, specifically the idea of blocking a circuit into balanced inversion-parity circuit blocks. The lecture ended with an overview of the RIBTEC algorithm. |
| Textbook Reading |  |
|  | Chapter 18, sections 18.5 and 18.6 in Bushnell and Agrawal text. Sections 18.1 through 18.4 are recommended, but not required. |
| LECTURE 26 |  |
| Topic |  |
|  | Design Verification |
| Summary of Lecture Content |  |
|  | This lecture focused on methods of Design Verification. The problems facing verification were discussed, particularly the example of the Intel Pentium’s floating division error and how it occured. The advantages and disadvantages of several methods were discussed, including formal verification (using mathamatical logic to prove that the implementation is equal to the specification), correctness by construction, and simulation. An example of formal verification was analyzed. The specification was given as c = a OR b, and the implementation was given as a NOR gate followed by an inverter. The similarities between design verification and hardware testing were discussed, as was a form of software testing called mutation testing. Next, the error-based method of design verification was discussed. Different ways of collecting design error information were mentioned, as well as how such information could be applied in developing tests. The lecture concluded with a summary of basic error models and how tests are generated from them. |
| LECTURE 27 |  |
| Topic |  |
|  | Final Exam Review A sample final exam is included as an attachment, 579-1998final.pdf |