EECS627: VLSI Design II

| SYLLABUS |  |
| --- | --- |
| LECTURE 1 |  |
| Topic |  |
|  | Introduction to the class and project ideas |
| Summary of Lecture Content |  |
|  | The beginning of the lecture is used to introduce the professor and to give important information related to the class, such as the class website, home assignments, project design review dates and the exam dates. Then the topics to be covered in the course are listed and briefly summarized. The final project is a substantial part of the course and some interesting project ideas are discussed. Project milestone dates are provided to help the students keep their projects on track. At the end of the class, students are provided non-disclosure agreements forms for tsmc18 and MOSIS to be filled and returned in the next class. Discussion of the final project: Projects must include developing a behavioral model, a verification environment, a synthesizable Verilog description, synthesis, design for test, placement and routing, LVS/DRC and timing back annotation. Projects may include custom modules, analog or mixed signal components, and new design techniques, such as Dual-Vt or multi-Vdd design techniques. The project grade is based on two intermediate design reviews and a final design review. Each review consists of a written report and an oral presentation by a representative of the team. In addition, each team will hand in a project proposal at the start of the semester to identify their project, which is not graded. Project reviews are graded both on technical content and presentation quality. It is imperative that student start on their projects at the start of the term and continue to work diligently through the semester. The position of design reviews during the semester is not stringent. Students should have done their verilog coding, their verification environment setup and at least the block-level verification before Design Review-1. For design review-2, the students should have completed their verification completely, have synthesized the whole design, and they should be able to achieve timing closure and have some kind of floorplan in mind to actually lay out the chip. |
| LECTURE 2 |  |
| Topic |  |
|  | Project Topics See Attachment: 627-Project_ideas |
| Homework – Lab |  |
|  | Lab 1 discussed and assigned. The chip for this lab consists of an 8-bit multiplier that computes the result in four pipeline stages. Students are expected to write a C-based golden brick, a Verilog description of the design, verify its functionality and synthesize it in Design Compiler for the highest frequency possible. See Attachment: 627-Lab1 |
| LECTURE 3 |  |
| Topic |  |
|  | Circuit sizing using logical effort |
| Summary of Lecture Content |  |
|  | This topic is covered during a span of 2 lectures. Logical effort is a simple method to determine the optimal number of stages and sizing of transistors in a given circuit, in order to minimize delay. First, the delay of an inverter (d) is derived (using its RC equivalent model) and expressed as a sum of its intrinsic delay (p) and the effort delay (f), which in turn is the product of electrical effort (g) and logical effort (h). Then the method to calculate intrinsic delay, p and electrical effort, g for any arbitrary gate is explained. Next, the concept of branching effort, b is introduced and the total delay of a generalized muti-stage logic networks is derived. The delay for the multi-stage network is minimized to obtain the condition for minimum delay of the circuit (stage effort of each stage should be same for minimum delay). |
| Textbook Reading |  |
|  | Chandrakasan: Chapter 7.5.2 Sutherland, Logic Effort - designing fast CMOS Circuits Academic Press, 1999 |
| LECTURE 4 |  |
| Topic |  |
|  | Circuit sizing using logical effort |
| Summary of Lecture Content |  |
|  | Students are explained how the delay of a given path can be reduced even further by adjusting the number of stages in the path. A table is provided with optimal number of stages for different ranges of values of path effort. Next, some examples of 3-4 stage networks are solved and discussed in the class. In the end, the limitations of logical effort method are discussed. |

| LECTURE 5 |  |
| --- | --- |
| Topic |  |
|  | Logic Circuit Styles (static and dynamic) |
| Summary of Lecture Content |  |
|  | This topic is covered during a span of 3 lectures. In this topic, performance, power and robustness of different static and dynamic logic families are discussed. In static logic families, pulse-static CMOS logic, DCVS (differential cascode voltage switch), CNTL (cascode non-threshold logic), pass-gate logic, CPL (complementary pass-gate logic), push-pull logic and OPL (output prediction logic) are explained and their advantages and disadvantages are examined. |
| Textbook Reading |  |
|  | Chandrakasan: Chapter 7.1, 7.2, 7.3, 7.4, 7.5.1, 8.1, 8.2, 8.3 |
| Homework |  |
|  | Lab 2 is assigned. Behavioral code for two more blocks (a pseudo-random test pattern generator and a signature analyzer) are added to the multiplier, constituting a BILBO to the design. Students have to synthesize the design and perform place-and-route (APR) of the multiplier, lfsr and signature analyzer separately. Finally, all the blocks are assembled at the top-level along with clock, power and ground routing. Cadence Silicon Ensemble is used for performing the block-level APR and top-level chip assembly. See Attachment: 627-Lab2 Project Proposals are due |
| LECTURE 6 |  |
| Topic |  |
|  | Logic Circuit Styles (static and dynamic) |
| Summary of Lecture Content |  |
|  | Signal integrity issues in dynamic design, such as charge leakage, charge sharing, capacitive coupling, miller and back-gate coupling are discussed. In dynamic logic families, a basic dynamic gate, NORA/Zipper logic, basic domino gate and domino-cascading, footless domino, multi-output domino logic, compound domino logic, dual-rail domino logic and self-resetting domino are discussed. The speed, noise immunity and power of every logic family is analyzed. |
| LECTURE 7 |  |
| Topic |  |
|  | Logic Circuit Styles (static and dynamic) |
| Summary of Lecture Content |  |
|  | Continuation of previous lecture. |
| Homework |  |
|  | Lab 3 is assigned. The chip assembled in Lab3 is imported into Cadence ICFB and Mentor Graphics Calibre is used to perform DRC/LVS checks on the individual blocks as well as the top-level. See Attachment: 627-Lab3 |

| LECTURE 8 |  |
| --- | --- |
| Topic |  |
|  | Noise sources, analysis and avoidance |
| Summary of Lecture Content |  |
|  | This topic is covered during a span of 2 lectures. Different noise sources are discussed and their impact on performance is analyzed. First interconnect coupling with an aggressor driver/net and a victim driver/net is analyzed using an equivalent RC model. Dependence of peak noise value and duration of noise pulse on aggressor driver size, victim driver size, coupling capacitance and ground capacitance is discussed. Next the noise due to charge sharing and back-gate coupling in a simple dynamic gate is analyzed. Various other noise sources such as IR-drop, substrate coupling, soft error rates are introduced to the class and methods to analyze these noise sources are discussed. Noise margins/noise budgeting in a chain of static inverters is analyzed in detail. |
| Textbook Reading |  |
|  | Chandrakasan: Chapter 8.1, 8.2, 8.3, 9.2 |
| LECTURE 9 |  |
| Topic |  |
|  | Noise sources, analysis and avoidance |
| Summary of Lecture Content |  |
|  | Various schemes for noise avoidance, such as sizing of victim/aggressor drivers, widening of wires, spacing/shielding, buffering and some active noise-avoidance methods are discussed. Some error detection techniques for noise avoidance in buses are discussed. Various soft-error immune hardened latches for soft error avoidance are also discussed. |

| LECTURE 10 |  |
| --- | --- |
| Topic |  |
|  | Low power design |
| Summary of Lecture Content |  |
|  | This topic is covered in one lecture. Various low power design schemes, such as energy recovery logic families, dynamic voltage scaling, clock gating, low power bus encoding schemes, design of systems with multiple voltage islands are introduced and discussed. Energy recovery circuit design is discussed as a fundamentally different method of performing computation with lower dynamic power dissipation. The idea of using resonant systems for low-power signalling is generalized to the use of such resonant systems for computation. Fine grained and coarse grained energy recovery logic design styles are discussed. |
| LECTURE 11 |  |
| Topic |  |
|  | Midterm Exam See Attachment: 627-Exam1 |

| LECTURE 12 |  |
| --- | --- |
| Topic |  |
|  | Leakage current and low leakage design |
| Summary of Lecture Content |  |
|  | This topic is covered during a span of 2 lectures. With an example of a CMOS inverter, various components of leakage currents are described in the beginning of the lecture. The relative magnitudes of different components of leakage currents in current process technologies are discussed. The expression of subthreshold current, showing exponential dependence on Vth is presented and ways to minimize subthreshold current are discussed. The effect of technology scaling on various components of leakage are discussed. The dependence of logic states on leakage is analyzed. Then various design schemes for reduction of leakage such as MTCMOS, sleep state assignment, dual-Vth circuit optimization are presented. Pros and cons of each technique are discussed. The key difference between active leakage reduction and standby leakage reduction is explained. |
| Textbook Reading |  |
|  | Chandrakasan: Chapters 3 and 4 |
| LECTURE 13 |  |
| Topic |  |
|  | Leakage current and low leakage design |
| Summary of Lecture Content |  |
|  | Schemes for reducing leakage in caches, such as use of high Vth, architectures as drowsy caches, low leakage state assignments and lower Vdd in sleep modes are presented. |

| LECTURE 14 |  |
| --- | --- |
| Topic |  |
|  | Packaging and on-chip power supply design |
| Summary of Lecture Content |  |
|  | This topic is covered during a span of two lectures. First, various components in a power supply network (board inductance, package inductance, onchip resistance, non-linear devices that draw current and decoupling capacitances) are discussed. Next, the modelling of non-linear devices as time varying current sources is described. Then the difference between implicit on chip capacitance due to non-switiching devices and explicit decoupling capacitance is explained. A simple method for calculating implicit decoupling capacitance from total power consumed by the chip is presented. |
| Textbook Reading |  |
|  | Chandrakasan: Chapter 24 |
| LECTURE 15 |  |
| Topic |  |
|  | Packaging and on-chip power supply design |
| Summary of Lecture Content |  |
|  | Next, different grid topologies like tree, grid, power planes are presented and advantages/disadvantages of each topology is discussed. Overall drop and its two components: IR-drop and Ldi/dt drop are explained. Effect of decoupling capacitances on suppressing the high frequency components of voltage drop is explained. Different decap structures proposed and used in current technology are discussed and leakage problem with excessive explicit decap is explained. Next, resonance in power grid network is explained and different scenarios which can make a grid resonate causing excessive supply drop are discussed. In the end, slides showing transient analysis of power grid, different frequency components of the drop, impact of additional decoupling capacitance and resonance are shown to the class. |

| LECTURE 16 |  |
| --- | --- |
| Topic |  |
|  | Interconnect analysis and inductance modeling |
| Summary of Lecture Content |  |
|  | This topic is covered during a span of two lectures. In the beginning of class, a qualitative explanation of occurrence of inductance is provided. Then the frequency dependence of impedance on frequency and skin effect is explained. Next, inductive coupling between signal nets is analyzed and the Porch effect is explained with an example of a bus. Inductive coupling is compared to capacitive coupling and both are added to obtain the overall interconnect coupling. The effect of inductive coupling on slew and delay of the signal is explained. Next part of the lecture is focused on inductance modelling. We start with the four Maxwell’s equations and discuss the techniques to solve them. Next the complexity in solving complete Maxwell’s equations in a chip are discussed. Then the common simplifying assumptions like the electrostatic assumption and DC assumption, used for capacitive extraction are presented. For inductance extraction, the need for a quasi-static assumption is explained. Using the quasi-static assumption, the concept of partial inductance is explained, which is further used to describe the PEEC (partial equivalent electrical circuit) model for interconnect parameter extraction. Then a full PEEC based model for a driver-receiver pair in a power supply network is presented and the complexity in inverting the fully dense L matrix are explained. |
| Textbook Reading |  |
|  | Chandrakasan: Chapters 17, 19 |
| LECTURE 17 |  |
| Topic |  |
|  | Interconnect analysis and inductance modeling |
| Summary of Lecture Content |  |
|  | Various explicit (block diagonal and shift/truncate) and implicit (FastHenry) sparsification techniques are explained. Next, inductance avoidance techniques, like shielding, dedicated gnd planes, interdigitated wires, staggered inverters, twisted-bundle layout structures are explained. The end of the lecture is devoted to the analysis of transmission lines. We start with the wave equation in 1D and 1D harmonic systems. Then the properties of wave equations, boundary conditions, characteristic impedance, wave velocity etc. are described. Next, transmission line termination, discontinuities in transmission line and application of t-lines to VLSI data transmission are discussed. |

| LECTURE 18 |  |
| --- | --- |
| Topic |  |
|  | Clock design, PLL, DLL |
| Summary of Lecture Content |  |
|  | This topic is covered during one lecture. In the beginning of the lecture, different grid topologies like H-tree, spline and grid-tree are presented, and their advantages and disadvantages are discussed. Then the concept behind some low power resonant clock generation schemes like rotary clocks are explained. Next, various components of clock uncertainty: skew, jitter, duty cycle are explained. In the end, slides showing trends in clock frequency, fanout-4 inverter delay and fanout-4 inverter delays/cycle in Intel microprocessors are presented and discussed. |
| LECTURE 19 |  |
| Topic |  |
|  | Synchronization, clocking schemes, meta-stability |
| Summary of Lecture Content |  |
|  | This topic is covered during a span of three lectures. In the first lecture, the dependence of clock period on clock uncertainty in a flip-flop based design is explained with the help of a simple example. The exploitation of time-borrowing and skew tolerance in latch based design is explained with another example circuit, consisting of latches. Then the need of non-overlapping clocks in a latch based design is explained. Next the short path constraints in flip-flop based and latch based designs is discussed. Then some skew tolerant designs like skew tolerant dominant domino and 2-phase and 4-phase clocking schemes are discussed. |
| Textbook Reading |  |
|  | D. Harris, Skew Tolerant Circuit Design, Morgan Kaufmann Publications, 2000 W. J. Dally, J. W. Poulton, Digital Systems Engineering |
| LECTURE 20 |  |
| Topic |  |
|  | Synchronization, clocking schemes, meta-stability |
| Summary of Lecture Content |  |
|  | The problems arising from synchronization failure and metastability are explained. The synchronization dynamics and synchronization time are explained with the example of a NAND SR latch and expression for the decision time of the latch is derived. Next, the concept of metastable states is explained and expressions for probability of synchronization failure, frequency of synchronization failure and mean time between synchroniation failures (MTBF) are derived. With the help of an example, students are given an intuition of the typical values of mean time between synchronization failures. |
| LECTURE 21 |  |
| Topic |  |
|  | Synchronization, clocking schemes, meta-stability |
| Summary of Lecture Content |  |
|  | The concept of completion detection and its advantage in asynchronous systems is explained. The lecture is finished with a discussion of common mistakes in synchronizer design. |

| LECTURE 22 |  |
| --- | --- |
| Topic |  |
|  | Cache and memory design |
| Summary of Lecture Content |  |
|  | This topic is covered during a span of one lecture. First, structures of SRAM and DRAM cells are explained in detail. Next their read/write operations are discussed. In the end, reliability issues due to soft errors and implementation of redundancy to counter them are explained. |
| Textbook Reading |  |
|  | Chandrakasan: Chapters 14, 15.1, 15.2 |
| LECTURE 23 |  |
| Topic |  |
|  | SOI |
| Summary of Lecture Content |  |
|  | This topic is covered during a span of three lectures. In the first lecture, students are taught how SOI can be used for improving the performance of CMOS as scaling is rapidly approaching its limits. Various advantages of SOI over bulk devices are discussed in detail. Figures showing SOI performance improvement over bulk for a recent design are presented and discussed. SOI is classified into partially depleted (PD) and fully depleted (FD) and their structures are presented. Device design considerations and manufacturing processes for both PD and FD are discussed. |
| Textbook Reading |  |
|  | Chandrakasan: Chapter 5 K. Bernstein, N. Rohrer, SOI - Circuit design Concepts |
| LECTURE 24 |  |
| Topic |  |
|  | SOI |
| Summary of Lecture Content |  |
|  | Various problems in PD-SOI are discussed in detail. Discussion starts with equilibrium, transient and steady state value of the floating body voltage in PD-SOI device. Vt variation and kink effect are explained in detail. Next, the history dependence of propagation delay and the effect of first/second switching on output transition in PD-SOI is discussed. |
| LECTURE 25 |  |
| Topic |  |
|  | SOI |
| Summary of Lecture Content |  |
|  | Problems such as transient pass-gate leakage on circuits, reduced noise margins and Vt mismatch in SOI are explained. Various body tie styles, like T-structure and smart body contacts, are presented. |

| LECTURE 26 |  |
| --- | --- |
| Topic |  |
|  | Special Topic: Process variation |
| Summary of Lecture Content |  |
|  | This lecture starts with an overview of different sources of variation like environmental factors (power supply variation, switching activity variation, temperature etc.) and physical factors (structural device parameter variation, interconnect size variation). Next, the statistical description of parameter variation and the difference between inter-die and intra-die variations are explained.Then various methods to characterize process variation like statistical device models, sensitivity based analysis an worst-case analysis are explained in detail. In the end, slides showing effect of effective gate length variation on frequency, leakage and yield are presented and discussed. |
| Textbook Reading |  |
|  | Chadrakasan: Chapter 6 |
|  | FINAL EXAM |