Project P4

B: Multihop TCP Performance with Wireless Channel Awareness

Consider a multihop wireless network. Evaluate the performance of TCP assuming wireless link layer feedback about the channel quality is available. The wireless link-layer decides to send or not to send packets based on the quality of the channel. Consider different cases of link layer queue management and the impact of scheduling policy on TCP.  Compare the performance of channel-aware TCP to one without channel feedback for the following cases:

1) Single FIFO queue

2) Priority queue with priority defined based on channel quality