# Install Guide

### FastTrak Series Linux Driver Version 1.02.0.20

### For Red Hat	7.2 (kernel 2.4.7-10)
	7.1 (kernel 2.4.2-2)
	7.0 (kernel 2.2.16-22)

# Introduction

This driver diskette should be used with all Promise FastTrak Series
adapter and onboard chipsets running under RedHat Linux operating
systems. Do NOT use this diskette for other versions of Linux for FastTrak support.

Linux kernel 2.4.x ATA issue on FastTrak Series PDC20262/PDC20265/PDC20270
Linux Kernels 2.4.x misidentifies PDC20262/PDC20265 (FastTrak 66/
FastTrak 100 Lite motherboard-embedded solution/FastTrak100TX2) cards
and/or chips as simple IDE controllers. This results in the built-in Linux IDE driver trying to handle the controller and can prevent the proper FastTrak ATA RAID driver to be loaded. Follow the installation instructions AND the parameter commands. The instructions will refer to you as "RHF upgrader for New System installs these have been added for you in the instructions."

## Preparing a Floppy Disk

1.	Obtain a formatted blank diskette and label it “FastTrak Driver Diskette”. Insert it into your PC’s floppy drive.

2.	Extract the driver file you downloaded from Promise web to the root directory of your FastTrak Driver Diskette using either WinZip in Windows or Unzip in Linux.

You will use this diskette to load the new Linux driver onto your PC.

# New Systems

As part of the Linux installation process, follow these instructions for installing FastTrak Series Linux Driver Version 1.02.0.20 into a NEW system:

1.	Boot your PC from the RedHat 7.2 Linux Installation CD-ROM

At the Welcome to Red Hat Linux... installation screen, a prompt labeled boot: will appear at the bottom of the screen.

2.	At the boot: prompt, press the Tab key, then type the following:

linux ide0=0x1f0,0x3f6, ide1=0x170,0x376, ide2=0 ide3=0 ide4=0 ide5=0 ide6=0 ide7=0 ide8=0 ide9=0 expert (or expert text, if you prefer).

3.	Press Enter to continue.

4.	At the Devices dialog box, click Yes. Insert your FastTrak Driver Diskette into the floppy drive. When prompted, click OK.

5.	When the Boot Loader Configuration menu displays, type the following parameters in the Kernel parameters field:

ide0=0x1f0,0x3f6, ide1=0x170,0x376, ide2=0 ide3=0 ide4=0 ide5=0 ide6=0 ide7=0 ide8=0 ide9=0

6.	Press Enter (or Click OK) to continue.

7.	At the Congratulations menu press Ctrl-Alt-F2.

8.	Type the following commands to instruct Linux to install the new driver when it boots:

# chroot /mnt/sysimage
# mount /dev/fd0 /mnt
# cd /mnt
# umount /mnt
# exit

9.	Choose your selection after the driver sh setup-ft is successful.

10.	Press Ctrl-Alt-F7 and click the Exit button (or type exit and press Enter) to finish the installation.

# Existing Systems

Follow these instructions for installing FastTrak Series Linux Driver Version 1.02.0.20 into an EXISTING system:

## For RHF Upgraders

You are an RHF Upgrader if your PC runs Linux kernel version 2.4.x and your FastTrak controller card has one of the following Promise ASIC chips:

PDC20262 (FastTrak66)

PDC20265 (FastTrak100 Lite)

PDC20267 (FastTrak100)

PDC20270 (FastTrak100TX2)

If the ASIC chip on your FastTrak controller card does not match the numbers above or you are running a different Linux kernel, skip to the section For Non-RHF Upgraders below.

## To install the FastTrak Series Linux Driver into an EXISTING SYSTEM:

1.	Boot linux system and login as root.

2.	Insert FastTrak Driver Disk for install FastTrak Driver by issuing the following commands :
# mount /dev/fd0 /mnt/floppy
# cd /mnt/floppy
# sh install (Answer Yes/No when inquire setup configuration)
(You can answer Yes to load FastTrak driver when linux booting everytime)
# cd ..
# umount /dev/fd0

NOTE: All IDE channels except the first and second one are disabled 
do to the "Linux 2.4 ATA issue". It can be disabled or enable by removing 
the parameters in the
/etc/lilo.conf or /boot/grub/grub.conf if you are an RHF Upgrader

Please see Note section below for List of Parameters to add at the end of the corresponding boot configuration Lilo.conf or Grub.cof

3.	Boot the redhat linux system.

[NOTE]

"ide0=0x1f0,0x3f6, ide1=0x170,0x376, ide2=0 ide3=0 ide4=0 ide5=0 ide6=0 ide7=0 ide8=0 ide9=0"