Function Test Procedure

(Test plan W030611 for HPT3XX RAID serious product)

| Project: | 374-v202s-0711 |
| --- | --- |
| Date: | 2003-7-14 |
| Tester: | Yang, Jun & Li, Song |
| Test OS: | # Win98/ME/NT/2K/XP/2003 |

- RAID array operation/data reliability test:

- User manual check process:

- RAID array Server test (RAID-1):

- RAID array Server test (RAID-0):

- RAID array Server test (RAID-10):

- RAID array Server test (RAID-5):

- RAID array performance test:

| # RAID array Configuration: |  |
| --- | --- |
| # Test environment: | - M/B:GA-8IEXP - CPU: P4 2.4g - Memory: 512MB - HDD: MAXTOR 6L040J2*4 - OS: WINXP(SP1) |

- Winbench 99

| WinBench | single | RAID1 | RAID5 | RAID0 | JBOD | RAID0/1 |
| --- | --- | --- | --- | --- | --- | --- |
| Disk business Winmark | 10800 | 12000 | 8080 | 12500 | 11100 | 11400 |
| Disk High-End Winmark | 25900 | 26900 | 19700 | 43500 | 25800 | 33400 |
| Disk Transfer Rate Beginning | 41600 | 62000 | 93600 | 97100 | 40700 | 83400 |
| Disk Transfer Rate End | 25300 | 42600 | 75900 | 101000 | 29600 | 64500 |
| CPU Utilize | 14.9 | 17.2 | 41.9 | 63.9 | 14.4 | 38.6 |

- Iometer

| Iometer | Single |  | RAID1 |  | RAID0 |  | RAID5 | RAID0/1 | JBOD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 1MB Sequential Read # /Queue Depth | HPT374 | FastTrak 133 | HPT374 | FastTrak 133 | HPT374 | FastTrak 133 | HPT374 | HPT374 | HPT374 |
| 1 | 39.97714 | 39.98121 | 51.56558 | 24.25155 | 88.66525 | 69.54524 | 73.65525 | 64.55855 | 39.87154 |
| 2 | 39.78455 | 39.98741 | 64.53625 | 36.62514 | 96.51254 | 79.88414 | 95.58441 | 96.85254 | 39.82554 |
| 4 | 39.88471 | 39.97854 | 72.26365 | 36.62545 | 95.82514 | 79.86525 | 94.85525 | 98.86552 | 39.88715 |
| 8 | 39.77152 | 39.97821 | 72.32251 | 36.65821 | 95.86655 | 79.94414 | 94.68525 | 98.67445 | 39.92656 |
| 16 | 39.76552 | 39.92154 | 73.66925 | 36.68741 | 95.63251 | 79.98822 | 94.56871 | 99.76555 | 39.95525 |
| 32 | 39.75547 | 39.82545 | 73.66415 | 36.82514 | 95.6625 | 79.88145 | 94.5625 | 98.26525 | 39.89552 |

| Iometer | Single |  | RAID1 |  | RAID0 |  | RAID5 | RAID0/1 | JBOD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 1MB Sequential Write # /Queue Depth | HPT374 | FastTrak 133 | HPT374 | FastTrak 133 | HPT374 | FastTrak 133 | HPT374 | HPT374 | HPT374 |
| 1 | 39.88414 | 40.02155 | 39.98125 | 39.84542 | 63.55251 | 78.55251 | 13.15254 | 31.15455 | 39.75525 |
| 2 | 38.92125 | 40.03251 | 39.85525 | 39.92214 | 62.36682 | 78.85251 | 13.16625 | 31.16254 | 39.86525 |
| 4 | 39.81254 | 40.03525 | 39.71455 | 39.86525 | 62.65851 | 78.65854 | 13.15482 | 31.18525 | 39.86625 |
| 8 | 39.85254 | 40.03258 | 39.93662 | 39.8625 | 64.52515 | 78.98841 | 13.15824 | 31.16254 | 39.86675 |
| 16 | 39.87152 | 40.03254 | 39.9771 | 39.93215 | 64.58414 | 79.87445 | 13.15525 | 31.14125 | 39.85525 |
| 32 | 39.76215 | 39.98714 | 39.82514 | 39.82154 | 65.86251 | 79.99621 | 13.17554 | 31.15258 | 39.71552 |

| Iometer | Single |  | RAID1 |  | RAID0 |  | RAID5 | RAID0/1 | JBOD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 1MB Random # Read # /Queue Depth | HPT374 | FastTrak 133 | HPT374 | FastTrak 133 | HPT374 | FastTrak 133 | HPT374 | HPT374 | HPT374 |
| 1 | 23.55154 | 23.22452 | 33.52525 | 23.11554 | 49.82515 | 30.22558 | 40.55251 | 42.85525 | 23.81256 |
| 2 | 23.55254 | 23.25154 | 39.82154 | 25.22541 | 52.56231 | 33.55525 | 45.56255 | 52.52545 | 41.45825 |
| 4 | 23.57416 | 24.12545 | 39.63251 | 26.66525 | 54.25525 | 35.47147 | 50.26251 | 56.71466 | 47.6525 |
| 8 | 24.76251 | 24.25425 | 40.12515 | 26.63582 | 58.42514 | 36.82514 | 53.48221 | 60.55254 | 37.78825. |
| 16 | 25.41452 | 25.54155 | 41.33625 | 26.35252 | 58.42514 | 38.77414 | 53.12545 | 64.45525 | 30.52555 |
| 32 | 25.43251 | 25.56214 | 44.56714 | 26.52541 | 58.33251 | 38.85214 | 53.03212 | 64.53258 | 30.66214 |

| Iometer | Single |  | RAID1 |  | RAID0 |  | RAID5 | RAID0/1 | JBOD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| # 1MB Random Write # /Queue Depth | HPT374 | FastTrak 133 | HPT374 | FastTrak 133 | HPT374 | FastTrak 133 | HPT374 | HPT374 | HPT374 |
| 1 | 22.41445 | 19.22525 | 18.37415 | 20.22155 | 42.87552 | 31.22541 | 11.81255 | 30.82555 | 37.81555 |
| 2 | 20.23689 | 20.25458 | 18.33925 | 21.23552 | 44.42515 | 36.55144 | 11.78251 | 30.86625 | 51.52515 |
| 4 | 20.12525 | 20.25452 | 19.83252 | 22.93651 | 45.55841 | 36.65771 | 11.93252 | 30.86554 | 54.25542 |
| 8 | 20.52515 | 20.23584 | 19.89 | 23.22515 | 51.56254 | 37.24555 | 11.86525 | 30.86652 | 45.56255 |
| 16 | 19.8652 | 18.23652 | 21.55254 | 23.25881 | 51.78621 | 36.62514 | 11.86255 | 30.86654 | 30.25885 |
| 32 | 19.7144 | 18.25874 | 21.65825 | 23.18447 | 51.78633 | 36.85254 | 11.86714 | 30.92214 | 29.71444 |

C . Sisoftware Sandra 2003

| Sisoftware Sandra 2003 | Single |  | RAID1 |  | RAID0 |  | RAID5 | RAID1/0 | JBOD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HPT374 | FastTrak 133 | HPT374 | FastTrak 133 | HPT374 | FastTrak 133 | HPT374 | HPT374 | HPT374 |
| File System Benchmark (kb/s) | 25176 | 25953 | 38424 | 26462 | 57355 | 45903 | 46812 | 46760 | 25848 |

D . HD Tach v2.61

| HD Tach v2.61 | Single |  | RAID1 |  | RAID0 |  | RAID5 | RAID1/0 | JBOD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | HPT374 | FastTrak 133 | HPT374 | FastTrak 133 | HPT374 | FastTrak 133 | HPT374 | HPT374 | HPT374 |
| Random Access Time | 11.9 | 12.2 | 9.8 | 10.2 | 12.7 | 12.2 | 12.2 | 10.4 | 12.2 |
| Read Burst Speed | 80 | 80 | 80 | 80 | 80 | 80 | 80 | 80 | 80 |
| Read Speed Maximum | 42342.0 | 44168.0 | 42383.0 | 44168.0 | 77872.0 | 61114.0 | 75241.0 | 65240.0 | 45608.0 |
| Read Speed Minimum | 21887.0 | 22104.0 | 21890.0 | 22104.0 | 7035.0 | 4830.0 | 15979.0 | 22632.0 | 21881.0 |
| Read Speed Average | 34977.4 | 34937.3 | 35015.8 | 34941.6 | 28273.0 | 36159.1 | 24670.5 | 36000.9 | 35065.7 |
| Write Speed Maximum | 25478.0 | 28435.0 | 26928.0 | 26229.0 | 50518.0 | 45807.0 | 13065.0 | 33715.0 | 29257.0 |
| Write Speed Minimum | 12119.0 | 6011.0 | 4324.0 | 4890.0 | 5230.0 | 6255.0 | 3684.0 | 4784.0 | 6304.0 |
| Write Speed Average | 20483.2 | 21609.9 | 21180.0 | 20830.0 | 35076.4 | 32733.6 | 10967.6 | 25686.8 | 22268.1 |
| CPU Utilization | 2.9 | 3.1 | 4.5 | 3.7 | 3.3 | 3.5 | 63.8 | 5.2 | 3.2 |

- RAID array compatibility test list:

| No. | MotherBoard | Result | Description |
| --- | --- | --- | --- |
| 1 | Intel D845EBG2(must be check) | PASS |  |
| 2 | GA-6VX7B-4x(must be check) | PASS |  |
| 3 | ABIT KT7E | PASS |  |
| 4 | ASUS P4T | PASS |  |
| 5 | TYAN S2460 | PASS |  |
| 6 | DFI AD77 | PASS |  |
| 7 | ASUS A7V133 | PASS |  |
| 8 | GA 8IEXP | PASS |  |

| No. | Hard Disk | Fdisk/Partition | Fully Format | RAID5 Zero build | Description |
| --- | --- | --- | --- | --- | --- |
| 1 | Maxtor 6L020J1(must be check) | PASS | PASS | PASS |  |
| 2 | Maxtor 6L040J2(must be check) | PASS | PASS | PASS |  |
| 3 | IBM IC35L180AVV207-1(must be check) | PASS | PASS | PASS |  |
| 4 | WD 2000JB(must be check) | PASS | PASS | PASS |  |
| 5 | Maxtor 4G160J8(must be check) | PASS | PASS | PASS |  |
| 6 | Seagate ST3120023A(must be check) | PASS | PASS | PASS |  |
| 7 | Seagate ST360021A | PASS | PASS | PASS |  |
| 8 | WD 800JB | PASS | PASS | PASS |  |
| 9 | Maxtor 5T020H2 | PASS | PASS | PASS |  |
| 10 | Seagate ST340016A | PASS | PASS | PASS |  |
| 11 | WD 300AB | PASS | PASS | PASS |  |
| 12 | WD 300BB | PASS | PASS | PASS |  |
| 13 | Seagate ST3120022A | PASS | PASS | PASS |  |