YAMAHA Corporation May. 28th, 1997

OPL3-SAx driver technical document

How to change functions

YAMAHA Corporation

System Development Dept.

Functions can be changed without compiling  in this OPL3-SAx driver. This document describes how we can do that. Following the instructions as below.

1. Execute resource editor. Generally resource editor is included in the software development package. (ex. Developer Studio in Visual C++ 4.0,  App Studio in Visual C++ 1.xx,  Resource Workshop in Borland C++)

2. Open “OPL3SA32.DLL” . We can edit resources (such as dialog box, bitmap and string table) included in the EXE or DLL using resource editor.

3. See string table. There are numbers in string table (Neutral). Each string has ID number. Driver sets functions as default value, but you can use ID number from 425 to 428 to change functions in value range. The following table shows the connections between each ID number and Functions setting.

| ID number | Functions setting | Value Range |
| --- | --- | --- |
| 424 | FM Volume Max Value | 0 - 31 (32 steps and volume is highest for value 31) |
| 425 | Mic+20dB | 0 (use default value as before), 1(disable when system start or hardware resource is set again.) |
| 426 | ZVPort | 0 (manual setting for SA2 board, auto setting for SA3 board and higher than SA3 board) 1 (disable) 2 (manual setting) |
| 427 | Force SA2 | 0 (disable), 1(enable) |
| 428 | Force SA3A | 0 (disable), 1(enable) |
| 429 | Use DMA Channel 7 | 0 (use DRQ3 as before), 1(use DRQ7 instead of DRQ3) |

N.B.: Use DMA Channel 7(ID 429) can not be used OPL3-SA2 board and lower than OPL3-SA2 board.

4. Change values and save. You will get a modified driver with new functions.