May 29, 1997

YAMAHA Corporation

Concerning 16 bit DMA support

The following is an overview of the specifications for 16 bit DMA support.

1) For the NT driver, 16 bit DMA is used for playback of a Wave-Out device in the following conditions. (Only for Full Duplex.)For all cases other than the above, 8 bit DMA will be used.

(A) 16 bit DMA will be used if either the PNPBIOS or the PNPISA driver indicated that a 16 bit DMA channel (channels 4--7) has been assigned as OPL3-SA3 DMA channel resource. However since it is possible only on NT 4.0 to obtain resource data from the PNPBIOS or install the PNPISA driver, NT 3.51 does not support 16 bit DMA in these cases.

[Caution]

If a 16 bit connected DMA channel (on the OPL3-SA3 side) is specifiedmanually in NT 3.51, the sound will be incorrect.

(B) If DMA channel 3 of the OPL3-SA3 is connected to DMA channel 7 of thepersonal computer, the driver can be customized so that 16 bit DMA can be selected by manual hardware settings from the control panel. To use 16 bit DMA, select channel 7 as the DMA channel for playback of the Wave-Out device.

In this case, hardware resource data will be written in manual configuration mode for the PnP ISA configuration register of the OPL3-SA3 specified by the driver.

In this case, 16 bit DMA can be used by both NT 3.51 and NT 4.0.

For driver customization, refer to the separate document "OPL3-SAx driver technical document: How to change functions" (FUNCCUST.DOC).

［Supplement］

If you customized driver and use OPL3-SA3/OPL3-SA3B, you can select DMAChannel 7 instead of DMAChannel 3 on full duplex on manual configuration.

If you configure HalfDuplex, DMAChannel 7 is not available, so select Channel 0 or Channel 1.

If you use OPL3-SA/OPL3-SA2, you can select DMAChannel 3, but can not select DMAChannel 7.

These are hardware configuration dialogbox on FullDuplex and HalfDuplex.

HalfDuplex

FullDuplex

Details for 16BitDMA which can be use:

| Hardware resource | (A) |  | (B) |  |
| --- | --- | --- | --- | --- |
| configuration | NT4.0 | NT3.51 | NT4.0 | NT3.51 |
| PnPBIOS | OK | - | OK | - |
| PnPISA.SYS | OK | - | OK | - |
| Manual configuration | NG | NG | OK | OK |

<Tip> In WindowsNT3.51, PnP ISA Enable driver is not available so configure hardware resource configuration using Control Panel.

2) Although transmission is 16 bit, the upper 8 bits are not received by the OPL3-SA3, meaning that the actual data transmission unit is 8 bits.