To : Users of Manual VA-502

Addendum (25-10800-12 )

This manual had been updated since it was printed :

1). Page 10,  Page 11, the jumper introduction of SRAM was added.

CPU to SRAM Data Transacting Mode Selection: SRAM

This jumper allows you to select the CPU-to-SRAM data read/write mode. If you install a Cyrix or IBM processor on this mainboard, please set at 2-3 pin pair and set the Linear Burst Mode of Chipset Features Setup.

2). Page 42, Page 43, Chipset Features Setup, the display has been changed. A new feature Linear Burst Mode was added.

Linear Burst Mode

When enabled, allows you to configure the CPU to SRAM data read/wirte mode. If you use a Cyrix CPU, select Enabled; if you use an Intel CPU or AMD-K5 CPU, please stay with the default value, Disabled.

3). Page 40, The options of Boot Sequence was changed into: A, C, SCSI (Default); C, A, SCSI; C, CDROM, A; CDROM, C, A; D, A, SCSI; E, A, SCSI; F, A, SCSI; SCSI, A, C; SCSI, C, A; C only; LS/ZIP, C.

4). Page 32, Item E, F in the table are:

| Item | Connector | Pin Type | Feature |
| --- | --- | --- | --- |
| E | SP_LED | 2-pin male | indicates the system is in Suspend mode, when the LED lit |
| F | Turbo Switch | 2-pin male | (Reserved) |

The note message was changed into:

NOTE : Software Turbo Speed feature is not supported.

5). Page 33, The note message was canceled and the connector Susp was introduced as below.

Suspend Switch Connector: Susp

The mainboard provides Suspend mode switch (2-pin male) connector. If the connector is onboard, it is used to enter the system into Suspend mode and Suspend LED (SP_LED) will light.

6). Page 48, the default setting of IRQ Sequence was:

9, 10, 11, 5, 7, 4, 3, 12, 15, 14 (Default).

7. Page 51, a new feature was added.

HDD S.M.A.R.T. Capability

“S.M.A.R.T” is the abbrevation of “Self-Mointoring, Analysis and Reproting Technology”. To enable it will assist you in preventing some (but not all) system down time due to hard disk drive failure.

The options are: Disabled, Enabled (Default).