FIC PT-2006 PCI SYSTEM BOARD

BOARD LEVEL COMPATIBILITY TEST REPORT

JUNE 1996

Document Number: 	MB-264

Product Version:  	V 2.1

BIOS Version:  	V 8.023J800

| Supervisor | Writer |  |
| --- | --- | --- |
| Mick Yen |  | Tony Chou |

FIC Research & Development Department

5F #52 Min-Chuan Road, Hsin-Tien City Taipei,Taiwan, R.O.C

Tel : 886-2-918-6001     Fax : 886-2-915-6728

Table of Contents

1. 	Introduction

2. 	Mainboard Features

3.	Mainboard CPU Support

4. 	System Specification for Hardware and Software Utilities

5. 	Jumper Function Verification

6. 	Field (Stress) Test : Dynamic Burn-in Tests

7. 	Base Operating System SCT/HCT Tests

8. 	Hardware Compatibility Test Suite

8.1 		Display Adapter Products

8.2		SCSI Adapter Products

8.3 		SCSI Peripheral Products

8.4 		IDE Fixed Disk Drive Products

8.5 		Floppy Disk Drive Products

8.6 		Network Card Products

8.7		ATAPI CD-ROM Device Products

8.8 		Printer Peripheral Products

8.9		Mouse Products

8.10		Sound Blaster Products

8.11		MPEG Video Products

8.12		FAX/Modem Products

8.13 	Power Supply Products

8.14 	SRAM

8.15		DRAM Modules

9. 	Software Compatibility Test Suite

9.1 		Base Operating Systems

9.2. 		Tested Software List

Communications, CAD, CD Titles (Games), CD Titles (Multimedia),

Databases, Graphics, Suites, Word Processing , Presentations, Spreadsheets, Programs, Diagnostic Programs, Tools, Time Management, Scanning

# 1. Introduction

FIC’s motherboards undergo a comprehensive suite of stringent software and hardware compatibility tests to ensure that they can not only run a complete range of operating systems and software packages, but that they can also be installed with a full selection of add-on cards and peripheral devices, such as VGA, SCSI, and network adapters as well as HDDs, FDDs, CD-ROM Drives and Fax/Modems. A board is deemed to have passed the compatibility testing suite when it has met the following criteria:

- All application software available for IBM compatible products works properly.

- All common internal and external hardware available to the IBM AT-Bus and PCI compatible systems functions properly.

- All hardware and software can be installed according to the instructions provided to the  user.

- All legal configurations (IRQ , I/O addresses, DMA levels ) function properly.

- All applications have been tested with and without drivers provided on the support disk.

- All operating systems have been installed and tested with each processor, with and without  the most critical mode selected by setup.

# 2. Mainboard Features

Model  Name:	PT-2006  PCB Rev  2.1

Product Name:	Pentium-Processor Based Mainboard

System BIOS:	AWARD 8.03J800

PCI IDE:	Internal PIIX3 PCI Master dual IDE

I/O:	Winbond W83877F

______________________________________________________________________________

# 3. Mainboard CPU Support:

| Intel CPUs | Status |  | Cyrix CPUs | Status |
| --- | --- | --- | --- | --- |
| P54C-75 | Pass |  | 6x86-P120+ at 100MHz | Pass |
| P54C-90 | Pass |  | 6x86-P133+ at 110MHz | Pass |
| P54C-100 | Pass |  | 6x86-P150+ at 120MHz | Pass |
| P54C-120 | Pass |  | 6x86-P166+ at 133MHz | Pass |
| P54C-133 | Pass |  | AMD CPUs | Status |
| P54C-150 | Pass |  | AMD-K5-PR75 | Pass |
| P54C-166 | Pass |  | AMD-K5-PR90 | Pass |
| P54C-200 | Pass |  | AMD-K5 PR100 | Pass |

# 4. System Specification for Hardware and Software Utilities

The mainboard hardware portion was configured as follows:

| Item | Description |
| --- | --- |
| System Core logic | Inter PIIX3 & TXC |
| System Cache | 0KB/256KB/512K Pipeline Burst SRAM |
| System Memory | Maximum: 128MB (Two Banks) |
|  | 72-pin single RAS and Double RAS RAM Module |
|  | Fast Page/EDO |
|  | 168-pin single RAS and Double RAS RAM Module |
|  | DIMM module supports 3.3V EDO or SDRAM |
| PCI Bus clock | Synchronous mode (depending on CPU Clock divided by 2) |
| System Slots | 4 PCI and 3 ISA slots |
| IO Core Logic | Winbond W83877F Support Normal, Bi-Dir, EPP, ECP mode and IR interface |
| PCI IDE | Inter PIIX3 internal Bus Master PCI IDE. |

# 5. Jumper Function Verification

| Description | Jumper Name | Default Option | Status |
| --- | --- | --- | --- |
| Speaker connector | SPK | Open |  |
| Keyboard lock and Power LED | KB_Lock | Open |  |
| Turbo LED connector | TB_LED | Open |  |
| CPU FAN connector | FAN | Open |  |
| Suspend Switch | SUP_SW | Open |  |
| IDE LED Connector | IDE_LED | Open |  |
| Reset Switch | RST | Open |  |
| Normal / Remote Power Supply Selector | J1 | Short |  |
| Remote Power Control Connector | PRW_CON | Open |  |
| Remote Power Switch | RPW_SW | Open |  |
| CPU Clock Rate | FREQ1/ FREQ2 | 2-3 /2-3 (2:3) |  |
| CPU Clock Selector | CLK1/CLK2 | 1-2 / 2-3 (66MHz) |  |
| CPU Voltage Sector | VR1 / VR2 | 1-2 / 1-2,3-4 (STD) |  |
| External Battery | EXTBAT | Open |  |

# 6. Field (Stress) Test : Dynamic Burn-in Tests

| Software Name | Duration | Result |
| --- | --- | --- |
| BAPCO 95 for Windows NT | 24 hrs. | Pass |
| Test configurations for NT | Intel P54C-166 MHz FIC 1st Grafx-65MP 1MB PCI VGA Card Adaptec AHA-2940UW ST-22550N SCSI On Board 64MB Memory |  |
| Novell Server version 4.1 | 48 hr. | Pass |
| Test configurations for Novell | Intel P54C-133 MHz Matrox Millennium 2MB WRAM PCI VGA Card Adaptec AHA-2940UW 4 set Conner CFA-540A IDE (Dual channel Disk mirror ) 3COM 3C595 10/100 PCI Netware Card (Connects to 8~10 workstations ) Olicom PC-3116 PCI Token-Ring Card (Connects to 3 worksations) On Board 32MB Burst EDO Memory Test Program: Novell Nwtest. |  |
| SCO Unix Open Desktop 5.0 | 24 hr. | Pass |
| Test configurations for SCO | Intel P54C-166 MHz FIC 1st Grafx-65MP 2MB EDO PCI VGA Card Goldstar GCD-540B APAPI 4X CD-ROM Conner 1.6GB CFS-1621A Novell NE2000 Plus (Connect 3 DOS TCP/IP Workstation) On Board 16MB Burst EDO Memory Test Program: Create a batch |  |
| IBM OS/2 Warp 3.0 | 24 hr. | Pass |
| Test configurations for OS/2 | Intel P54C-100 MHz FIC 1st Grafx-65MP 2MB EDO PCI VGA Card Panasonic CDR-581 4x CD-ROM Seagate 1GB ST-1080A On Board 8MB EDO Memory Test Program : Create a batch |  |
| Winstone 96 under Win 95 | 48hr. | Pass |
| Test configurations for Winstone 96 | Intel P54C-133 MHz Matrox Millennium 2MB WRAM PCI VGA Card Goldstar GCD-580B 8x CD-ROM Quantum 1.2GB FB-1280A On Board 16MB BEDO Memory Tests condition: Run continuous demo mode (Must skip dbase). |  |
| BACPO 95 for WFW V3.11 | 24hr. | Pass |
| Test configurations for BAPCo 95 under Windows for Workgroups V3.11 | Intel P54C-133 MHz Diamond Viper stealth 64 4MB VRAM PCI VGA Card Quantum 1.2GB FB-1280A On Board 16MB BEDO Memory Tests condition: Run continuous demo mode (Must skip dbase). |  |

# 7. Base Operating Systems Certification Tests

| Operating System | Company | HCT / SCT Name | Result |
| --- | --- | --- | --- |
| IBM OS/2 Version 3.0 | IBM | OS/2 SCT 3.0 | Pass |
| Test configurations | Intel P54C-100 MHz TEAC 1.44 MB FDD Seagate ST-31220A On Board 16MB Fast Page Memory |  |  |
| Windows NT Version 3.51 | Microsoft | Windows NT SCT 3.51 | Pass |
| Test configurations | Intel P54C-166 MHz YEDA -702J Adaptec AHA-2940UW Quantum FB-1280S 1.2GB SCSI HDD On Board 16MB EDO Memory |  |  |
| Windows 95 | Microsoft | Windows 95 SCT 4.0a | Pass |
| Test configurations | Intel P54C-133 MHz TEAC 1.44 MB FDD Quantum FB-1280A 1.2GB IDE HDD NEC CDR-1300 ATAPI 6x CD-ROM On Board 16MB EDO Memory |  |  |

# 8. Hardware Compatibility Test Suite

## 8.1 Display Adapter Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| ATI | Mach 64 | Graphic Pro-Turbo 2MB | PCI | Pass |
|  | Mach 64 DRAM | Mach 64 DRAM | PCI | Pass |
| Alliance | ProMtion 6410 | Alliance Pro Motion 6410 | PCI | Pass |
| Cardex | ET4000 W32 | Tseng W32P PCI VGA | PCI | Pass |
| Diamond | Stealth 64 VRAM | Stealth 64 2/4 MB VRAM (S3 968) | PCI | Pass |
|  | Stealth 64 | Stealth 64 PCI VGA Card (S3 964) | PCI | Pass |
|  | Stealth 64 DRAM | Stealth 64 2MB DRAM (S3 864) | PCI | Pass |
|  | Viper SE | Viper SE Weitek P9100 | PCI | Pass |
|  | Viper PCI | Viper PCI Weitek 9000 PCI | PCI | Pass |
| FIC | VGA-302P | Advance 2302 1/2 MB DRAM | PCI | Pass |
|  | Trio 32P | S3 732 1/2 MB DRAM PCI VGA | PCI | Pass |
|  | 1stGrafx- 64P | S3 764 1/2 MB DRAM | PCI | Pass |
|  | 1st Grafx-65MP | S3 765 1/2 MB DRAM | PCI | Pass |
|  | 1st Grafx-40MP | Cirrus 54M40 1/2 MB DRAM | PCI | Pass |
|  | 1st Grafx-46MP | Cirrus 5446 1/2 MB DRAM | PCI | Pass |
| Leadtek | WinFastT230 | Tseng ET4000/W32P | PCI | Pass |
|  | WinFastS400 | S3 964 2MB VRAM | PCI | Pass |
| Matrox | MAG-PCI/2 | Matrox PCI 2MB VRAM | PCI | Pass |
|  | Millennium 3D | Matrox 4MB VRAM | PCI | Pass |
| Orchid | Kelvin64-PCI | Cirrus 5434 2MB DRAM | PCI | Pass |
| Number 9 | Number 9 | #9 GXE 64Pro | PCI | Pass |

## 8.2 SCSI Adapter Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| Adaptec | AHA-1530P | Adaptec AHA-1530P ISA PnP SCSI | ISA PnP | Pass |
|  | AHA-1542 | Adaptec AHA-1542C ISA SCSI | ISA | Pass |
|  | AHA-2940 | Adaptec AHA-2940 PCI SCSI card | PCI | Pass |
|  | AHA-2940UW | Adaptec AHA-2940UW PCI SCSI card | PCI | Pass |
|  | AHA-3940 | Adaptec AHA-3940 PCI to PCI Bridge card | PCI | Pass |
|  | AHA-3985 | Adaptec AHA-3985 PCI SCSI card | PCI | Pass |
| Buslogic | BT-946 | Buslogic BT-946 SCSI | PCI | Pass |
|  | FlashPoint LT | Buslogic BA81C15 PCI SCSI | PCI | Pass |
| Feature | PnP 1630 | Feature Domain 9441 PnP ISA SCSI | ISA PnP | Pass |
| Domain | TMC-3260 | Feature Domain 36C70 PCI SCSI | PCI | Pass |
| FIC | SCSI-810P | NCR-810P PCI SCSI Card | PCI | Pass |
| NCR | 825S | NCR-825S PCI SCSI Card | PCI | Pass |

## 8.3 SCSI Peripheral Products

| ITEM | Vendor | Model Name | Product Description | Result |
| --- | --- | --- | --- | --- |
| Hard Disk | Conner | CFA-54OS | 540MB SCSI Hard Disk Drive | Pass |
|  | Quantum | LPS-540S | 540MB SCSI Hard Disk Drive | Pass |
|  |  | FB-1080S | 1GB SCSI Hard Disk Drive | Pass |
|  | Seagate | ST-32550W | 2GB SCSI Hard Disk Drive | Pass |
| CD-ROM | SONY | CDU-55S | SCSI CD-ROM Drive 2X | Pass |
|  |  | CDU-76E | SCSI CD-ROM Drive 4X | Pass |
|  | NEC | CDR-222 | SCSI CD-ROM Drive 2X | Pass |
|  |  | CDR-510 | SCSI CD-ROM Drive 4X | Pass |
| TAPE | Tandberg | TDC 3820 | 525MB SCSI Tape Drive | Pass |
|  | Wangtek | 5525ES | 525MB SCSI Tape Drive | Pass |
| DAT | Wangtek |  | SCSI DAT 2.0G | Pass |

## 8.4 IDE Fixed Disk Drive Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| Bigfoot | BF-1280 | 5.25” 1.2GB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | BF-2550 | 5.25” 2.5GB PIO mode 4/ DMA mode 1 | IDE | Pass |
| Conner | CFS-541A | 540MB PIO mode 3 / DMA mode 0 | IDE | Pass |
|  | CFA-850A | 850MB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | CFS-1080A | 1GB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | CFS-1621A | 1.6GB PIO mode 4/ DMA mode 1 | IDE | Pass |
| Fujitsu | M2684TA | 540MB PIO mode 3 / DMA mode 0 | IDE | Pass |
|  | M3684TA | 850MB PIO mode 3 / DMA mode 1 | IDE | Pass |
| IBM | DPEA-31080 | 1GB PIO mode 3 / DMA Mode 0 | IDE | Pass |
|  | DJAA-21080 | 1.7 GB PIO Mode 4 / DMA Mode 1 | IDE | Pass |
|  | DPEA-31080 | 1GB PIO mode 3 / DMA Mode 0 | IDE | Pass |
|  | DJAA-21080 | 1.7 GB PIO Mode 4 / DMA Mode 1 | IDE | Pass |
| Maxtor | 712626AP | 1.6GB PIO mode 3 / DMA mode 1 | IDE | Pass |
| Seagate | ST-3660A | 540MB PIO mode 3 / DMA mode 0 | IDE | Pass |
|  | ST-850A | 850MB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | ST-31220A | 1.2GB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | ST-51270A | 1.2GB PIO mode 4 / DMA mode 1 | IDE | Pass |
| Quantum | FB-1080A | 1GB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | FB-1280A | 1.2GB PIO mode 4 / DMA mode 1 | IDE | Pass |
| WDC | AC2540 | 540MB PIO mode 3 / DMA mode 1 | IDE | Pass |
|  | AC21000 | 1GB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | AC21200 | 1.2GB PIO mode 4 / DMA mode 1 | IDE | Pass |
|  | AC31600 | 1.6GB PIO mode 4 / DMA mode 1 | IDE | Pass |

## 8.5 Floppy Disk Drive Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| EPSON | SMD-1340-301 | 1.2MB Floppy Disk Drive | FDC | Pass |
|  | SMD-1304-301 | 1.44 MB Floppy Disk Drive | FDC | Pass |
| Mitsumi | D509V3 | 1.2MB Floppy Disk Drive | FDC | Pass |
|  | D35375 | 1.44 MB Floppy 3 Mode Disk Drive | FDC | Pass |
| Panasonic | JU-475-3AG1 | 1.2MB Floppy Disk Drive | FDC | Pass |
|  | JU-257 | 1.44 MB Floppy Disk Drive | FDC | Pass |
|  | JU-259 | 2.88 Floppy Disk Drive | FDC | Pass |
| Sony | MPF-520-1 | 1.44MB Floppy Disk Drive | FDC | Pass |
| TEAC | FD-55GF | 1.2 MB Floppy Disk Drive | FDC | Pass |
|  | FD-235HG | 1.44 MB Floppy Disk Drive | FDC | Pass |
|  | FD-235J | 2.88 Floppy Disk Drive | FDC | Pass |
|  | FD-235HF | 3 Mode MB Floppy Disk Drive | FDC | Pass |
| YEDATA | YD-380 | 1.2MB Floppy Disk Drive | FDC | Pass |
|  | YD-702J | 1.44 MB Floppy Disk Drive | FDC | Pass |

## 8.6 Network Card Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| 3COM | 3C507 | 3COM Etherlink II | ISA | Pass |
|  | 3C509 | 3COM Etherlink III | ISA | Pass |
|  | Combo 3C509B | 3COM Ethernet PnP ISA Network card | ISA PnP | Pass |
|  | 3C-590 | 3COM Etherlink III PCI | PCI | Pass |
|  | 3C-5950TX | 3COM Etherlink III 10/100 PCI | PCI | Pass |
| Accton | ACC40 | Accton ACC40 Ethernet card | ISA | Pass |
|  | EN-166X | PnP ISA Ethernet card | ISA PnP | Pass |
|  | PCI-40 | Digital DC1003C PCI Network | PCI | Pass |
| C-NET | CN970EBT | AMD AM97C970-A4 PCI Ethernet | PCI | Pass |
| Intel | Ether Express Pro/100 | Intel Ether Express Pro/100 Lan Adapter | PCI | Pass |
| IBM | IBM 04H8095 | IBM 73G2692 Auto LAN Streamer PCI | PCI | Pass |
| Olicom | OC-3136 | Olicom PCI Token-Ring card | PCI | Pass |
| SMC | SMC 8432BT | SMC Ether Power PCI Combo | PCI | Pass |
|  | SMC9332DST | SMC Ether Power 10/100 PCI | PCI | Pass |
| ZNXY | ZX-312 | Diagital DC-1003 PCI Ethernet | PCI | Pass |
| D-Link | DE-220 CAT | D-Link DE-2000 CAT Ethernet card | ISA | Pass |
| Intel | Express 16 | Intel Express 16 ISA Ethernet card | ISA | Pass |
| Novell | NE2000 Plus | Novell NE2000 Plus | ISA | Pass |
|  | NE2100 | Novell NE2100 | ISA | Pass |
| Thomas | TC-4045 | Thomas Corned Token-Ring | ISA | Pass |
| Corned | TC-4145 | Thomas Corned PnP ISA Token-Ring | ISA PnP | Pass |
| D-Link | EZ-220 CT | PnP ISA Ethernet card | ISA PnP | Pass |
| IBM | IBM 92G7632 | IBM 23H3004 Auto 16/4 Token-Ring | ISA PnP | Pass |
| Olicom | OC-3118 | Olicom ISA PnP Token-Ring Card | ISA PnP | Pass |
| SMC | EtherEZ | SMC Ethernet card | ISA PnP | Pass |

## 8.7 ATAPI CD-ROM Device Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| AZTECH | CDA668-01I | ATAPI CD-ROM Device 6X | IDE | Pass |
| Aelta | ODC-6101 | ATAPI CD-ROM Device 6X | IDE | Pass |
| Acer | 665A-002 | ATAPI CD-ROM Device 6X | IDE | Pass |
| GoldStar | GCD-R540 | ATAPI CD-ROM Device 4X | IDE | Pass |
|  | GCD-R542B | ATAPI CD-ROM Device 4X | IDE | Pass |
| Hitachi | CDR-7730 | ATAPI CD-ROM Device 4X | IDE | Pass |
|  | CDR-7830 | ATAPI CD-ROM Device 6X | IDE | Pass |
| NEC | CDR-273 | ATAPI CD-ROM Device 4X | IDE | Pass |
|  | CDR-1300A | ATAPI CD-ROM Device 6X | IDE | Pass |
| Panasonic | CDR-581-B | ATAPI CD-ROM Device 4X | IDE | Pass |
|  | CDR-581-M | ATAPI CD-ROM Device 4X | IDE | Pass |
| Sony | CDU55E | ATAPI CD-ROM Device 2X | IDE | Pass |
| Tatung | CD-650B | ATAPI CD-ROM Device 6X | IDE | Pass |
| TEAC | CD-561 | ATAPI CD-ROM Device 6X | IDE | Pass |
| Vertos | 600ESD | ATAPI CD-ROM Device 6X | IDE | Pass |
| Wearnes | CD-220 | ATAPI CD-ROM Device 4X | IDE | Pass |
|  | CD-620 | ATAPI CD-ROM Device 6X | IDE | Pass |

## 8.8 Printer Peripheral Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| HP | Lserjet III | HP lserjet III | LPT | Pass |
|  | Lserjet 5P | HP Lserjet 5P support IR Interface | LPT | Pass |
| D-Link | DE-EPP | D-Link EPP Pocket LAN | LPT | Pass |
| Up-select |  | External EPP SCSI BOX | LPT | Pass |
| Road | Express T305 | External Parallel Port Tape Backup | LPT | Pass |
| Runner | Express FD144 | External Parallel Port Floppy Drive | LPT | Pass |
|  | Express | External Parallel Port CD-ROM Driver | LPT | Pass |
|  | Express XD | External Parallel Port Hard Driver | LPT | Pass |

## 8.9 Mouse Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| Microsoft | Microsoft Mouse | Microsoft Serial - PS/2 Mouse | KB / Serial | Pass |
|  | Microsoft Mouse | Mouse Port Compatible Mouse 2.0A | Keyboard | Pass |
| Logitech | M-S34-6MD | PS/2 Compatible Mouse | Keyboard | Pass |
|  | S3-34 | Serial Mouse | Serial | Pass |
| Qtronix | Lynx-25P | PS/2 Compatible Mouse | Keyboard | Pass |
|  | Lynx-25S | Serial Mouse | Serial | Pass |
|  | Easy | Serial Mouse 3-Button | Serial | Pass |
|  | Kwik-200/PS2 | PS/2 compatible Mouse | Keyboard | Pass |
|  | Kwik-200/Serial | Serial Mouse | Serial | Pass |
|  | M-30P | PS/2 compatible Mouse | Keyboard | Pass |

## 8.10 Sound Blaster Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| AZTECH | Nova 16 Extea II | Aztech Sound Card | ISA | Pass |
|  | Waverider 32+ | Aztech Sound Galaxy Waverider 32+ | ISA | Pass |
| Creative | CT-2700 | Creative Sound Blaster | ISA | Pass |
| Diamond | Multimedia Kit 6000 | Diamond Viper Multimedia Kit 6000 | ISA | Pass |
| JAZZ | Sound card | Jazz sound card | ISA | Pass |
| AMD | Inter Wave | AMD AM78C201KC PnP Sound | ISA PnP | Pass |
| Creative | AWE32 | Creative AWE32 Sound Blaster | ISA PnP | Pass |
|  | CT-2890 | Creative ISA PnP Sound Blaster | ISA PnP | Pass |
|  | CT-2940 | Creative ISA PnP Sound Blaster | ISA PnP | Pass |
|  | CT-2950 | Creative ISA PnP Sound Blaster | ISA PnP | Pass |
|  | CT-3600 | Creative ISA PnP Sound Blaster | ISA PnP | Pass |
|  | CT-3792 | Creative ISA PnP Sound Blaster | ISA PnP | Pass |
|  | CT-3990 | Creative ISA PnP Sound Blaster | ISA PnP | Pass |
| Everex | Wave Machine | Crystal CS4332 PnP Sound card | ISA PnP | Pass |

## 8.11 MPEG Video Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| FIC | 1stGrafx-CP3 | S3 443 1/2 DRAM | PCI | Pass |
| Creative | Video Blaster SE-100 | Creative SE-100 MPEG card | ISA | Pass |
|  | Video Blaster RT300 | Creative RT-300 Video Capture board | ISA | Pass |
| Formosa | MPEG-MP020 | Formosa Full Function MPEG card | ISA | Pass |
|  | MPEG-MP021 | Formosa Full screen MPEG | ISA | Pass |
|  | TV-Tuner | Formosa TV-Tuner board | ISA | Pass |
| RealMagic | MPEG | Real Magic Full Function MPEG card | ISA | Pass |

## 8.12 FAX/Modem Products

| Vendor | Model Name | Product Description | Bus I/F | Result |
| --- | --- | --- | --- | --- |
| Creative | Modem Blaster | Creative Modem Blaster 28.8 | External | Pass |
|  | Phone Blaster | Creative Phone Blaster | Internal | Pass |
| Diamond | Telecommander | Diamond Telecommander 3500XL | Internal | Pass |
| HAYES | ACCUR | HAYES ACCURA 288V.FC + FAX | Internal | Pass |
| Megahertz |  | Internal 2.88 Modem | Internal | Pass |

## 8.13 Power Supply Products

| Vendor | Model Name | Product Description | Result |
| --- | --- | --- | --- |
| Enhance | E520 | 200W 110/ 220 V PS II | Pass |
|  | V520 | 200W 110/ 220 V PS II | Pass |
| Lead Year | ESP-2200 | 200W 110 / 220 V PS II | Pass |
|  | ES-2250 | 250W 110 / 220 Remote Power Supply | Pass |
| Seventeam | ST-230WHF | 230W 110 / 220 VPS II | Pass |
|  | ST-211WHF | 200W 110 / 220 VPS II | Pass |
| Sea Sonic | SSG-150G | 150W 110 / 220 VPSII | Pass |
|  | SSG-200G | 200W 110 / 220 V PSII | Pass |

## 8.14 SRAM

| Vendor | Part No | Cache Sizes | Description | Result |
| --- | --- | --- | --- | --- |
| Winbond | W25P010AF-8(ZB) | 256K | 32K x 8 Pipeline SRAM | Pass |
|  |  | 512K | 64K x 8 Pipeline SRAM | Pass |
| TM / VIA | T32M3232B-60 | 256K | 32K x 8 Pipeline SRAM | Pass |
|  |  | 512K | 64K x 8 Pipeline SRAM | Pass |
| Samsung | KMF32V589G-15 | 256K | 32K x 8 Pipeline SRAM | Pass |
|  |  | 512K | 64K x 8 Pipeline SRAM | Pass |

## 8.15 DRAM Modules

| Type | Vendor | Parts No | Description | Result |
| --- | --- | --- | --- | --- |
| Fast Page | Goldstar | GM71C18160AJ7 | 4MB 72pin RAM module 70ns | Pass |
|  |  |  | 8MB 72pin RAM module 70ns | Pass |
|  | Hyundai | HY514400A | 4MB 72pin RAM module 70ns | Pass |
|  |  | Hy514400A | 8MB 72pin RAM module 70ns | Pass |
|  |  | Hy514400A | 16MB 72pin RAM module 70ns | Pass |
|  | Hitachi | HM514400CLS7 | 4MB 72pin RAM module 70ns | Pass |
|  |  |  | 8MB 72pin RAM module 70ns | Pass |
|  | IBM | B101320BA-70 | 4MB 72pin RAM module 70ns | Pass |
|  |  | B014400J1E-70 | 4MB 72pin RAM module 70ns | Pass |
|  |  |  | 8MB 72pin RAM module 70ns | Pass |
|  |  | B014400J1D-70 | 8MB 72pin RAM module 70ns | Pass |
|  | Mitsubishi | M5M418160BJ-7 | 4MB 72pin RAM module 70ns | Pass |
|  |  | M5M4100BJ-7 | 4MB 72pin RAM module 70ns | Pass |
|  |  | FC1121 REV 1.0 | 8MB 72pin RAM module 70ns | Pass |
|  |  | M5M417400AJ-7 | 16MB 72pin RAM module 70ns | Pass |
|  |  | M5M417400BJ-7 | 16MB 72pin RAM module 70ns | Pass |
|  | Micron | MT-12D136G-7 | 4MB 72pin RAM module 70ns | Pass |
|  | NEC | UPD4217400LA-70 | 4MB 72pin RAM module 70ns | Pass |
|  | NEC | UPD424400LA-70 | 4MB 72pin RAM module 70ns | Pass |
|  | Samsung | KM416C1200AJ-L7 | 4MB 72pin RAM module 70ns | Pass |
|  |  | KM44C1000BJ-7 | 4MB 72pin RAM module 70ns | Pass |
|  | TI | TMS418160-70DZ | 4MB 72pin RAM module 70ns | Pass |
|  |  | TMS417400DJ | 32MB 72pin RAM module 70ns | Pass |
|  | Toshiba | TC514400ASJL-7 | 4MB 72pin RAM module 70ns | Pass |
|  |  | TC5117400BSJ-60 | 32MB 72pin RAM module 70ns | Pass |
| EDO | SEC | KM416C1204AJ-60NS | 8MB 72pin RAM module 60ns | Pass |
|  |  | KM44C4104AK-60NS | 16MB 72pin RAM module 60ns | Pass |
|  | MICRON | MT4C4007JDJ-60 | 4MB 72pin RAM module 60ns | Pass |
|  |  | MT4C4007JDJ-70 | 4MB 72pin RAM module 70ns | Pass |
|  | TOSHIBA | TC5118165BJ-70 | 8MB 72pin RAM module 70ns | Pass |
|  | SIEMENS | HYB5118165BSJ-60 | 4MB 72pin RAM module 60ns | Pass |
|  | MITSUMI | M5M418165BJ-60 | 8MB 72pin RAM module 60ns | Pass |
| Burst EDO | Micron | MT4LC2M8F4DJ-52 | 8MB 72pin RAM module 60ns | Pass |

# 9. Software Compatibility Test Suite

## 9.1. Base Operating Systems

The board is tested to ensure that it runs a complete range of Operating Systems and can thus operate in all network environments. The board’s Power Management and Remote functions are also tested.

| Operating System | Version | Language | Company | Status |
| --- | --- | --- | --- | --- |
| MS DOS | 6.22 | English | Microsoft | Pass |
|  | 6.22 | German | Microsoft | Pass |
| Windows for Workgroups | 3.11 | English | Microsoft | Pass |
| IBM DOS | 7.0 | German | IBM | Pass |
| MS Windows 95 | 4.0a | English | Microsoft | Pass |
|  | 4.0a | Chinese | Microsoft | Pass |
| Windows NT Advance Server | 3.51 | English | Microsoft | Pass |
| Windows NT Workstation | 3.51 | English | Microsoft | Pass |
| IBM OS/2 Warp | 3.0 | English | IBM | Pass |
|  | 3.0 | German | IBM | Pass |
| ACAD 386 R13 |  | English | Autodesk | Pass |
| Novell Network | 3.12 | English | Novell | Pass |
| Novell Network | 4.10 | English | Novell | Pass |

## 9.2 Software Applications

| Item | Descriptions | Version | Language | O.S. | Status |
| --- | --- | --- | --- | --- | --- |
| Communication | cc: Mail/Add-on |  | English | Windows | Pass |
|  | Telix |  | English | Windows | Pass |
|  |  |  | Chinese | Windows | Pass |
|  | WinFAX Pro | 4.0 | English | Windows | Pass |
|  | WinFAX Pro for Win 95 | 4.0 | English | Windows | Pass |
|  | LapLink | 6.0 | English | Windows | Pass |
|  | Norton Anywhere | 1.0 | English | Windows | Pass |
| CAD | 3D Design Plus | 2.0 | English | Windows | Pass |
|  | AutoCAD | R13 | English | DOS | Pass |
|  | AUTOCAD | R13 | English | Windows | Pass |
|  | Animator Pro | 1.3 | English | DOS | Pass |
|  | ORCAD |  | English | DOS | Pass |
|  | 3D Studio | 4.0 | English | DOS | Pass |
|  | Micro Station | 5.0 | English | DOS/Win | Pass |
|  | PCAD | 4.5 | English | DOS | Pass |
|  | ORCAD | 4.01 | English |  | Pass |

| Item | Descriptions | Version | Language |  | O.S. | Status |
| --- | --- | --- | --- | --- | --- | --- |
| CD Game | Wing Commander III |  | English |  | DOS | Pass |
| CD Title | Cinderella |  | English |  | Windows | Pass |
| Multimedia | LivingBook Grandma & Me |  | English | Windows |  | Pass |
|  | Top Gun |  | English | Windows |  | Pass |
|  | Multimedia SuperPack |  | English | Windows |  | Pass |
|  | Compton’s Interactive Encyclopedia |  | English | Windows |  | Pass |
|  | Adams Family |  | English | Windows |  | Pass |
|  | Black Rain |  | English | Windows |  | Pass |
| CD Title | Action | 3.0 | English | Windows |  | Pass |
|  | Calendar Address Book |  | English |  | Windows | Pass |
|  | The Ultrasound CD |  | English |  | Windows | Pass |
|  | TripMaker |  | English |  | Windows | Pass |
| Database | Approach | 2.1 | Chinese |  | Windows | Pass |
|  | Lotus Notes | 3.0 | English |  | Windows | Pass |
|  | Lotus Notes | 3.0 | Chinese |  | Windows | Pass |
|  | DBASE III PLUS | 1.1 | English |  | DOS | Pass |
|  | DBASE IV | 2.0 | English |  | DOS | Pass |
|  | MS FoxBase | 2.1 | English |  | Windows | Pass |
|  | MS FoxPro | 2.6 | English |  | Windows | Pass |
|  | MS Access for Windows | 2.0 | English |  | Windows | Pass |
|  | MS Access for Windows | 2.0 | Chinese |  | Windows | Pass |
|  | Paradox for Windows |  | English |  | Windows | Pass |
|  | Clipper | 5.2 | English |  | DOS | Pass |
|  | Dbase | 5.0 | English |  | Windows | Pass |
|  | FUNCKY II |  | English |  | DOS | Pass |
|  | CodeBase |  | English |  | DOS | Pass |
|  | Paradox | 4.5 | English |  | Windows | Pass |
| Graphics | Corel Draw | 4.0 | English |  | Windows | Pass |
|  | Corel Draw | 6.0 | English |  | Windows | Pass |
|  | CorelVentura for Windows |  | English |  | Windows | Pass |
|  | Windows Draw | 3.0 | English |  | Windows | Pass |
|  | Visio | 3.0 | English |  | Windows | Pass |
|  | Visio HOME |  | English |  | Windows | Pass |
|  | Designer | 4.1 | English |  | Windows | Pass |
|  | Photo Shop Pro | 3.0 | English |  | Windows | Pass |
|  | ABC Flowcharter | 3.0 | English |  | Windows | Pass |

| Item | Descriptions | Version | Language | O.S. | Status |
| --- | --- | --- | --- | --- | --- |
| Suites | Lotus SmartSuite | 3.0 | English | Windows | Pass |
|  | MS Works for Windows | 3.0 | English | Windows | Pass |
|  | MS Works for Windows 95 | 4.0 | English | Windows 95 | Pass |
|  | MS Office for Windows | 4.3 | English | Windows | Pass |
|  | MS Office for Windows | 4.3 | Chinese | Windows | Pass |
|  | MS Office for Windows 95 | 7.0 | English | Windows 95 | Pass |
|  | Framework IV |  | English | DOS | Pass |
|  | Geoworks |  | English | DOS | Pass |
|  | Symphony | 3.0 | English | DOS | Pass |
| Word Processing | Ami Pro for Windows | 3.1 | English | Windows | Pass |
|  | Ami Pro for Windows | 3.1 | Chinese | Windows | Pass |
|  | Word Pro for Windows 96 |  | Chinese | Windows | Pass |
|  | MS Word for Windows | 6.0 | English | Windows | Pass |
|  | MS Word for Windows | 6.0 | Chinese | Windows | Pass |
|  | WordPerfect for Windows | 6.0 | English | Windows | Pass |
|  | WordPerfect | 6.0 | English | Windows | Pass |
|  | PE III (DW3) | 4.0 | English | DOS | Pass |
| Presentations | MS PowerPoint | 4.0 | English | Windows | Pass |
|  | Freelance | 2.1 | English | Windows | Pass |
| Spreadsheets | Lotus 1-2-3 | 5.0 | Englsh | Windows | Pass |
|  | Lotus 1-2-3 for Win |  | English | Windows | Pass |
|  | MS Excel | 5.0 | English | Windows | Pass |
|  | MS Excel | 5.0 | Chinese | Windows | Pass |
|  | Borland Quattro | 3.0 | English | Windows | Pass |
|  | Quattro Pro | 5.0 | English | Windows | Pass |
|  | Improv | 2.1 | English | Windows | Pass |
| Programs | MS Visual Basic | 3.0 | English | Windows | Pass |
|  | MS Visual C++ | 2.0 | English | DOS | Pass |
|  | Borland Turbo C++ | 4.0 | English | Windows | Pass |
|  | Turbo C++ | 3.1 | English | Windows | Pass |
| Diagnostic | Speed for Win95 | 3.0 | English | Windows | Pass |
| Programs | QAPlus | 5.20 | English | DOS | Pass |
|  | Landmark(Speedcom) |  | English | DOS | Pass |
|  | QAPlus | 4.8 | English | DOS | Pass |
|  | Bapco95 for Win NT | 1.0 | English | Windows | Pass |
|  | PC Magazine Benchmark 96 |  | English | Windows | Pass |
| Tools | AfterDark | 3.0 | English | DOS | Pass |
|  | AfterDark | 3.0 | English | Windows | Pass |
|  | AfterDark for Win95 | 3.0 | English | Windows | Pass |
|  | Central Point Backup | 1.0 | English | Windows | Pass |
|  | Desq View | 2.6 | English | Windows | Pass |
|  | Desq View386 | 2.6 | English | Windows | Pass |
|  | MS Testing Tools |  | English | Windows | Pass |
|  | Norton Backup | 2.2 | English | Windows | Pass |
|  | Norton Commander | 4.0 | English | DOS | Pass |
|  | Norton Desktop | 3.0 | English | Windows | Pass |
|  | Norton Utility | 8.0 | English | DOS | Pass |
|  | Norton Utility | 8.0 | English | Windows | Pass |
|  | PC Tools | 9.0 | English | DOS | Pass |
|  | QAplus | 4.7 | English | DOS | Pass |
|  | Page Maker | 5.0 | English | Windows | Pass |
|  | MS-Plus 21Century | 4.0 | English | DOS | Pass |
|  | QEMM386 | 7.03 | English | DOS | Pass |
|  | Sidekick | 2.0 | English | DOS | Pass |
| Time | Project for Win95 | 4.1 | English | Windows | Pass |
| Management | Organizer | 2.1T | Chinese | Windows | Pass |
| Scan | PC-Cillin | 4.0 | English | DOS | Pass |
|  | VB66 | 6.6 | English | DOS | Pass |