FIC PAC-2003 PCI SYSTEM BOARD

BOARD LEVEL RELIABILITY TEST REPORT

Document Number:  MB-364

Product Version:  V1.2

BIOS Version:  V 6.19K9F9

| Manager | Supervisor | Writer |
| --- | --- | --- |

FIC Research & Development Department

5F #52 Min-Chuan Road, Hsin-Tien City Taipei,Taiwan, R.O.C

Tel : 886-2-918-6001     Fax : 886-2-915-6728

TABLE OF CONTENTS

1.	 Introduction

2. 	High Temperature/Humidity (Storage) Test

3. 	Low Temperature/Humidity (Storage) Test

4. 	Overstress Temperature (Operating) Test

5. 	Temperature Strife Test

6. 	Voltage Strife Test

7. 	AC Power Consumption Test

8. 	Hardware/Software Detailed Notes

9. 	QAPlus 7.0 for Windows 95 Testing Item Description

# 1. Introduction

To ensure that it can withstand even the most extreme climactic and electrical conditions, each FIC motherboard undergoes an exhaustive battery of reliability tests before being released for production. FIC conducts the tests based on the worst-case calculation of loading and timing, and utilizes overspec thermal, voltage and frequency strife testing to accelerate failure conditions in order to detect and resolve potential defects. A brief description of the objective and content of each of these tests is given below:

## High Temperature/Humidity (Storage) Test

The High Temperature Test is conducted to determine the resistance of the motherboard to elevated temperatures that may be encountered during service life either in storage (without protective packaging). The Humidity Test is conducted to determine the resistance of the motherboard to the effects of exposure to warm, highly humid conditions such as those encountered in tropical areas. This is an exaggerated environmental test, accomplished by the continuous exposure of the motherboard to high relative humidity at cycling elevated temperatures. These conditions impose a vapor pressure on the board during the test which constitutes the major force behind the moisture migration and penetration to the equipment.

## Low Temperature/Humidity (Storage) Test

The Low Temperature Test is conducted to determine the effects of low temperature on  the motherboard during storage without protective packaging or frequent use. The Humidity Test is conducted to determine the resistance of the motherboard to the effects of exposure to warm, highly humid conditions such as those encountered in tropical areas.

## Overstress Temperature (Operating) Test

The Overstress Temperature Test is conducted to determine the resistance of the motherboard to elevated temperatures that may be encountered while the board is in operation.

## Temperature Strife Test

The Temperature Strife Test is conducted to determine the ability of the motherboard and its components to withstand dramatic changes in temperature, ranging from 80°C to -20°C, while in operation.

## Voltage Strife Test

The Voltage Strife Test is conducted to determine the ability of the motherboard and withstand changes in voltage, from 3.10V to 3.70V.

## AC Power Consumption Test

The AC Power Consumption Test is conducted to measure the board’s power consumption in Full-On Mode, Doze Mode, and Suspend Mode, and ensure that it meets the US EPA Energy Star Program’s requirements.

## 1.1. Configuration of Equipment Under Test

| ITEM | MODEL | VENDER | REMARK |
| --- | --- | --- | --- |
| CPU | Pentium (54C) | Intel | 166MHz |
| DRAM | 64MB | HY | SIMM 16MB 70ns X 4pcs |
| CACHE | 256K | Tm | 60ns |
| VGA | Trio64v2/dx | S3 | 2MB DRAM |
| Monitor | SRC1491 | Shamrock | 15” |
| Hard Disk Driver | Caviar 2340 | WD | 340MB |
| Floppy Disk Driver | SMD-1000 | Epson | 1.44MB 3.5 inch |
| Keyboard | KB-5311 | Chicony | 101keys |
| COM1 | Onboard | NS | QAPlus Loopback |
| COM2 | Onboard | NS | QAPlus Loopback |
| LPT | Onboard | NS | QAPlus Loopback |
| Power Supply | SS-200GPX | Seasonic | 200w |

# 2. High Temperature/Humidity (Storage) Test

## 2.1. Test Equipment:

KATO SE-47CRT Chamber.

## 2.2. Lab Ambiance:

Temperature: 	22°C ~ 25°C.

Humidity: 	50% ~ 56% RH.

## 2.3. Test Conditions:

Non-Operation:

1. 	Temperature: 60°C.

2. 	Humidity: 98%.

3. 	Adjust temperature to 60°C at a rate of 20°C/hr to 25°C/hr ranges.

4. 	Adjust humidity at 98% RH.

5. 	Retain temperature and humidity for at least 48 hours.

6. 	Return to room temperature at a rate of 20°C/hr to 25°C/hr ranges.

7. 	Retain humidity at 50% RH.

8. 	High temperature/humidity (storage) chamber operating step table:

| Step | Temperature | Humidity | Detain time | Climb/descend time | Total time |
| --- | --- | --- | --- | --- | --- |
| 1 | 25°C | 50% | 1 hr |  | 1 hr |
| 2 | 25°C ~ 60°C | 50% ~ 98% |  | 2 hr | 3 hrs |
| 3 | 60°C | 98% | 48 hrs |  | 51 hrs |
| 4 | 60°C ~ 25°C | 98% ~ 50% |  | 2 hr | 53 hrs |
| 5 | 25°C | 50% | 1 hrs |  | 54 hrs |

## 2.4 Test Results:

| Description | Results |
| --- | --- |
| Mechanical | PASS |
| Functional | PASS |

# 3. Low Temperature/Humidity (Storage) Test

## 3.1. Test Equipment:

KATO SE-47CRT Chamber.

## 3.2. Lab Ambiance:

Temperature: 	22°C ~ 25°C.

Humidity: 	50% ~ 56% RH.

## 3.3. Test Conditions:

Non-Operation:

1. 	Temperature: -5°C.

2. 	Humidity: 0%.

3. 	Adjust temperature to -5°C at a rate of 20°C/hr to 25°C/hr ranges.

4. 	Adjust humidity at 0% RH.

5. 	Retain temperature and humidity for at least 48 hours.

6. 	Return to room temperature at a rate of 20°C/hr to 25°C/hr ranges.

7. 	Retain humidity at 50% RH.

8. 	Low temperature/humidity (storage) chamber operating step table:

| Step | Temperature | Humidity | Detain time | Climb/descend time | Total time |
| --- | --- | --- | --- | --- | --- |
| 1 | 25°C | 50% | 1 hr |  | 1 hr |
| 2 | 25°C ~ -5°C | 50% ~ 0% |  | 2 hr | 3 hrs |
| 3 | -5°C | 0% | 48 hrs |  | 51 hrs |
| 4 | -5°C ~ 25°C | 0% ~ 50% |  | 2 hr | 53 hrs |
| 5 | 25°C | 50% | 1 hrs |  | 54 hrs |

## 2.4 Test Results:

| Description | Results |
| --- | --- |
| Mechanical | PASS |
| Functional | PASS |

# 4. Overstress Temperature (Operating) Test

## 4.1. Test Equipment:

KATO SE-47CRT Chamber / QAPlus 7.0 for Windows 95 software.

## 4.2 Lab Ambiance:

Temperature: 	22°C ~ 25°C

Humidity: 	50% ~ 56% RH.

## 4.3. Test Conditions:

Operation:

1. 	Temperature: 0°C & 55°C.

2. 	Humidity: 0% & 90%.

3. 	Heat Resistance:

a. Adjust temperature from room temperature to 55°C/90%.

b. Retain this temperature/humidity for at least 24 hours of operation.

c. Retain humidity at 50% RH.

d. Return to room temperature 25°C within 1 hour.

4. 	Cold Resistance:

a. Adjust temperature from room temperature to 0°C/0%.

b. Retain this temperature/humidity for at least 24 hours of operation.

c. Retain humidity at 50% RH.

d. Return to room temperature 25°C within 1 hour.

- Heat resistance chamber operating step table:

| Step | Temperature | Humidity | Detain time | Climb/descend time | Total time |
| --- | --- | --- | --- | --- | --- |
| 1 | 25°C | 50% | 1 hr |  | 1 hr |
| 2 | 25°C ~ 55°C | 50% ~ 90% |  | 2 hr | 3 hrs |
| 3 | 55°C | 90% | 24 hrs |  | 27 hrs |
| 4 | 55°C ~ 25°C | 90% ~ 50% |  | 2 hr | 29 hrs |
| 5 | 25°C | 50% | 1 hrs |  | 30 hrs |

6. Low resistance chamber operating step table:

| Step | Temperature | Humidity | Detain time | Climb/descend time | Total time |
| --- | --- | --- | --- | --- | --- |
| 1 | 25°C | 50% | 1 hr |  | 1 hr |
| 2 | 25°C ~ 0°C | 50% ~ 0% |  | 2 hr | 3 hrs |
| 3 | 0°C | 0% | 24 hrs |  | 27 hrs |
| 4 | 0°C ~ 25°C | 0% ~ 50% |  | 2 hr | 29 hrs |
| 5 | 25°C | 50% | 1 hrs |  | 30 hrs |

## 4.4 Test Results:

| Description | Results |
| --- | --- |
| Mechanical | PASS |
| Functional | PASS |

# 5. Temperature Strife Test

## 5.1. Test Equipment:

KATO SE-47CRT Chamber.

YOKOGAWA HR1300 Thermal Recorder.

QAPlus 7.0 for Windows 95 Software.

## 5.2. Lab Ambiance:

Temperature: 	22°C ~ 25°C.

Humidity: 	50% ~ 56% RH.

## 5.3. Test Conditions:

Operation:

1. 	Set up for highest performance setting.

2. 	Attach QAPlus/fe loopback plug to COM1,COM2, LPT1.

3. 	Attach thermocouples on expected high temperature components.

4. 	Temperature Setting:

Climb from 25°C to 80°C and fall from 80°C to -20°C and climb from

-20°C to 25°C.

5. 	Humidity Setting:

45% when over 20°C, 0% when under 20°C.

6. 	Voltage Setting:

a. 4.75V for system supply voltage.

b. 3.45V for Intel P54C-166MHz supply voltage.

7. 	Clock Setting:

66MHz for Intel P54C-166MHz.

8. 	Need to switch the machine on and off 10 times at each temperature level (stay at off state 10 seconds each time) and check that the machine operates properly.

9. 	Thermal recorder sensor points:

| Channel 1 | DRAM | Channel 2 | CACHE |
| --- | --- | --- | --- |
| Channel 3 | VT82C586CD | Channel 4 | VT82C585VP/CD |
| Channel 5 | VT82C587VP/CD | Channel 6 | NC |
| Channel 7 | CL-GD5446 | Channel 8 | CPU |
| Channel 9 | PC87336 | Channel 10 | CHAMBER |
| Channel 11 | REGULATOR | Channel 12 | VGA RAM |
| Channel 13 | NC | Channel 14 | NC |

10. Temperature strife test chamber operating step table:

| Step | Temperature | Humidity | Detain time | Climb/descend time | Total time |
| --- | --- | --- | --- | --- | --- |
| 1 | 25°C | 45% | 1 hr |  | 1 hrs |
| 2 | 25°C ~ 40°C | 45% |  | 0.5 hr | 1.5 hrs |
| 3 | 40°C | 45% | 1 hr |  | 2.5 hrs |
| 4 | 40°C ~ 60°C | 45% |  | 0.5 hr | 3 hrs |
| 5 | 60°C | 45% | 1 hr |  | 4 hrs |
| 6 | 60°C ~ 65°C | 45% |  | 0.5 hr | 4.5 hrs |
| 7 | 65°C | 45% | 1 hr |  | 5.5 hrs |
| 8 | 65°C ~ 70°C | 45% |  | 0.5 hr | 6 hrs |
| 9 | 70°C | 45% | 1 hr |  | 7 hrs |
| 10 | 70°C ~ 75°C | 45% |  | 0.5 hr | 7.5 hrs |
| 11 | 75°C | 45% | 1 hr |  | 8.5 hrs |
| 12 | 75°C ~ 80°C | 45% |  | 0.5 hr | 9 hrs |
| 13 | 80°C | 45% | 1 hr |  | 10 hrs |
| 14 | 80°C ~ 10°C | 45% ~ 0% |  | 2 hr | 12 hrs |
| 15 | 10°C | 0% | 1 hr |  | 13 hrs |
| 16 | 10°C ~ 0°C | 0% |  | 0.5 hr | 13.5 hrs |
| 17 | 0°C | 0% | 1 hr |  | 14.5 hrs |
| 18 | 0°C ~ -10°C | 0% |  | 0.5 hr | 15 hrs |
| 19 | -10°C | 0% | 1 hr |  | 16 hrs |
| 20 | -10°C ~ -20°C | 0% |  | 0.5 hr | 16.5 hrs |
| 21 | -20°C | 0% | 1 hr |  | 17.5 hrs |
| 22 | -20°C ~ 25°C | 0% ~ 45% |  | 1 hr | 18.5 hrs |
| 23 | 25°C | 45% | 1 hr |  | 19.5 hrs |

## 5.4. Test Results:

| Chamber | CH1 | CH2 | CH3 | CH4 | CH5 | CH7 | CH8 | CH9 | CH10 | Results |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Setup | DRAM | Cache | 586 | 585 | 587 | 5446 | CPU | 87336 | Chamber |  |
| -20°C | -18.9°C | -14.9°C | -9.4°C | -6.4°C | -11.4°C | -12.0°C | 2.5°C | -16.9°C | -20.0°C | PASS |
| -10°C | -8.8°C | -4.7°C | 0.6°C | 3.4°C | -1.0°C | -2.1°C | 12.6°C | -7.0°C | -10.1°C | PASS |
| 0°C | 1.1°C | 5.1°C | 10.5°C | 13.2°C | 8.7°C | 7.8°C | 22.2°C | 3.0°C | -0.1°C | PASS |
| 10°C | 11.9°C | 15.6°C | 21.5°C | 24.0°C | 18.9°C | 18.5°C | 32.5°C | 13.5°C | 10.5°C | PASS |
| 25°C | 27.0°C | 30.8°C | 36.5°C | 39.0°C | 34.2°C | 33.5°C | 47.4°C | 28.7°C | 25.6°C | PASS |
| 40°C | 41.9°C | 45.3°C | 51.2°C | 53.4°C | 48.6°C | 47.9°C | 67.2°C | 45.0°C | 40.4°C | PASS |
| 60°C | 62.0°C | 65.2°C | 71.4°C | 73.4°C | 69.2°C | 68.1°C | 88.2°C | 65.4°C | 60.6°C | PASS |
| 65°C | 67.1°C | 70.3°C | 76.2°C | 78.4°C | 73.8°C | 73.4°C | 91.9°C | 70.5°C | 65.8°C | PASS |
| 70°C | 72.0°C | 75.2°C | 81.4°C | 83.3°C | 78.2°C | 78.3°C | 96.7°C | 75.4°C | 70.8°C | PASS |
| 75°C | 76.7°C | 80.0°C | 86.0°C | 88.0°C | 83.2°C | 83.0°C | 96.3°C | 78.2°C | 75.6°C | PASS |
| 80°C | 82.0°C | 85.0°C | 91.0°C | 92.6°C | 88.4°C | 88.2°C | 101.0°C | 83.7°C | 80.8°C | PASS |
| 85°C | 86.9°C | 90.2°C | 96.0°C | 97.8°C | 93.8°C | 93.4°C | 106.5°C | 88.6°C | 85.6°C | PASS |
| Chamber | CH11 | CH12 | CH13 | CH14 | CH15 | CH16 | CH17 | CH18 | CH19 | Results |
| Setup | Regulator | Vgaram | NC | NC | NC | NC | NC | NC | NC |  |
| -20°C | 6.2°C | -16.5°C |  |  |  |  |  |  |  | PASS |
| -10°C | 16.7°C | 6.5°C |  |  |  |  |  |  |  | PASS |
| 0°C | 26.2°C | 3.6°C |  |  |  |  |  |  |  | PASS |
| 10°C | 37.3°C | 14.2°C |  |  |  |  |  |  |  | PASS |
| 25°C | 51.8°C | 29.2°C |  |  |  |  |  |  |  | PASS |
| 40°C | 56.9°C | 43.9°C |  |  |  |  |  |  |  | PASS |
| 60°C | 77.6°C | 63.9°C |  |  |  |  |  |  |  | PASS |
| 65°C | 82.6°C | 69.1°C |  |  |  |  |  |  |  | PASS |
| 70°C | 87.4°C | 74.2°C |  |  |  |  |  |  |  | PASS |
| 75°C | 101.4°C | 78.9°C |  |  |  |  |  |  |  | PASS |
| 80°C | 105.2°C | 83.7°C |  |  |  |  |  |  |  | PASS |
| 85°C | 111.7°C | 88.9°C |  |  |  |  |  |  |  | PASS |

# 6. Voltage Strife Test

## 6.1. Test Equipment:

KATO SE-47CRT Chamber.

YOKOGAWA HR1300 Thermal Recorder.

GW PPS-1860 DC Source.

QAPlus 7.0 for Windows 95 Software.

## 6.2  Lab Ambiance:

Temperature: 	22°C ~ 25°C.

Humidity: 	50% ~ 56% RH.

## 6.3. Test Conditions:

Operation:

1. 	Set up for highest performance setting.

2. 	Attach QAPlus/fe loopback plug to COM1, COM2, LPT1.

3. 	Temperature setting: Fixed at 50°C.

4. 	Humidity setting: Fixed at 45%.

5. 	Voltage setting:

a. 4.75V for system supply voltage.

b. 3.1V to 3.70V for Intel P54C-166MHz supply voltage.

6. 	Clock setting: 66MHz for Intel P54C-166MHz.

7. 	Voltage strife test chamber & DC source operating step table:

| Step | Temperature | Humidity | Detain time | Voltage | Total time | Results |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 50°C | 45% | 1 hr | 3.10V | 1 hr | PASS |
| 2 | 50°C | 45% | 1 hr | 3.15V | 2 hrs | PASS |
| 3 | 50°C | 45% | 1 hr | 3.20V | 3 hrs | PASS |
| 4 | 50°C | 45% | 1 hr | 3.25V | 4 hrs | PASS |
| 5 | 50°C | 45% | 1 hr | 3.30V | 5 hrs | PASS |
| 6 | 50°C | 45% | 1 hr | 3.35V | 6 hrs | PASS |
| 7 | 50°C | 45% | 1 hr | 3.40V | 7 hrs | PASS |
| 8 | 50°C | 45% | 1 hr | 3.45V | 8 hrs | PASS |
| 9 | 50°C | 45% | 1 hr | 3.50V | 9 hrs | PASS |
| 10 | 50°C | 45% | 1 hr | 3.55V | 10 hrs | PASS |
| 11 | 50°C | 45% | 1 hr | 3.60V | 11 hrs | PASS |
| 12 | 50°C | 45% | 1 hr | 3.65V | 12 hrs | PASS |
| 13 | 50°C | 45% | 1 hr | 3.70V | 13 hrs | PASS |

# 7. AC Power Consumption Test

## 7.1. Test Equipment:

CHITAI 2402A Digital Power Meter.

HITACHI Oscilloscope V-1060.

One floppy disk drive, keyboard, VGA card or onboard & standard power supply.

## 7.2. Lab Ambiance:

Temperature: 	22°C ~ 25°C.

Humidity: 	50% ~ 56% RH.

## 7.3. Test Conditions:

Operation:

1. 	BIOS setup is default with P.M. enabled.

2.	256KB CACHE.

3. 	Test the following 3 configurations. Each configuration has 3 modes.

a. Intel P54C-166MHz & 8MB DRAM:

1. Full-on mode. / 2. Idle mode. / 3. Standby mode.

b. Intel P54C-166MHz & 16MB DRAM:

1. Full-on mode. / 2. Idle mode. / 3. Standby mode.

c. Intel P54C-166MHz & 64MB DRAM:

1. Full-on mode. / 2. Idle mode. / 3. Standby mode.

## 7.4. Test Results:

| Model | Full-on | Idle | Standby mode |
| --- | --- | --- | --- |
| Intel P54C-166MHz 8MB DRAM | 33.8W | 28.4W | 22.8W |
| Intel P54C-166MHz 16MB DRAM | 33.8W | 28.4W | 22.8W |
| Intel P54C-166MHz 64MB DRAM | 34.2W | 28.5W | 22.8W |

# 8. Hardware/Software Configuration Notes

## 8.1. Hardware Configuration Notes (Board Level):

IDE HDD cable should be 18 inches.

Only the motherboard will be placed inside the oven chamber.

Voltages are measured on the motherboard.

## 8.2. Software Configuration Notes:

QAPlus 7.0 for Windows 95

## 8.3. Setup Notes:

The chipset should be setup at highest performance.

## 8.4. QAPlus 7.0 for Windows 95 Testing Notes:

QAPlus 7.0 for Windows 95 testing should be conducted for the following items:

| CPU | Do all tests. |
| --- | --- |
| Memory | Do all tests. |
| VGA | All graphics modes up to 1280 x 1024 (view the display to make sure no jitter, flicker, etc.) |
| COM1 | Internal and external loopback. |
| COM2 | Internal and external loopback. |
| LPT1 | Internal and external loopback. |
| IDE HDD | Disk read/write. |
| FDD | Disk read/write. |
| Keyboard | Do all tests. |
| Mouse | Do all tests. |

# 9. QAPlus 7.0 for Windows 95 Testing Items Description

| 1.System board testing | 4.COM port testing |
| --- | --- |
| CPU Test | Internal Data Path Test |
| IC Data Paths Test | Internal Loopback Test |
| Refresh Interval Test | RTS/CTS(Ext. Loopback) |
| CMOS Test | DTR/DSR(Ext. Loopback) |
| Calendar /Clock Tests | 5.Hard disks |
| NPU Test | Controller Test |
| Speaker Test | Random Cylinder Access Test |
| 2.Memory testing | Linear Cylinder Access Test |
| Pseudo random | Butterfly Access Test |
| Walking bit left | 6.Floppy disks |
| Walking bit right | Sequential Seek/Verify Test |
| Inverse walking bit left/right | Sequential Read/Write Test |
| Checkerboard | Butterfly Access Test |
| Inverse checkerboard |  |
| Bit stuck high/low |  |
| Bus noise |  |
| 3.Video testing |  |
| Black And White |  |
| Linearity Test |  |
| Color Test |  |