Getting Started

2.

Running the BIOS Setup Program					O

Using the Brightness and Contrast Controls		O

Using the Glidepad									O

Using the Keyboard									O

Using Disk Drives									O

Using the Internal Numeric Keypad				O

Using the Connectors and Ports					O

Adding Memory										O

Using PC Cards										O

Turning the Speakers On and Off					O

Using a Microphone									O

# Running the BIOS Setup Program

Your computer is likely to have been properly setup and configured by your dealer prior to delivery.  However, you may find it necessary to use the computer’s BIOS (Basic Input-Output System) Setup program to change system configuration information, such as the current date and time, or your hard disk drive type.  The Setup program is stored in the Read-Only Memory (ROM), and can be accessed when you boot up the system.  The Setup program is accessed by pressing the F2 function key after powering on the Computer.

The settings that you specify within the Setup program are recorded in a special area of memory called the CMOS RAM.  This memory is backed up by a battery so that it will not be erased when you turn off or reset the system.  Whenever you turn on the computer, the system will read the settings stored in the CMOS RAM and compares them to the equipment check conducted during the Power On Self Test (POST).  If an error occurs, an error message will be displayed on the screen, and you will then be prompted to run the Setup Program.

The Setup program allows you easy access to a variety of configurable options.  At this point, however, you need only be concerned with the standard configuration (“Main”) options on page 1 of the Setup program.  Once you have the computer up and running, you will want to refer to the next chapter for a discussion of the Setup program’s power management and other configuration options.

To enter the Setup program, press the F2 function key when the following prompt appears:

Press <F2> to enter SETUP

This will display the first page of the Setup menu,  which is illustrated below.

| PhoenixBIOS Setup Utility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main | Advanced | Security | Power Savings | Boot |  |  | Exit |  |  |
|  |  |  |  |  |  |  |  | Item Specific Help |  |  |  |  |  |  |
|  |  |  | System Time : |  |  | [10 : 10 : 40] |  |  |  |  | <Tab, <Shift-Tab>, or |  |  |  |
|  |  |  | System Date : |  |  | [04/01/1996] |  |  |  |  | <Enter> selects field. |  |  |  |
|  |  |  | Diskette A : |  |  | (1.44/1.25 MB, 3 1/2") |  |  |  |  |  |  |  |
|  |  | 4 | Internal HDD |  |  | [2168MB] |  |  |  |  |  |  |
|  |  |  | Internal CD-ROM |  |  | [None] |  |  |  |  |  |  |
|  |  |  | Boot Display Device |  |  | [Both] |  |  |  |  |  |  |
|  |  |  | System Memory |  |  | 640 KB |  |  |  |  |  |  |
|  |  |  | Extended Memory |  |  | 15360 KB |  |  |  |  |  |  |
|  |  |  | CPU Type |  |  | Pentium with MMX |  |  |  |  |  |  |  |
|  |  |  | CPU Speed |  |  | 166 MHz |  |  |  |  |  |  |
|  |  |  | BIOS Version |  |  | 03D-0102 |  |  |  |  |  |  |
|  | F1 | Help |  | Select Item | ( - / + ) | Change Values |  | F9 | Setup Defaults |  |  |
|  | Esc | Exit |  | Select Menu | Enter | Select Sub-Menu |  |  | F10 | Previous Values |  |  |

Table 2 - 1	BIOS Setup Main Menu

- Note: If you do not press the F2 key at the correct time and the system attempts to boot, an error message may be displayed ( such as “Missing operating system”).  There is no cause for alarm.  Simply, press CTRL+ALT+DEL to reset the system, and then, at the appropriate time, press F2 to enter BIOS Setup.

- Item Specific Help

- The right side of the screen.  This area describes each parameter and its available settings.

- Menu Bar

- The top line of the screen.  Each of the five selections displays its own screen.

- Parameters

- The left side of the screen.  This area lists the parameters and their current settings.

- Key Status Bar

- The bottom part of the screen.  These lines display the keys available to move the cursor, select a particular function, and so forth.

## Changing Bootup Settings

The Main settings page of the Setup program will allow you to change the following information:

### System Time:

This option allows you to change the system time using the format hour:minute:second.  You can change the system date here and/or from your operating system command prompt.

### System Date:

This option allows you to change the system date using the format month, day, year.  You can change the system date here and/or from your operating system’s command prompt

### Diskette A:

This option allows you to specify the type of floppy disk drive mounted inside your computer.  The selections are: [2.88 MB, 3 ½”], [1.44/1.25 MB, 3 ½”], [720 KB, 3 ½”],  and [Disabled].  The computer comes equipped with a 3.5”, 1.44 MB floppy disk drive module.  Default is [1.44/1.25, 3 ½”].

### Internal HDD

This option has it’s own option menu window.  Setting this option to [Auto] will automatically detect and configure the hard disk drive type for drives that comply with ANSI specifications.  If type [User] is selected, you will then have to insert the proper Cylinders, Heads, and Sectors parameters into the appropriate selections.  If type [1-39] is selected, the user will select the pre-determined type of hard disk drive installed.  If type [CD-ROM] is selected a CD-ROM is installed here.  If type [ATAPI Removable] is selected, an ATAPI removalbe disk is installed here.  The default is [Auto].  The next setting on this page is the 32 Bit I/O.  This setting enables or disables the 32 bit IDE data transfers.  The default is [Disabled].

### Internal CD-ROM

This field reports whether there is a CD-ROM installed or not.  The default is [None].

### Boot Display Device

This option allows you to select the display device the systems boots to if both a CRT and a LCD are available.  If only the CRT is available, the system always boots to the CRT.  If only the LCD is available, the system always boots to the LCD.  The selections are: [Both], [CRT], or [LCD].  The default is [Both].

### System Memory

This field reports the amount of base (or conventional) memory found by the BIOS during its POST.  The value will not exceed 640 KB.

### Extended Memory

This field reports the amount of extended memory found by the BIOS during its POST.  The value displayed is the amount of memory located (above 1 MB) in the microprocessor’s memory address map

### CPU Type

This field reports the type of CPU used in the notebook computer.

### CPU Speed

This field reports the speed of the CPU used in the notebook computer.

### BIOS Version

This field reports the present version level of the installed BIOS.

| PhoenixBIOS Setup Utility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main | Advanced | Security | Power Savings | Boot |  |  | Exit |  |  |
|  |  |  |  |  |  |  |  | Item Specific Help |  |  |  |  |  |  |
|  |  | PS/2 Mouse |  |  |  |  | [Enabled] |  | "Disabled" prevents any |  |  |  |  |  |
|  |  | Plug & Play O/S : |  |  |  |  |  | [Yes] |  | installed PS/2 mouse |  |  |  |  |
|  |  | Secured Setup Configurations |  |  |  |  |  | [No] |  | from functioning, but |  |  |  |  |
|  | 4 | I/O Device Configuration |  |  |  |  |  |  |  | frees up IRQ 12. |  |  |  |  |
|  | 4 | Audio Options Menu |  |  |  |  |  |  |  | "Enabled" allows the |  |  |  |  |
|  |  |  |  |  |  |  |  | operating system to |  |  |  |  |
|  |  |  |  |  |  |  |  | determine whether to |  |  |  |  |
|  |  |  |  |  |  |  |  | enable or disable the |  |  |  |  |
|  |  |  |  |  |  |  |  | mouse. |  |  |  |  |
|  | F1 | Help |  | Select Item | ( - / + ) | Change Values |  | F9 | Setup Defaults |  |  |
|  | Esc | Exit |  | Select Menu | Enter | Select Sub-Menu |  |  | F10 | Previous Values |  |  |

Table 2 - 2	BIOS Setup Advanced Menu

## Advanced  Menu

- Setup Warning!	Setting items on this menu to incorrect values may

- cause your system to malfunction.

### PS/2 Mouse

Selecting [Disabled] prevents any installed PS/2 mouse from functioning, but will free up IRQ 12.  Selecting [Enabled] allows the OS to determine whether to enable or disable the mouse.  The default is [Enabled].

### Plug & Play O/S:

Select [Yes] if you are using a Plug & Play capable operating system.  Select [No] if you need the BIOS to configure non-boot devices for non Plug and Play O/S, such as DOS. The default is [Yes].

### Secured Setup Configurations

Select [Yes] if you want the system settings to be secured from change by a Plug & Play operating system.  The default is [No].

### I/O Device Configuration

This option has it’s own option menu window.  This menu page allows you to configure the port allocations.

#### Serial Port A:

Configure the Serial Port A using the following options:

[Disabled]			-	No Configuration

[Enabled]			-	User configuration

[Auto]			-	The BIOS or OS chooses the configuration

[OS Controlled]	-	OS configuration

The default for this option is [Auto].

#### Serial Port B:

Configure the Serial Port B using the following options:

[Disabled]			-	No Configuration

[Enabled]			-	User configuration

[Auto]			-	The BIOS or OS chooses the configuration

[OS Controlled]	-	OS configuration

The default for this option is [Auto].

##### Mode:

This option sets the mode for Serial Port B.  The default is [Normal].

#### Parallel Port:

Configure the Parallel Port using the following options:

[Disabled]			-	No Configuration

[Enabled]			-	User configuration

[Auto]			-	The BIOS or OS chooses the configuration

[OS Controlled]	-	OS configuration

The default for this option is [Auto].

##### Mode:

This option sets the mode for the parallel port using the following options:

ECP

Output only

Bi-directional

The default for this option is [ECP].

#### Floppy disk controller:

Configure the floppy disk controller using the following options:

[Disabled]			-	No Configuration

[Enabled]			-	User configuration

The default for this option is [Enabled].

### Audio Options Menu

This option also has it’s own option menu window.  This menu page allows you to configure the audio allocations.

#### Sound:

Configure the Sound device using the following options:

[Disabled]			-	No Configuration

[Enabled]			-	User configuration

[Auto]			-	The BIOS or OS chooses the configuration

[OS Controlled]	-	OS configuration

The default for this option is [Auto].

#### Modem:

Configure the Modem device using the following options:

[Disabled]			-	No Configuration

[Enabled]			-	User configuration

The default for this option is [Disabled].

| PhoenixBIOS Setup Utility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main | Advanced | Security | Power Savings | Boot |  |  | Exit |  |  |
|  |  |  |  |  |  |  |  | Item Specific Help |  |  |  |  |  |  |
|  |  | Set User Password |  |  |  |  |  | [Enter] |  |  | Supervisor Password |  |  |  |
|  |  | Set Supervisor Password |  |  |  |  |  | [Enter] |  |  | controls access to the |  |  |  |
|  |  |  |  |  |  |  |  |  | setup utility. |  |  |
|  |  | Password on boot : |  |  |  |  |  | [Disabled] |  |  |  |  |  |
|  |  | Fixed disk boot sector : |  |  |  |  |  | [Normal] |  |  |  |  |  |
|  |  | Diskette access : |  |  |  |  |  | [Supervisor] |  |  |  |  |  |
|  | F1 | Help |  | Select Item | ( - / + ) | Change Values |  | F9 | Setup Defaults |  |  |
|  | Esc | Exit |  | Select Menu | Enter | Select Sub-Menu |  |  | F10 | Previous Values |  |  |

Table 2 - 3	BIOS Setup Security Menu

## Security Menu

### Set User Password

This selection allows you to choose whether the system will require a password to be entered on boot for a user.  You will choose “Enabled” if you want a password, and “Disabled” if you  do not want a password.  The default is [Disabled].

### Set Supervisor Password

Press <Enter> to set a password.  Type the desired password, press Enter and type it again.  The system will now require the password to enter the Setup Utility.  To remove the password when the password is enabled, press Enter twice without typing any password in between the blanks.  After setting a password the “User Password” will then become enabled.  The default is [Disabled].

### Password on Boot:

This option “Enables” or “Disables” password entry onto the  boot. If the User or Supervisor Password is set up.  The default is [Disabled.]

### Password Check During  Resume

This selection allows you to choose if the system will require a password  to be entered upon resuming from a suspend mode.  You will choose “Enabled” if you want a password, and “Disabled” if you do not want a password.  The CapsLock and ScrollLock LED’s will flash in sequence for 30 seconds to indicate that a password is required to resume.  The LED’s will continue flashing again until any key is hit, or  until the correct password is entered.  The default is “Disabled”.

### Fixed Disk Boot Sector:

If set to Write protect, the hard disk boot sector will be protected from viruses that effect the hard disk drive boot sector (the most damaging type).  This also prevents accidental formatting of the hard disk drive.  If set to normal, the entire hard disk is accessible.  The default is [Normal].

### Diskette Access:

If set to [Supervisor], a user without the Supervisor Password will not have read or write access to the floppy disk drive.  If set to [User], the user and the supervisor will both have access right.  The default is [Supervisor].

| Phoenix NoteBIOS Setup Utility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main | Advanced | Security | Power Savings |  | Boot |  |  | Exit |  |  |  |
|  |  |  |  |  |  |  |  |  | Item Specific Help |  |  |  |  |  |  |
|  | Power Management Function |  |  |  |  |  |  | [Battery only] |  |  |  |  |  |  |  |  |  |
|  | Power Savings Level: |  |  |  |  |  |  | [Maximum Power Savings] |  |  |  |  | Select Power Management |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | mode. |  |  |  |  |  |
|  | Standby Timeout : |  |  |  |  |  | [1 Min.] |  |  |  |  |  |  |  |  |
|  | Suspend Timeout : |  |  |  |  |  | [5 Min.] |  |  |  |  |  |  |  |  |
|  | Suspend Mode : |  |  |  |  |  | [Suspend] |  |  |  |  |  |  |  |  |
|  | Auto Save to Disk : |  |  |  |  |  | [Off] |  |  |  |  |  |  |  |  |
|  | Hard Disk Timeout : |  |  |  |  |  | [1 Min.] |  |  |  |  |  |  |  |  |
|  | Video Timeout : |  |  |  |  |  | [Disabled] |  |  |  |  |  |  |  |  |
|  | Battery Low Suspend : |  |  |  |  |  |  | [On] |  |  |  |  |  |  |  |  |
|  | Resume on Modem Ring : |  |  |  |  |  |  | [On] |  |  |  |  |  |  |  |  |
|  | Resume on Time : |  |  |  |  |  | [On] |  |  |  |  |  |  |  |  |
|  | Resume Time : |  |  |  |  |  | [00:00:00] |  |  |  |  |  |  |  |  |
|  | F1 | Help |  | Select Item | ( - / + ) | Change Values |  |  | F9 | Setup Defaults |  |  |  |
|  | Esc | Exit |  | Select Menu | Enter | Select Sub-Menu |  |  |  | F10 | Previous Values |  |  |  |

Table 2 - 4	BIOS Setup Power Saving Menu

## Power Saving Menu

### Power Management Function

This selection allows you to select the Power Management Mode.

Choose “Disabled” to turn off Power Management.

Choose “Battery Only” to turn off Power Management only while connected to AC power, but will turn on Power Management while on battery power.

Choose “Always” to turn on Power Management all the time.

The default is [Battery Only].

### Power Savings Level:

Select the Power Management mode.  Choosing modes will change the system power management settings.

[Maximum Power Savings] conserves the greatest amount of system power.

[Maximum Performance] conserves power but allows the greatest system

performance.

[Customize] allows the user to alter these settings.

[Disabled] allows the user to turn off Power Management.

The default is [Maximum Power Savings].

### Standby Timeout

This selection allows you to choose the length of  time the system will be idle before entering the Standby Mode.  Standby Mode turns off various devices in the system, including the screen, until you start using the computer again. The default is [1 min].

### Suspend Timeout

This selection allows you to choose the length of time the system will be in the Standby mode before entering the Suspend Mode.  Suspend Mode will turn off all devices and stores all data until you press the Power On/Off switch.  It will then restore the system to the last working condition.  The default is [5 min].

### Suspend Mode

Selects the type of Suspend Mode.

If you choose [Save-to-Disk] the system will save its state to disk and power off.

If you choose [Suspend] the system will save its state, but remain in a low power mode.

If you choose [Suspend] then you also have the option of choosing [Save-to-Disk].

The default is [5 min].

### Auto Save to disk

Turn [On] or [Off] the Auto Save To Disk feature.  When Auto Save To Disk is turned on, the system will save its state to disk and then power off after being in Suspend mode for a period of time.

### Hard Disk Timeout:

The length of time the hard disk is inactive before the hard disk is put into the Power Savings Mode.  The default is [Disabled].

### Video Timeout:

The length of time the user input devices need to be inactive before the screen is put into the Power Savings Mode.  The default is [Disabled].

### Battery Low Suspend:

By selecting [On] the system will enter the Suspend Mode upon a battery low condition.  Selecting [Off] the system will not enter the Suspend Mode upon a battery low condition.  The default is [On].

### Resume on Modem Ring:

Turning this feature “On” will wake the system up when an incoming call is detected on your modem or fax.  If [Suspend Mode] is supported on this machine, Resume on Ring will not work if Suspend Mode is set to [Save-To-Disk].  The default is [Off].

### Resume on Time:

Turning this feature on will wake up the system at a specified time if [Suspend Mode:] is supported on this machine, the Resume on Time feature will not work if Suspend Mode is set to [Save-To-Disk].

### Resume Time:

Specify the time when the system is to wake up.  Enter the time values by directly entering the numbers or by using the <Tab>, <Shift-Tab>, or <Enter> to select the fields.  Use  < - >  to change the settings in each field.  Time is set in 24 hour format.

| Phoenix NoteBIOS 4.0 Setup - Copyright 1985-95 Phoenix Technologies Ltd. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Main | Advanced | Security | Power Savings |  |  | Boot |  |  |  | Exit |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Item Specific Help |  |  |  |  |  |  |  |  |  |
|  |  | 1. | [Diskette Drive] |  |  |  |  |  |  |  |  | Use the < > or < > to |  |  |  |  |  |  |  |  |  |
|  |  | 2. | [Hard Drive] |  |  |  |  |  |  |  |  | select a device, then |  |  |  |  |  |  |  |  |
|  |  | 3. | [ATAPI CD-ROM Drive] |  |  |  |  |  |  |  |  | press < > to move it up |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | the list, or < > to |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | move it down the list. |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | Press <Esc> to exit this |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  | menu. |  |  |  |  |  |  |  |  |
|  | F1 | Help |  | Select Item | ( - / + ) | Change Values |  |  |  |  | F9 | Setup Defaults |  |  |  |  |  |  |
|  | Esc | Exit |  | Select Menu | Enter | Select Sub-Menu |  |  |  |  |  | F10 | Previous Values |  |  |  |  |  |  |

Table 2 - 5	BIOS Setup Boot Menu

## Boot Menu

To select the boot device, use the up and down arrows, then press < + > to move the device up the list, or < - > to move it down the list.  Press <Esc> to exit this menu.

| PhoenixBIOS 4.0 Setup Utility |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Main |  | Advanced |  |  | Security |  | Power Savings |  | Boot |  |  |  | Exit |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Item Specific Help |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Exit Saving Changes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Exit System Setup and |  |  |  |  |  |  |  |
|  |  |  | Exit Discarding Changes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | save your changes to |  |  |  |  |  |  |  |
|  |  |  | Load Setup Defaults |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | CMOS. |  |  |  |  |  |  |
|  |  |  | Discard Changes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Save Changes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | F1 |  | Help |  | áâ |  | Select Item |  |  | ( - / + ) |  | Change Values |  |  | F9 |  | Setup Defaults |  |  |  |  |  |  |
|  |  | Esc |  | Exit |  | ß à |  | Select Menu |  |  | Enter |  | Select 4 Sub-Menu |  |  |  | F10 |  | Previous Values |  |  |  |  |  |

Table 2 - 6	BIOS Setup Exit Menu

## Exit Menu

### Exit Saving Changes

Pressing this will cause the program to Exit after writing all changed SETUP item values to CMOS.  Only Power Management and Security Setting changes will take effect immediately, all other changes will take effect the next time the system is booted.

- Warning - Any changes in password will still take effect whether you Save

Changes or Do Not Save Changes..

### Exit Discarding Changes

Pressing this will cause the program to Exit without saving changed SETUP item values.

### Load Setup Defaults

Pressing this will Load the Default values for all SETUP items.

### Discard Changes

Reads previous values from CMOS for all SETUP items.

### Save Changes

Writes all SETUP values to CMOS.

# Using the Brightness and Contrast Controls

On DSTN screens you can adjust the screen brightness and contrast levels using your computer hot keys.  On TFT screens you can adjust the brightness level only, using your computer hot keys.

- Note:		The hot keys for brightness and contrast are continuous keys.  They			will adjust as long as you hold them down.

## Brightness Control

Press Fn +  and Fn + ¯ to increase and/or decrease the brightness of the display respectively.  The brighter the screen setting, the more power is used during battery operation.

## Contrast Control

Press Fn + ® and Fn + ¬ to increase and/or decrease the contrast of the display respectively.  The higher the contrast setting, the more power is used during battery operation.

- Note: You can adjust the contrast only on DSTN LCD’s.

# Using the Glidepad

## Moving the Cursor

The embedded Glide Pad offers a unique and efficient way of pointing and selecting in a Windows environment.  The following figure shows the Glide Pad.

Glide Pad

Figure 2 - 1.   Glidepad Location.

The Glide Pad responds to finger movements on its surface.  To move the cursor, move your finger along the Glide Pad surface.

Once the cursor is in the proper place, tap once on the surface of the Glide Pad or use the left button to click just the same as you would for a mouse.  Tap twice to double-click.

# Using the Disk Drives

Unlike a “floppy” diskette, a hard disk drive is rigid and completely sealed in a protective, dust-free environment.  A hard disk drive works very much the same as a diskette, but can retrieve and record data much faster and has a much larger storage capacity.

The computer is equipped with a 2.5” IDE (Integrated Drive Electronics) drive.  This type of drive embodies the latest in fast, reliable mass storage by integrating all of the control circuitry necessary for its operation directly onto the drive itself. This in turn allows the hard drive manufacturer to carefully optimize drive performance.  Once you have installed an operating system onto your hard drive, you will want to install all of your software applications onto it as well, and perform your work from the hard disk, using diskettes only for backup and archival purposes.

# Caring  for the Hard Disk Drives

The hard disk drive included with the computer is designed for portability, but it is not indestructible. Moreover, while modern hard disk drives are exceptionally reliable, they do fail occasionally.  To avoid damaging your disk drive, and to protect your valuable data, you should take the following precautions:

Never turn off or reset the computer while the HDD status icon is lit.

Make regular backups of your hard disk drive to diskettes, and keep a backup copy on hand.

Keep the AC adapter at least six (6) inches away from your hard disk drive(s).

# Using the CD-ROM Drive

The computer comes with a 20X (or higher) speed CD-ROM drive that is compatible with existing music and data CD’s.

This is a Class 1 Laser product.

## Inserting a CD

Press the eject button on the CD-ROM drive to eject the disk tray.

Place the CD disk onto the tray with the label side up.

Push the tray in by hand to return the tray to the drive.

## Handling CDs

Follow these guidelines to avoid damaging your CDs and to prolong the life of your CD-ROM player:

- When removing a disk from it protective cover or when loading a disk into a drive, hold the disk by its central core hole and an outer edge.  Never touch the disk’s data surface (non-labeled side).

- Never write on a disk or place a label on the disk surface.

- To protect the disk against scratches and dirt when not in use, always keep the disk in its protective case or cover.

- When cleaning a disk, always wipe in the direction of the center to the outer edge.  Don’t wipe the CD in a clockwise or counterclockwise direction.

- To remove dust or fingerprints, use a clean, soft and dry cloth.  Never use benzene or anti-static fluids.

- Keep disk away from high temperatures or direct sunlight.

## CD-ROM Drive Guidelines

Failure to observe the following precautions can damage both the CD-ROM drive and the data on the CD:

- Do not open the disk tray except when inserting or removing a disk.

- Never push down on an open disk tray.

# Using the Keyboard

The computer is equipped with either an 87- or 88-key keyboard, which provides all the functionality of a full-sized desktop 101-key keyboard.  Even if you are an experienced PC user, you should take a moment to become familiar with the layout of your computer’s keyboard.  In particular, you should familiarize yourself with the special computer function keystrokes which allow you to quickly and easily control and access the power management hot keys.

The computer has many special keys, most of which depend on an application for their functionality.

Figure 2 - 2	Keyboard Layout

## Special Keys

The following keys have special functions at the command level of MS-DOS and within many different programs:

Pause

Stops a command or application; primarily used to stop the screen from scrolling; pressing any other key resumes the execution of the command or application.

Shift+PrtSc

Sends the contents of the screen to the printer port; prints only text characters unless you have run the GRAPHICS.COM utility to enable printing graphics.

Ctrl+Break

Terminates the current command or application.

Ctrl+P

Sets the computer to echo keystrokes to the printer; prints a line when you press Enter; continues until you press Ctrl+P again.

Ctrl+Alt+Del

Terminates all programs, reloads MS-DOS and executes the AUTOEXEC file; also called “warm start” or “warm boot’.

The following keys are the system hot keys for your computer:

Fn + F2

Standby

Fn + F3

Suspend Mode

Fn + F4

Suspend to Disk

Fn + F7

PC Speaker On / Off

Fn + F10

Display Expand / Shrink

Fn + F11

LCD Backlight On / Off

Fn + F12

Toggles the screen output between the LCD, external CRT, or to LCD and CRT at the same time (called SIMULSCAN).

Fn + PgUp

Volume Increase

Fn + PgDn

Volume Decrease

Fn+®

Adjusts the contrast brighter.  (On DSTN LCD panels only.)

Fn+¬

Adjusts the contrast darker. (On DSTN LCD panels only.)

Fn+

Adjusts the brightness up.

Fn+¯

Adjusts the brightness down.

Fn + End

Audio Mute

## Using the Internal Numeric Keypad

The keyboard has an embedded keypad that provides the same functions as the discrete numeric keypad on an AT® enhanced keyboard.

The embedded numeric keypad keys generate AT-keypad characters and functions when pressed in conjunction with Num Lock, Fn and Shift.

The embedded numeric keypad has two modes you can enter by toggling the Num Lock hot key Fn + F10 or pressing the Num Lock Key on the keyboard (Num Lock) and is signaled by the Num Lock indicator: ON or OFF.

Num Lock On

When the Num Lock indicator is on, pressing a key generates the numeric characters shown.

Pressing Shift with a key generates the characters shown in the following figure.

Figure 2 - 3	The Keyboard Numerical Number Pad

If you press and hold Fn in the Num Lock mode, the keypad generates their normal characters.

Num Lock Off

When the Num Lock indicator is OFF, the keyboard acts as normal.

Pressing Fn with a key generates the same characters mentioned in the previous paragraph, Num Lock On (with Shift).

# Using the Connectors and Ports

USB Connector

This section provides a description of the connectors and ports on the rear and left side panels of the computer.

## The Rear Panel

Parallel Port

VGA Port

PS/2 Port

Serial Port

Expansion Port

Kensington Lock Slot

Figure 2 - 4	Rear panel of the Notebook Computer

PS/2 Port

Connects to an external PS/2 keyboard, numeric keypad, or mouse.

If you are connecting a keyboard with a 5-pin DIN connector, you will need to purchase a 6-pin mini-DIN adapter.  It also can support an optional “Y” connector.

Parallel Port

Connects to a parallel printer or other device that uses a standard parallel interface; EPP/ECP compatible; also connects to the floppy disk drive when used externally.

Serial Port

Connects to external devices such as a serial printer.  16550 UART compatible.

VGA Port (External Monitor)

Connects to an external analog monitor.  (15-pin)

USB Connector

The Universal Serial Bus (USB) connector allows you to connect up to 127 USB equipped peripheral devices (for example; printers, monitors, scanners) toyour notebook computer.

Expansion Port

Connects to a port replicator or Docking Station.

## The Left Side Panel

Battery Bay

DC in Jack

PCMCIA Card Slots

Power On / Off Switch

Figure 2 - 4.	Left Side Panel of the Notebook Computer

Power On/Off Switch

Use this switch to power on or off the notebook computer.  This switch will also be used to resume after a Save-to-Disk suspend.  You can also use this switch to resume from Standby or Save-to-RAM suspend without exiting the system.

DC In

Connect the AC adapter output connector to this jack to recharge the battery and to supply power to the computer.

- Caution: Use only the supplied AC adapter with your compute.  Other adapters could cause serious damage to the electronic circuits.

PC Card

The PC Card slots support one Type III or two Type I/II PC Cards.

## The Right Side Panel

Line-In Jack

Headphone Jack

Fax/Modem Slot

Line-Out Jack

Microphone Jack

FIR Window

Figure 2 - 5	Right Side Panel of the Notebook Computer

Headphone

Connects to an external set of headphones for private listening.

Line Out

Connects to a line out device such as amplified speakers.

Line In

Connects to a line-in device such as a synthesizer, stereo Walkman or

audio CD player.

Microphone

Connects to an external microphone.

Fax/Modem

There is space for an optional Fax/Modem Port in the center of the right side panel.

FIR

Connects to any IrDA compliant device (such as another IrDA computer or printer) without the use of a cord or cable.  Transmits up to 4 M bits per second.

# Adding Memory

Your computer has 16 MB / 32 MB / 64 MB of EDO Dynamic Random Access Memory (DRAM) for computer system memory.  The computer also allows you to install and expand the system memory via a single onboard SO-DIMM (Small Outline Double In-line Memory Module, 144 Pins) socket.  The SO-DIMM socket is easily accessible and can accept a 16 MB, 32 MB  or 64 MB SO-DIMM module, which are available from your dealer.

Whenever you change your computer memory size, you should run the PHDISK utility to re-size your Suspend to Disk file.  This file is used to store all your computer information onto the hard disk drive for the 0V Suspend function.  If you do not re-size the Suspend to Disk file, you will not be able to use the Suspend to Disk utility.

To enlarge the EDO memory of your computer it advisable to return the unit to an authorized dealer and have them install the additional memory chip.

# Using PC Cards

The computer is equipped with two PCMCIA sockets which can accommodate Type I/II or Type III cards conforming to the standards of the PCMCIA (Personal Computer Memory Card International Association).  These include the following types of devices:

## Flash Memory

A solid-state storage device that works like a floppy drive, with a write-protect switch on the edge of the card.  Unlike conventional memory, a flash memory card does not require an external power source to maintain data when removed from the socket.

## SRAM Memory

A static RAM memory device that can be formatted as a diskette or an IDE hard disk drive, with a write-protect switch on the edge of the card.  An on-card battery maintains the data when it is removed from the computer’s PCMCIA socket.

## Network

A device that connects the computer to a local area network (LAN) such as an Ethernet.

## Fax/Modem

A device that connects the computer to the telephone line for use as a fax and/or a modem.

## ATA Device

An AT Attachment, rotating or solid-state mass-storage device that works like an IDE hard disk drive.  An ATA device does not require an external power source to maintain data when removed from the computer’s PCMCIA socket.

## Inserting and Ejecting PC Cards

PC cards are inserted and ejected in much the same way as diskettes.

- Open the PCMCIA compartment cover on the left side of the computer.  (It is unnecessary to power off the system when handling PC cards and devices.)

- To insert a PC card, align the card with the appropriate socket and slide the card into the socket until it locks into place.  Note, that some PC Cards must be prepared by a PCMCIA driver, before you can use them for data storage, see your PC card manual for details.

- To eject a PC card, first ensure that the computer is not accessing the memory card or device, and then press the appropriate eject button on the socket.  When the card pops out of the socket, remove the card and store it properly.

## Configuring and Using PC Cards

To use a PC card with the computer, your operating system must be configured to recognize the device.  PCMCIA utility software is provided with the computer, containing the latest available drivers and a file containing instructions on how to use them.  The PCMCIA utilities provide the following features:

- an installation program for simplified installation and configuration of PCMCIA drivers and utilities.

- support for the Microsoft Flash File System II (FFSHII).

- a PC Format utility which can format a SRAM PC card as floppy disk drive D: or E:.

- an easy-to-use information utility for managing PC cards formatted with PC Format.

- support for hot insertion and removal of PC cards under DOS without the need to restart the computer.

- support for Power Management.

- support for booting from a PC card designated as a hard disk drive or floppy disk drive.

The following instructions should improve the relationship between the computer hardware and the supplied PCMCIA utility software under DOS.

### Installation

To install computer PCMCIA support, insert the SystemSoft PCMCIA driver utility and type A:Install.exe at the operating system command prompt.  Select the PCMCIA installation option from the setup menu and follow the instructions on the screen.

### Configuration

To configure or reconfigure PCMCIA software support after installation, change directions to the directory where you installed the PCMCIA software and type PCMSETUP at the prompt.  Follow the instructions on the screen.

### Documentation

To install the driver, insert the SystemSoft PCMCIA driver utility and type A:Install.exe at the operating system command prompt.  Select the appropriate computer documentation option from the Setup menu and follow the instructions on the screen.  To view PC card documentation, copy the USERMAN.ZIP file from drive A to drive C, decompress the file, then view the documentation in MS Word 6.0.

### Windows Installation

To install the PCMCIA driver for Windows 95, follow the steps listed below:

Insert the PCMCIA driver utility diskette into drive A.

- Double click, Control Panel / Add New Hardware / PCMCIA socket.

- Click “Have Disk” to change the driver.

- Follow the instructions on the screen to complete the installation process.

# Using the Audio System

The audio capabilities of the computer bring high quality stereo sound to your software applications, including features like an FM synthesizer, and digital recording and playback.  The computer’s integrated audio system is fully compatible with the Windows Sound System, and includes a number of applications on diskette which allow you to record, compress, store, and playback voice, sound, and music under the Windows environment.  The audio system is also Sound Blaster compatible, so you can hear all the digital sound effects and voice recordings used by the latest entertainment software.

The computer’s audio capabilities include the following features:

- Digitized audio playback capability (up to 44KHz), which faithfully plays back and reproduces all kinds of digitized sounds with a 16-bit Digital-to-Analog Converter(DAC).  The audio system works with a number of applications that utilize the digital voice channel to deliver realistic human speech and sound effects.

- Digitized audio recording capability (up to 44KHz) allowing digitization and recording of any kind of sound through the computer’s built-in microphone or an external source.

- High quality sound, dynamically filtered for low noise digital recording and playback.  Input from the computer’s built-in microphone uses Automatic Gain Control (AGC) to compress sound input to adapt dynamically to different recording conditions.

- Audio compression which offers real-time compression of digital audio at rates of 2:1, and real-time decompression at rates of 8:1.  Compression turns large sound files into smaller files that can easily be stored and transferred to a diskette or even sent over a network.

- Built-in amplifier and speaker, including volume control, and a sound-out port for connection to an external pair of headphones with power amplified speakers.

## Adjusting Volume

Computer sound volume can be adjusted using the speaker hot keys (Fn + <Pgup>) or (Fn + <Pgdn>).

## Audio Software

The audio system supports a large library of third party software packages include music/sound drivers.  All you have to do is select the correct music/sound driver when you install the package, or follow the package’s instructions for selecting the driver.  You may select the Windows Sound System, or Sound Blaster Pro.

The computer also includes software which includes a special driver and accessories that are designed especially for the Microsoft Windows environment.  These audio tools require Microsoft Windows 3.1 or a later version.

To install the audio driver for Windows 3.1, follow the steps below:

- Insert the Yamaha audio driver utility diskette into drive A.

- Run Setup.exe on the diskette from the Windows file manager.

- Follow the instructions on the screen to complete the installation process.

When using the optional audio system with the computer’s save-to-file feature enabled, it may occasionally be necessary to restart Microsoft Windows after a resume event in order to reinitialize the audio device drivers.

To install the audio driver for Windows 95, follow the steps below:

- Insert the Yamaha audio driver utility diskette into drive A.

- Double click “Control Panel / System / Device Manager”.

- Remove the “?” device.

- Double click  “Control Panel / Add New Hardware / Sound”.

- Follow the instructions on the screen to complete the installation process.

### Using the Video System

To install the NM2160 VGA driver for Windows 3.1, follow the steps below:

- Insert the NM2160 VGA driver utility diskette into drive A.

- Run Setup.exe on the diskette from the Windows file manager.

- Follow the instructions on the screen to complete the installation process.

To install the VGA driver for Windows 95, follow the steps below:

- Insert the NM2160 VGA driver utility diskette into drive A.

- Double click “Control Panel / Add New Hardware / Display Adapters”.

- Click “Have Disk” and select “NeoMagic MagicGraph 128ZV” to change the driver.

- Follow instructions on the screen to complete the installation process.

# Using the Microphone

Your computer has a built-in microphone located in the middle left corner of the Palm Rest cover.

To use the microphone, face your computer from a normal distance and speak in a normal voice.  You do not need to bend down to speak directly into the microphone.