Using Software

5.

Using Supplied Software				O

Guidelines for Installing Applications		O

Using System Passwords				O

Getting Help						O

# Using Supplied Software

Your computer is shipped with the following software:

- Power-saving utilities	-	The Phdisk commands and utilities (Save 				to Disk or Save to File)

- VGA Drivers		-	A Set of drivers for the many different				types of Displays available.

- Audio Drivers	-	A set of drivers for running the different				audio systems available

- Glide Pad Drivers	- 	A set of drivers for running the notebook				computers Glide Pad.

- PCMCIA Drivers	-	A set of drivers for running and accessing				the several types of PCMCIA Cards 					available.  (For a list of the available PC 				Cards, refer to www.fic.com.tw .)

- Modem Utilities	-	A set of drivers for the manufactures					recommended fax/modem used on the 				notebook computer.

- USB Drivers		-	A set of drivers for the manufacture’s				recommended USB connector for the 				notebook computer.

# Guidelines for Installing Applications

If you want to customize the Save-to-Disk partition size or reinstall MS-DOS, please refer to the following section.

### Save-to-Disk (0 Volt Suspend)

If this feature is enabled, when the computer enters Suspend mode the contents of memory is saved to your hard disk drive for retrieval when a resume event occurs.  The advantage of this state is that the battery power is further conserved because current can also be cut to memory when the computer enters the Suspend mode.  However, if Save-to-File is enabled, the resume process takes somewhat longer because a hard disk drive takes longer to power up to an operating condition.

### Save-to-RAM (5 Volt Suspend)

If the computer has not been configured for Save-to-File, then when the system enters Suspend mode the computer will become inactive, but memory continues to receive current so that stored information can be retrieved instantly whenever a resume event occurs.  This is called Suspend-to-RAM.

If Suspend-to-RAM satisfies your portable computing requirements, you can skip the next section and proceed to the section on Advanced Power Management.

# Installing an operating System

One of the most important features of a notebook computer is Power Management. Among all the different settings, Save-to-Disk is the most power-saving and their are two options when activating this feature.  One is Save-to-File and the other is Save-to-Partition.  For Save-to-File, the program will create a hidden file to which all data will be saved when STD (Save-to-Disk) is invoked.  For Save-to-Partition, the program will need a portion of the hard disk drive in which to save all data where all data on the computer will be saved.  We would recommend that you use the Save-to-File, because it is more flexible than Save-to-Partition.  The procedures on how to run PHDISK are described below.

- Save-to-File   -	After formatting the hard disk drive, insert the utility		diskette.  Run A:\sysutil\phdisk /c /f and then there will be 	a message saying “Your save file is named 	C:\SAVE2DSK.BIN and has a size of XXXX bytes.  It has 	System, Hidden, and Read Only attributes.”  You will need		to restart the computer to allow the BIOS to recognize the 	changes.

- Save-to-Partition -	You will need to delete the original partition of the hard disk	and then run the “PHDISK/Create/Partition” (or 	“PHDISK/C/P”).  The PHDISK utility program will 	automatically assign a disk size in reference to the installed 	system RAM that will be allocated for the STD partition.  	Insert the utility diskette which contains the PHDISK 	program.  Run A:\sysutil\phdisk /c/p.  You will the need 	to run FDISK, format the hard disk drive, and install the 	operating system later on.

- Note:

- There will always be a message in a red column during POST (Power On Self Test), before you run PHDISK.  It will say “Save to Disk file not found. Save to Disk feature is disabled.  Run Phdisk for information.  FILE: create new. PARTITION: consult manual, Hit any key to exit.”  This message will be gone after you run PHDISK.

- For more information on how to use the PHDISK utility, type “PHDISK” on the command prompt from the directory where the PHDISK command is located.

# Advanced Power Management (APM)

In addition to the power saving features designed into the computer’s hardware and ROM BIOS firmware, your computer is also compliant with the Advanced Power Management (APM) specification.  Through an APM device driver supplied with your operating system, the operating system is able to notify the computer’s ROM BIOS when system resources, such as keyboard, I/O ports, display panel, and microprocessor, are not in use.  The computer’s APM-aware ROM BIOS is then able to selectively power down system components that are not in use.

Windows provides you with APM which gives you effective power management while using the Windows environment.  From the Windows start button, point to Settings, then Control Panel.  In this window you will see an icon of a battery and an AC plug labeled Power.  Double-click this icon to display the Power Properties window.  From this window you can select advanced power management, standard power management, or disable the power management.  The window also displays a battery meter. This window also allows you to also select whether the battery meter icon is displayed beside the clock on the taskbar.

# Installing Computer Drivers and Utilities

After copying your application software to the hard disk drive, you may want to install the enhanced drivers and utilities software included with your computer.  The following driver and utility diskettes are provided:

Disk 1 — 	Contains VGA drivers for DOS/Windows

Disk 2 —	Contains VGA drivers for Windows 95, PHDISK and Glide Pad for

DOS/Windows

Disk 3 — 	Contains Glide Pad drivers for Windows 95

Disk 4 — 	Contains PCMCIA drivers for DOS/Windows

Disk 5,6 — 	Contains PCMCIA drivers for Windows 95

Disk 7,8,9 —	Contains Audio Drivers for DOS/Windows/Windows 95

Disk 10 —	Contains CD-ROM Drivers

(Comes with CD-ROM Option only)

# Using System Passwords

The computer has a USER password security system.

If the User password is set, the password must be entered to gain access to the Setup Security options: USER PASSWORD, DISKETTE ACCESS, AND FIXED DISK BOOT SECTOR.

The password prevents unauthorized access to the computer at system startup or when the computer resumes from a  Suspend mode if the PASSWORD CHECK DURING RESUME parameter in Setup is enabled

## Setting a System Password

- Caution:	If you forget the system password, you will not be able to use 			your computer.  To regain access, you will need to return the 			computer to your dealer.  This service is not covered by warranty.

To set a password in Setup, follow these steps:

- Press F2 during boot to enter Setup.

- Press the arrow keys to move to the Security Screen.

- Move down the screen to a Password parameter, then press Enter.

## Window pops up requesting a new password.

Enter the new password (up to seven(7) printable text characters) and press Enter, then retype the password for verification and press Enter.

- Note:	The password becomes active after you save and exit Setup.

If you set a password, you are prompted for the new password before starting your computer or entering Setup.

## Disabling or Changing a System Password

To disable or change a system password:

- Press F2 during boot to enter Setup.

- Enter your password when prompted.

- Press the arrow keys to move to the Security Screen.

- Move down the screen to a Password parameter, then press Enter.

- Note: 	You will only be able to change the Password if you used the password to 	enter Setup.

Enter a new password to set the  password, or press Enter with no entries to clear the password.  Verify your entry, then press Enter.

- Note: 	Save the changes then exit Setup.