Introduction to the

1.

lightweight Multimedia

NOTEBOOK COMPUTER

Introducing the Computer				O

Features of the Computer				O

Unpacking the Computer				O

Starting the Computer					O

Using the AC Adapter					O

Using the Indicator Lights				O

Environmental Specifications			O

Safety Precautions					O

Using the Computer					O

Power Saving Modes					O

Securing the Computer				O

# Introducing the Computer

This computer is a lightweight, compact, and fully IBM compatible computer, featuring the latest in portable computing technology.  Designed for a wide range of general business and personal productivity applications, this computer makes an ideal choice for use in the office, the schoolroom, at home, and/or on the road.  Standard features include a powerful Pentium microprocessor, fast PCI-bus VGA graphics, a high performance hard disk drive, a high density floppy disk drive, a powerful 16X speed CD-ROM disk drive, support for PCMCIA cards and devices,  support for FIR (IrDA Compatible), a built-in Glide Pad, and advanced power management capabilities to prolong battery power.

LCD Assembly

Keyboard

Li-Ion Battery Pack

Hard Disk Drive

CD-ROM Module

Floppy Disk Drive Module

Glide Pad

Figure 1 - 1.   Your Notebook Computer

# Features of the Computer

This notebook computer includes a variety of innovative features designed to meet the most demanding computer requirements:

A powerful Intel Pentium microprocessor, with 512 KB on board cache and math coprocessor, supporting suspend and resume capabilities under several different operating systems.

A flexible and field-upgradeable memory architecture supporting up to 128 MB of onboard EDO RAM.

Two PCMCIA expansion sockets which can accommodate one Type III or two Type I/II PC cards.

A detachable and rechargeable Lithium-Ion (Li-Ion) battery pack.

A PCI-bus video accelerator driving a color LCD display panel, with a wide viewing area for high resolution VGA compatibility.

A wide range of I/O (Input/Output) ports including: a serial and parallel device, an external keyboard and PS/2 mouse port, an analog monitor, wireless IR (Infrared) communication port, jacks for a microphone and stereo audio input/output, headphone output, an expansion bus port, USB port, and Fax/Modem jack.

A large integrated Glidepad, ergonomically placed to facilitate left or right handed use with today’s graphical user interfaces.

Advanced power management capabilities which conserve 90-95% of battery power by automatically shutting down inactive peripheral devices and system components.

An optional CD-ROM drive, compatible with all major data formats and music CD’s.

A 1.44 MB, 3.5” Floppy Disk drive.

An integrated audio system, compatible with Sound Blaster, with audio input and output ports, a built-in microphone and stereo speakers.

# Unpacking the Computer

Your computer comes securely packaged in a sturdy cardboard shipping carton.  Upon receiving your computer, open the carton and carefully remove the contents.  In addition to this User’s Manual, the shipping carton should also contain the following items:

- The Notebook Computer

- An AC Adapter and AC Power Cord

- Li-Ion Battery Pack(s)

- Utility Diskettes

- User’s Manual

Carefully inspect each component to make sure that nothing is missing and/or damaged.  If any of these items are missing or damaged, notify your dealer immediately.  Be sure to save the shipping materials and the carton in case you need to ship the computer or if you plan to store the computer away sometime in the future.

# Using the AC Adapter

The computer can be powered by either the rechargeable battery pack or the included AC adapter.  You will find detailed instructions on using both power sources in the next chapter.  Follow the steps below to prepare the battery pack and to attach the AC adapter.

Although the battery is fully charged when shipped from the factory, transit and shelf time would likely have reduced its charge.  Before using the computer for the first time, you should recharge the battery pack by attaching the AC adapter, as follows:

- Insert the battery into the battery bay with the label facing up.

- Connect the DC power cable from the AC adapter to the AC adapter plug on the rear of the computer, as illustrated below.  Be sure to always handle the adapter cords by the plugs only.

Plug the AC power cord into the end of the AC adapter.

Figure 1 - 2.   The AC Adapter

- Connect the male end of the AC power cord to an electrical outlet.

- Warning: Use only the AC adapter that came with your computer.  Adapters for other electronic devices (including other notebook computers) may look similar, but could damage your computer if used.

When you have connected the AC adapter to the computer and to an electrical outlet, this connection will now supply all of the power needed to run the computer and will also recharge the battery.  To preserve the battery’s charge under normal use, you should use the AC adapter to power the computer whenever you have access to an electrical outlet (proper voltage).

# Starting the Computer

Once you have connected the AC adapter, you are now ready to turn on the computer.  Follow the steps below to begin using your new computer:

- If you have not done so, press the release latch on the front of the computer and raise the LCD panel to a comfortable viewing angle.

- To power on the system, press the Power On switch firmly, located on the left side of the computer.  You will then see the power indicator LED light up.

After a few seconds, the computer will begin to execute an internal diagnostic program which is automatically ran whenever the computer is turned on.  The Power-On-Self-Test (or POST) checks the memory, the keyboard, the system board, and other components of your computer before the computer begins normal operation.

- Warning: Never turn off or reset the computer while a disk drive is in use; doing so can result in the loss of or in the destruction of all data.  Also, always wait at least five (5) seconds after turning off the computer before turning it back on; turning the power on and off in rapid succession could damage the computer’s electrical circuitry.  If the computer does not power on, press the hardware reset switch and then press the power switch again.  This additional step may sometimes be necessary with fresh battery packs or if the system has been idle for a very long period of time.

Battery Pack Bay

Power Switch

PCMCIA Slots

Figure 1 - 3	The Power (On/Off) Switch

# Using the Indicator Lights

The computer is equipped with a set of LED indicators located above the keyboard, which will inform you of the current operating status of your computer at a glance.  The status indicators are illustrated and described below:

## Standby

This LED informs you that the system has gone into the Standby mode.  To awake the system, hit any key.

## Suspend

This LED informs you that the system has entered into the Suspend mode.  To awake the system, push the Power On/Off switch.

## NumLock

This LED informs you that the keyboard number lock function is engaged.  When lit, the keyboard’s embedded keypad is engaged.

## Cap Lock

This LED informs you that the keyboard caps lock function is engaged.  When lit, the characters are entered in the upper case.

## Battery Charge

This LED informs you that the computer is charging the battery when lit.  If off, the battery has been fully charged.

- Note: If the computer has been left unused for a prolonged period of time, it is possible for the battery to become completely discharged.  In such cases, it is necessary to connect the computer to the AC adapter for approximately thirty (30) minutes before the battery will begin to recharge normally.

## Hard Disk Drive

This LED informs you that the hard disk is being accessed.  If this LED is on it advisable not to move the notebook or attempt to turn the notebook off.  Turning off the notebook when this LED is on could result in lose of data or the destruction of your hard disk drive.

## Media Access

This LED informs you that the computer is reading or writing to the systems media bay drives (Floppy or CD-ROM).

## Power On

This LED informs you  that the computer is powered on when lit and is located on the right hinge cover.  When the battery is running critically low, the LED will blink and the computer will emit an intermittent audible beep.  You should consider this condition as a warning to save your data immediately and prepare an alternative power source (use an AC outlet or install a fresh battery pack).

# Environmental Specifications

This section provides information on the optimum operating environment for your notebook computer.

## Temperature

Operating:	41° to 95°F

(5° to 35°C)

Storage:		14° to 140°F

(-20° to 60°C)

## Relative Humidity (Noncondensing)

Operating:	20% to 80%

Storage:		10% to 90%

## Shock

Operating :	Impact Accelerator :	1g/10g Based on half sine-wave

shock pulses of 11 Msec

Number of Shocks :	3 Shocks for each of the six faces

Direction of Movement :	Six Faces

Storage :		Impact Accelerator :	10g, 20g, 30g, 40g, 50g Based on

half sine-wave shock pulses of 11 Msec

Number of Shocks :	3 Shocks for each of the six faces

Direction of Movement :	Six Faces

## Vibration

Operating :	Direction of Movement :	X, Y, Z Axis

Vibration Level :		5 ~ 500 ~ 5 Hz

0.5g Acceleration

Duration Time :		60 mins for each Axis

Storage:		Direction of Movement :	X, Y, Z Axis

Vibration Level :		5 ~ 500 ~ 5 Hz

2g Acceleration

Duration Time :		60 mins for each Axis

# Safety Precautions

Follow all warnings and instructions which may be marked on the computer.

Except as described elsewhere in this manual, refer all servicing to qualified personnel.

Immediately shut off the computer and refer to servicing if any of the following conditions occur:

- when the power cord or plug has become damaged or frayed

- if liquid has been spilled onto the computer

- if the computer has been dropped or if the cabinet has been damaged

Never push objects of any kind into cabinet openings.  They may contact dangerous voltage points or will short out parts that could result in fire or cause electrical shock.

Keep all liquids away from the computer and its accessories.

Turn off the computer before installing or removing a peripheral device.

Turn off the computer and disconnect the AC adapter before cleaning.

# Using the Computer

Never pick up or carry the computer by the display.

Never use the computer in harsh environments where it could be subjected to rapid temperature changes and/or excessive dust.

Never expose the computer to excessive vibration.

Never expose the hard disk drive or floppy disk(s) to strong magnetic fields, such as those generated by audio system speakers, telephone handsets, or hand-held metal detectors.

To avoid overheating the computer, never place anything over the top of the computer when it is operating or recharging the battery.

Before moving an active computer, set the computer into any of the systems suspend modes (Standby, 5 Volt Suspend, or 0 Volt Suspend) and close the display (refer to the Suspend Mode in the next chapter).

Do not try to force the display beyond its fully opened position - about 180 degrees.

# Power Saving Modes

Your computer offers several options for you to extend your battery life while you are not actively using your computer.  These options turn off different sections of the computer that are not currently being used.

These options are listed in the POWER MANAGEMENT page of the System Setup screen (accessed during boot up).  They are described below:

## Power Management

This option sets when you want the selected time outs to take effect.

ALWAYS ON: Enables the time out settings regardless of whether the computer is on AC or battery power.

BATTERY ONLY: Only enables the time outs when the computer is running off of the battery power.

DISABLED: Disables all power time out settings.

## Power Management Mode

This option selects the Power Management Mode.  Choosing modes changes the system power management settings.  Maximum Battery Life conserves the greatest amount of system power, while Maximum Performance conserves power, but allows greatest system performance.

## Standby Time-out

The amount of time the computer needs to be idle before the computer enters Standby mode.

## Auto Suspend Time-out

The amount of time the computer needs to be in Standby Mode before the computer enters the Suspend mode.

## Suspend Mode

This option gives you the choice of selecting the Suspend mode you want to use.  The choices are; 5 Volt Suspend (Suspend to RAM), or 0 Volt Suspend (Suspend to Disk).

## Hard Disk Timeout

The amount of time the computers hard disk needs to be idle before the hard disk will spin down the motor.

- Note: If the “Auto Insert Notification” option is enabled in the CD-ROM setting, the computer will not time out on any power management time-outs because Windows 95 will constantly be actively looking for a new CD to be inserted. This option is set to a default of “disabled” by the factory.  The computer will not enter any of the power management time out modes if the hard disk drive, CD-ROM, or floppy disk drive is active.

## Display Time-out

The amount of time the computer video display needs to be idle before the video display is powered down.

## Battery Low Suspend

This option allows the user to enable or disable the transfer to Suspend Mode if the battery enters a low power condition.

## Resume On Modem Ring

This option allows the user to enable or disable the system to resume if there is a modem ring.

## Resume On Alarm

This option allows the user to enable or disable the system to resumes after an alarm (RTC) is sent to the computer.

## Alarm Time

This option sets the time that the computer resumes by the RTC alarm.

# Suspend Mode

Suspend mode is the lowest power consumption level that your computer can sustain.  There are two different suspend modes available for your computer.  One is the 5 Volt Suspend and the other is 0 Volt Suspend.  As a default your system is set to 5 Volt Suspend at the factory.  You can change the program for the computer by accessing the Suspend Mode utility in the Setup Program.

## 5 Volt Suspend (Suspend-to-RAM)

When the computer enters 5 Volt Suspend, an audible beep will sound, and the present state of the computer is stored in RAM.  All but a few essential components of the computer will then power down.  Pressing power button will allow the computer to return to exactly the same position it was in before suspend mode was activated.  You can use 5 Volt Suspend when you want to take a break from your computing, but plan to resume after a short interval.  Operation will resume almost immediately after  pressing the power button.

## 0 Volt Suspend (Suspend-to-Disk)

When the computer enters 0 Volt Suspend, an audible beep will sound, and the system will save all running applications to the hard disk drive.  The computer will then automatically turn off.  When you next turn on the computer, it will read the suspend-to-file section and return all data to memory, so that your computer is returned to the exact state it was in before the suspend mode.

0 Volt Suspend is a very useful feature as people frequently open many applications and then have them iconized on the Windows screen.  It will take some time for all of these applications to be opened and begin running.  Normally they will all have to be closed before the system can be turned off.  If you use the 0 Volt Suspend feature, you won’t need to close applications because the system status will all be saved to disk.  When you next turn on the computer, your Windows screen, with all applications open, will be recreated in just a few minutes.

## Entering Suspend Mode

Suspend mode can be activated in any of the following ways:

- By pressing Fn + F2 - Standby Mode, Fn + F3 - Suspend Mode, or Fn + F4 - Suspend to Disk.

- By setting either the Standby Suspend, the 5V Suspend, or the 0V Suspend option in the Setup Program to automatically suspend after a certain period of computer inactivity.

- By closing the cover.

- By low battery voltage.

- Note: Your computer will not enter the Suspend To Disk mode if the Suspend-to-disk partition file is missing or if it is the incorrect size.  Refer to Chapter 2, Adding Memory section for further information.

## Resuming from Suspend Mode

Normally, resuming from a suspend mode is triggered as follows:

- By turning on the power switch.

- If 5 Volt Suspend is active, by pressing the power button.

- If 0 Volt Suspend is Active, by pressing the power-on button.

- If the suspend state was caused by the battery capacity dropping below a certain level, operation can be resumed only after AC power is connected or a charged battery installed, and then only by the power switch.

When a resume event occurs, the system returns to where it was when suspend mode was entered, e.g., Software applications open, the display reappears, etc.

# Securing the Computer

Your computer comes with a Kensington® Lock notch.  To secure your computer, follow the steps listed below:

Wrap the cable of a portable computer Kensington security lock around a table, desk, drawer handle, or any immovable object.

Locate the Kensington lock icon at the right rear of the computer and insert the lock into the notch.

Turn the key to secure the lock.

Remove the key from the lock.

Kensington Lock Tab

Figure 1 - 4.   Kensington Lock Tab location.