Getting Started

2.

Running the BIOS Setup Program					O

Using the Brightness and Contrast Controls		O

Using the Glidepad									O

Using the Keyboard									O

Using Disk Drives									O

Using the Internal Numeric Keypad				O

Using the Connectors and Ports					O

Adding Memory										O

Using PC Cards										O

Turning the Speakers On and Off					O

Using a Microphone									O

# Running the BIOS Setup Program

Your computer is likely to have been properly setup and configured by your dealer prior to delivery.  However, you may find it necessary to use the computer’s BIOS (Basic Input-Output System) Setup program to change system configuration information, such as the current date and time, or your hard disk drive type.  The Setup program is stored in the Read-Only Memory (ROM), and can be accessed when you boot up the system.  The Setup program is accessed by pressing the F2 function key when powering on the Computer.

The settings that you specify within the Setup program are recorded in a special area of memory called the CMOS RAM.  This memory is backed up by a battery so that it will not be erased when you turn off or reset the system.  Whenever you turn on the computer, the system will read the settings stored in the CMOS RAM and compares them to the equipment check conducted during the Power On Self Test (POST).  If an error occurs, an error message will be displayed on the screen, and you will then be prompted to run the Setup Program.

The Setup program allows you easy access to a variety of configurable options.  At this point, however, you need only be concerned with the standard configuration (“Main”) options on page 1 of the Setup program.  Once you have the computer up and running, you will want to refer to the next chapter for a discussion of the Setup program’s power management and other configuration options.

To enter the Setup program, press the F2 function key when the following prompt appears:

Press <F2> to enter SETUP

This will display the first page of the Setup menu,  which is illustrated below.

| Phoenix NoteBIOS 4.0 Setup - Copyright 1985-95 Phoenix Technologies Ltd. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Main |  |  |  | Advanced |  |  |  |  |  |  |  | Security |  |  |  |  | Power Savings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Exit |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Item Specific Help |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | System Time: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [13 : 00 : 00] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | System Date: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [04/10/1995] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Set System Time. Use |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | <Tab>, <Shift-Tab>, or |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Diskette A: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.44MB, 3 1/2"] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | <Enter> to select field. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Change values by |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | Internal HDD |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [C:1.0 GB] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | typing in the numbers |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | Internal CD-ROM |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (None) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | directly or using +/-. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 4 | Boot Sequence |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Speaker: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [Enabled] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Boot Display Device: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | [Auto] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | TV Mode |  |  |  |  |  |  |  |  |  |  |  | NTSC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | System Memory: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 640 KB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Extended Memory: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 15360 KB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CPU Type: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Pentium |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CPU Speed: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 133 MHz |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Serial Number |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | BIOS Version |  |  |  |  |  |  |  |  |  | 03E-0208-3001 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| F1 | Help | ¯ | Select |  |  | Item |  |  |  | + / - |  |  |  |  |  | Change Values |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F9 |  |  |  |  |  |  |  | Setup Defaults |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Esc | Exit | ¬ ® | Select Menu |  |  |  |  |  |  |  |  | Enter |  |  |  |  |  | Select > Sub Menu |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | F10 |  |  |  |  |  |  |  | Previous Values |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

- Note: If you do not press these keys at the correct time and the system attempts to boot, an error message may be displayed ( such as “Missing operating system”).  There is no cause for alarm.  Simply, press CTRL+ALT+DEL to reset the system, and then, at the appropriate time, press F2 to enter BIOS Setup.

- Item Specific Help

- The right side of the screen.  This area describes each parameter and its available settings.

- Menu Bar

- The top line of the screen.  Each of the five selections displays its own screen.

- Parameters

- The left side of the screen.  This area lists the parameters and their current settings.

- Key Status Bar

- The bottom part of the screen.  These lines display the keys available to move the cursor, select a particular function, and so forth.

## Changing Bootup Settings

The Main settings page of the Setup program will allow you to change the following information:

### System Time

This option allows you to change the system time using the format hour:minute:second.  You can change the system date here and/or from your operating system command prompt.

### System Date

This option allows you to change the system date using the format month, day, year.  You can change the system date here and/or from your operating system’s command prompt

### Diskette A

This option allows you to specify the type of floppy disk drive mounted inside your computer.  The selections are: [1.44 MB, 3 ½”], [Not Installed].  The computer comes equipped with a 3.5”, 1.44 MB floppy disk drive module.

### Internal HDD

This option will automatically detect the hard disk drive type for drives that comply with ANSI specifications.

### Internal CD-ROM

This option allows you to configure the internal CD-ROM.

### Boot Sequence

This option allows you to determine in what order the system searches the drives for the proper boot sequence.  The selections are: [Diskette Drive], [Hard Drive], or [CD-ROM Drive].

### Speaker

This option allows you to “Enabled” or “Disabled” the internal speakers.

### Boot Display Device

This option allows you to select the display device the systems boots to if both a CRT and a LCD are available.  If only the CRT is available, the system always boots to the CRT.  If only the LCD is available, the system always boots to the LCD.  The selections are: [Both], [CRT], [LCD], or [TV].  The default is [Both].

### TV Mode (For Port Replicator Only)

This option allows you to select the TV mode.  The selections are: [NTSC] or [PAL].  The default is [NTSC].

### System Memory

This field reports the amount of base (or conventional) memory found by the BIOS during its POST.  The value will not exceed 640 KB.

### Extended Memory

This field reports the amount of extended memory found by the BIOS during its POST.  The value displayed is the amount of memory located (above 1 MB) in the microprocessor’s memory address map

### CPU Type

This area displays the information about the type of CPU used in the computer.

### CPU Speed

This area displays the CPU speed in MHz.

### Serial Number

This area displays the software serial number.

### BIOS Version

This area displays the BIOS version installed in the computer.

## Internal Hard Disk Drive (None)

### Autotype Fixed Disk

Attempts to automatically detect the hard disk drive type for drives that comply with ANSI specifications.  Press Enter to continue on to the next field.

### Type

This field selects the drive type corresponding to the fixed hard disk drive installed in your system.  The selections for this option are: [User] / [Auto] / [None].  If type [User] is selected, Cylinder, Heads, Sectors/Track, and Write precomp fields will need to be edited directly.  A Write precomp of None can be specified by pressing N.

## Internal CD-ROM

### Autotype Fixed Disk

Attempts to automatically detect the CD-ROM drive types for drives that comply with ANSI specifications.  Press Enter to continue on to the next field.

### Type

Select “AUTO” type to determine CD-ROM type automatically.  Select “USER” type is the system can’t recognize the CD-ROM.  Select “NONE” to disable CD-ROM.  If “USER” type is selected, Cylinders, Heads, Sectors/track, and Write precomp fields can be edited directly.  A Write precomp of None can be specified by pressing N.

## Main Menu / Boot Sequence

This option allows you to determine in what order the system searches the drives for the proper boot sequence.  Place the following selections in the order that you want the computer to search the computer for a boot drive.  The selections are: [Diskett Drive], [Hard Drive], or [CD-ROM Drive].

# Advanced  Menu

## Serial Port

Selecting COM 1 will automatically set the serial port address as 3F8h and IRQ4; selecting COM 2 will automatically set the address as 2F8h and IRQ3; COM3 will automatically set the address as 3E8h and IRQ4; COM4 will automatically set the address as 2E8h and IRQ3.  The default is [COM1].

## IR Port

Selecting COM 1 will automatically set the serial port address as 3F8h and IRQ4; selecting COM 2 will automatically set the address as 2F8h and IRQ3; COM3 will automatically set the address as 3E8h and IRQ4; COM4 will automatically set the address as 2E8h and IRQ3.  The default is [COM2].

## Parallel Port

Selecting LPT1 will automatically set the parallel port address as ??: selecting LPT2 will automatically set the address as ??.  The default is [LPT1].

### Operation Mode

This option set the parallel port mode.  The selections are: [Bi-DIR], [Standard], or [ECP].  The default is [Bi-DIR].

## On Board Audio

This option “Enables” or “Disables” the ESS 1888 sound chip.

## Plug and Play O/S

Select “YES” if you are using a Plug and Play capable operating system.

## Internal Pointing Device

This option “Enables” or “Disables” the internal pointing device everytime the system is rebooted.

# Security Menu

## Supervisor Password is

Press <Enter> to set a password.  Type the desired password, press Enter and type it again.  The system will now require the password to enter the Setup Utility.  To remove the password when user password is enabled, press Enter twice without typing any password in the blanks.  After setting a password the “User Password” will become enabled.  The default is [Disabled].

## User Password is

This selection allows you to choose whether the system will require a password to be entered on boot.  You will choose “Enabled” if you want a password, and “Disabled” if you  do not want a password.  The default is [Disabled].

## Set Supervisor Password

Press “Enter” to setup a System Supervisor Password.  You may enter up to seven (7) standard text characters for your password.  If you are the opnly user, the Supervisor password also serves as the User password.  To clear a password, simply enter a blank password.  I.e. press only the <ENTER> key and no other characters when prompted for the new password.

## Set User Password

This selection allows you to specify the user password.

## Password on Boot

This option “Enables” or “Disables” password entry on boot. If the User or Supervisor Password is set up.

## Password Check During  Resume

This selection allows you to choose if the system will require a password  to be entered upon resuming from a suspend mode.  You will choose “Enabled” if you want a password, and “Disabled” if you do not want a password.  The CapsLock and ScrollLock LED’s will flash in sequence for 30 seconds to indicate that a password is required to resume.  The LED’s will start flashing again, as soon as any key is hit, until the correct password is entered.  The default is “Disabled”.

## Diskette Access

If set to Supervisor, a user without the Supervisor Password will not have read or write access to the floppy disk drive.  If set to User, the user and the supervisor will both have access right.

## Fixed Disk Boot Sector

If set to Write protect, the hard disk boot sector will be protected from viruses that effect the hard disk drive boot sector (the most damaging type).  This also prevents accidental formatting of the hard disk drive.  If set to normal, the entire hard disk is accessible.

# Power Saving Menu

## Power Management Mode

This selection allows you to select the Power Management Mode.

Choose “Disabled” to turn off Power Management.

Choose “Battery Only” to turn off Power Management only while connected to AC power, but will turn on Power Management while on battery power.

Choose “Always” to turn on Power Management all the time.

The default is “Battery Only”.

### Standby Timeout

This selection allows you to choose the length of  time the system will be idle before entering the Standby Mode.  Standby Mode turns off various devices in the system, including the screen, until you start using the computer again. The default is [2 min].

### 5 Volt. Suspend Timeout

This selection allows you to choose the length of time the system will be idle before entering the 5 Volt Suspend Mode.  The default is [2 min].

### 0 Volt. Suspend Mode

This selection allows you to choose the length of time the system will be idle before entering the 0 Volt Suspend Timeout Mode.  The default is [2 min].

### Hard Disk Timeout

The length of time the hard disk is inactive before the hard disk is put into the Power Savings Mode.  The default is [2 min].

### Video Timeout

The length of time the user input devices need to be inactive before the screen is put into the Power Savings Mode.  The default is [2 min].

### Auto Save to Disk

This selection allows you to choose the length of time the system will be in the 5 Volt Suspend Mode before the system will save its state to disk and then power off.  The default is [30 minutes].

### Battery Low Suspend

This selection “Enables” or “Disables” the system from entering 0 Volt Suspend Mode when the battery is critical low and AC is not attached within 1 minute.

### Resume on Modem Ring

Turning this feature “On” will wake up the system from a 5 Volt Suspend mode when an incoming call is received from a modem or fax.  The default is “Disabled”.

### Resume on Alarm

Turning this feature on will wake up the system from a 5 Volt Suspend at a specified time.

#### Alarm time

Specify the eime when the system is to wake up.  Enter the time values by directly entering the numbers or by using the + / - key to scroll through the options.  Time is set in 24 hour format.

#### Date

specify the date when the system is to wake up.  Enter the time values by directly entering the numbers or by using the + / - key to scroll through the option.  If set to 00/00/0000, the alarm will wake up the system every day at the same time.  Press F9 to setup default date to 00/00/0000.

### Hotkey Beep

Selecting “Enabled” will issue one short beep when the following HotKey’s are pressed: Fn+F2, Fn+F3, and Fn+T.  The function of this beep is to notify the user the HotKey was recognized by the system.

### Battery Low Warning Beep

“Enable” or “Disable” warning beep when system is in a battery low condition.

# Exit Menu

## Discard Changes & Exit

Pressing this will cause the program to Exit without saving changed SETUP item values.

- Warning - Any changes in password will still take effect whether you Save Changes or Do Not Save Changes..

## Save Changes & Exit

Pressing this will cause the program to Exit after writing all changed SETUP item values to CMOS.  Only Power Management and Security Setting changes will take effect immediately, all other changes will take effect the next time the system is booted.

## Get Default Values

Pressing this will Load the Default values for all SETUP items.

## Load Previous Values

Reads previous values from CMOS for all SETUP items.

## Save Changes

Writes all SETUP values to CMOS.

# Using the Brightness and Contrast Controls

You can adjust the screen brightness and contrast levels using your computer hot keys.

- Note: The hot keys for brightness and contrast are continuous keys.  They will adjust as long as you hold them down.

## Brightness Control

Press Fn +  and Fn + ¯ to increase and/or decrease the brightness of the display respectively.  The brighter the screen setting, the more power is used during battery operation.

## Contrast Control

Press Fn + ® and Fn + ¬ to increase and/or decrease the contrast of the display respectively.  The higher the contrast setting, the more power is used during battery operation.

- Note: You can adjust the contrast only on DSTN LCD’s.

# Using the Glidepad

## Moving the Cursor

The embedded Glide Pad offers a unique and efficient way of pointing and selecting in a Windows environment.  The following figure shows the Glide Pad.

Glide Pad

Media Bay

Slot

Battery Slot

Figure 2 - 1.   Glidepad Location.

The Glide Pad responds to finger movements on its surface.  To move the cursor, move your finger along the Glide Pad surface.

Once the cursor is in the proper place, tap once on the surface of the Glide Pad or use the left button to click just the same as you would for a mouse.  Tap twice to double-click.

# Using the Disk Drives

Unlike a “floppy” diskette, a hard disk drive is rigid and completely sealed in a protective, dust-free environment.  A hard disk drive works very much the same as a diskette, but can retrieve and record data much faster and has a much larger storage capacity.

The computer is equipped with a 2.5” IDE (Integrated Drive Electronics) drive.  This type of drive embodies the latest in fast, reliable mass storage by integrating all of the control circuitry necessary for its operation directly onto the drive itself. This in turn allows the hard drive manufacturer to carefully optimize drive performance.  Once you have installed an operating system onto your hard drive, you will want to install all of your software applications onto it as well, and perform your work from the hard disk, using diskettes only for backup and archival purposes.

# Caring  for the Hard Disk Drives

The hard disk drive included with the computer is designed for portability, but it is not indestructible. Moreover, while modern hard disk drives are exceptionally reliable, they do fail occasionally.  To avoid damaging your disk drive, and to protect your valuable data, you should take the following precautions:

Never turn off or reset the computer while the HDD status icon is lit.

Make regular backups of your hard disk drive to diskettes, and keep a backup copy on hand.

Keep the AC adapter at least six (6) inches away from your hard disk drive(s).

# Using the CD-ROM Drive

The computer comes with a 10X (or higher) speed CD-ROM drive that is compatible with existing music and data CD’s.

This is a Class 1 Laser product.

## Inserting a CD

Press the eject button on the CD-ROM drive to eject the disk tray.

Place the CD disk onto the tray with the label side up.

Push the tray in by hand to return the tray to the drive.

## Handling CDs

Follow these guidelines to avoid damaging your CDs and to prolong the life of your CD-ROM player:

- When removing a disk from it protective cover or when loading a disk into a drive, hold the disk by its central core hole and an outer edge.  Never touch the disk’s data surface (non-labeled side).

- Never write on a disk or place a label on the disk surface.

- To protect the disk against scratches and dirt when not in use, always keep the disk in its protective case or cover.

- When cleaning a disk, always wipe in the direction of the center to the outer edge.  Don’t wipe the CD in a clockwise or counterclockwise direction.

- To remove dust or fingerprints, use a clean, soft and dry cloth.  Never use benzene or anti-static fluids.

- Keep disk away from high temperatures or direct sunlight.

## CD-ROM Drive Guidelines

Failure to observe the following precautions can damage both the CD-ROM drive and the data on the CD:

- Do not open the disk tray except when inserting or removing a disk.

- Never push down on an open disk tray.

# Using the Keyboard

The computer is equipped with either an 87- or 88-key keyboard, which provides all the functionality of a full-sized desktop 101-key keyboard.  Even if you are an experienced PC user, you should take a moment to become familiar with the layout of your computer’s keyboard.  In particular, you should familiarize yourself with the special computer function keystrokes which allow you to quickly and easily control and access the power management hot keys.

The computer has many special keys, most of which depend on an application for their functionality.

Figure 2 - 2	Keyboard Layout

## Special Keys

The following keys have special functions at the command level of MS-DOS and within many different programs:

Pause

Stops a command or application; primarily used to stop the screen from scrolling; pressing any other key resumes the execution of the command or application.

Shift+PrtSc

Sends the contents of the screen to the printer port; prints only text characters unless you have run the GRAPHICS.COM utility to enable printing graphics.

Ctrl+Break

Terminates the current command or application.

Ctrl+P

Sets the computer to echo keystrokes to the printer; prints a line when you press Enter; continues until you press Ctrl+P again.

Ctrl+Alt+Del

Terminates all programs, reloads MS-DOS and executes the AUTOEXEC file; also called “warm start” or “warm boot’.

The following keys are the system hot keys for your computer:

Fn + F2

Suspend to Disk

Fn + F3

Suspend to RAM

Fn + F5

Not Defined

Fn + F6

Not Defined

Fn + F7

Not Defined

Fn + F8

Not Defined

Fn + F9

Not Defined

Fn+F10

Display Expand/Shrink

Fn + F11

LCD Backlight On/Off

Fn + F12

Toggles the screen output between the LCD, external CRT, or to LCD and CRT at the same time (called SIMULSCAN).

Fn+®

Adjusts the contrast brigher.  (On DSTN LCD panels only.)

Fn+¬

Adjusts the contrast darker. (On DSTN LCD panels only.)

Fn+

Adjusts the brightness up.

Fn+¯

Adjusts the brightness down.

Fn + End

Speaker On/Off

Fn + PgUp

Volume Up

Fn + Pg Dn

Volume Down

## Using the Internal Numeric Keypad

The keyboard has an embedded keypad that provides the same functions as the discrete numeric keypad on an AT® enhanced keyboard.

The embedded numeric keypad keys generate AT-keypad characters and functions when pressed in conjunction with Num Lock, Fn and Shift.

The embedded numeric keypad has two modes you can enter by toggling the Num Lock hot key (Num Lock) and is signaled by the Num Lock indicator: ON or OFF.

Num Lock On

When the Num Lock indicator is on, pressing a key generates the numeric characters shown.

Pressing Shift with a key generates the characters shown in the following figure.

Figure 2 - 3	The Keyboard Numerical Number Pad

If you press and hold Fn in the Num Lock mode, the keypad generates their normal characters.

Num Lock Off

When the Num Lock indicator is OFF, the keyboard acts as normal.

Pressing Fn with a key generates the same characters mentioned in the previous paragraph, Num Lock On (with Shift).

# Using the Connectors and Ports

This section provides a description of the connectors and ports on the rear and left side panels of the computer.

## The Rear Panel

PS/2 Port

Printer Port

Serial Port

VGA Port

Expansion Port

USB Port

Figure 2 - 4	Rear panel of the Notebook Computer

Serial

Connects to external devices such as a serial printer.  16550 UART compatible.

Expansion

Connects to a port replicator or Docking Station.

Parallel

Connects to a parallel printer or other device that uses a standard parallel interface; EPP/ECP compatible; also connects to the floppy disk drive when used externally.

External Monitor

Connects to an external analog monitor.  (15-pin)

PS/2

Connects to an external PS/2 keyboard, numeric keypad, or mouse.

If you are connecting a keyboard with a 5-pin DIN connector, you will need to purchase a 6-pin mini-DIN adapter.  It also can support an optional “Y” connector.

## The Left Side Panel

Fax/Modem JAck

FIR Window

Microphone Jack

Line - In Jack

Line - Out Jack

Headphone Jack

PCMCIA Card Slots

Figure 2 - 4.	Left Side Panel of the Notebook Computer

FIR

Connects to any IrDA compliant device (such as another IrDA computer or printer) without the use of a cord or cable.  Transmits up to 4 M bits per second.

Microphone

Connects to an external microphone.

Line Out

Connects to a line out device such as headphones or amplified speakers.

Line In

Connects to a line-in device such as a synthesizer, stereo Walkman or

audio CD player.

Headphone

Connects to an external set of headphones for private listening.

PC Card

The PC Card slots support one Type III or two Type I/II PC Cards.

Fax/Modem

There is space for an optional Fax/Modem Port toward the rear on the left side panel.

## The Right Side Panel

DC-In Slot

Reset Switch Slot

Kensington Lock Tab

Figure 2 - 5	Right Side Panel of the Notebook Computer

DC In

Connects the AC adapter output connector to this jack to recharge the battery and to supply power to the computer.

- Caution: Use only the supplied AC adapter with your compute.  Other adapters could cause serious damage to the electronic circuits.

# Adding Memory

Your computer has 16 MB of EDO Dynamic Random Access Memory (DRAM) for computer system memory.  The computer also allows you to install and expand the system memory via a single onboard SO-DIMM (Small Outline Double In-line Memory Module, 144 Pins) socket.  The SO-DIMM socket is easily accessible and can accept a 16 MB, 32 MB  or 64 MB SO-DIMM module, which are available from your dealer.

Whenever you change your computer memory size, you should run the PHDISK utility to re-size your Suspend to Disk file.  This file is used to store all your computer information onto the hard disk drive for the 0V Suspend function.  If you do not re-size the Suspend to Disk file, you will not be able to use the Suspend to Disk utility.

To add memory, locate the memory module unit on the (?) side of your computer.

FOLLOW THE INSTRUCTIONS THAT CAME WITH YOUR MEMORY UPGRADE OPTION TO INSTALL THE ADDITIONAL MEMORY.

# Using PC Cards

The computer is equipped with two PCMCIA sockets which can accommodate Type I/II or Type III cards conforming to the standards of the PCMCIA (Personal Computer Memory Card International Association).  These include the following types of devices:

## Flash Memory

A solid-state storage device that works like a floppy drive, with a write-protect switch on the edge of the card.  Unlike conventional memory, a flash memory card does not require an external power source to maintain data when removed from the socket.

## SRAM Memory

A static RAM memory device that can be formatted as a diskette or an IDE hard disk drive, with a write-protect switch on the edge of the card.  An on-card battery maintains the data when it is removed from the computer’s PCMCIA socket.

## Network

A device that connects the computer to a local area network (LAN) such as an Ethernet.

## Fax/Modem

A device that connects the computer to the telephone line for use as a fax and/or a modem.

## ATA Device

An AT Attachment, rotating or solid-state mass-storage device that works like an IDE hard disk drive.  An ATA device does not require an external power source to maintain data when removed from the computer’s PCMCIA socket.

## Inserting and Ejecting PC Cards

PC cards are inserted and ejected in much the same way as diskettes.

- Open the PCMCIA compartment cover on the (?) side of the computer.  (It is unnecessary to power off the system when handling PC cards and devices.)

- To insert a PC card, align the card with the appropriate socket and slide the card into the socket until it locks into place.  Note, that some PC Cards must be prepared by a PCMCIA driver, before you can use them for data storage, see your PC card manual for details.

- To eject a PC card, first ensure that the computer is not accessing the memory card or device, and then press the appropriate eject button on the socket.  When the card pops out of the socket, remove the card and store it properly.

## Configuring and Using PC Cards

To use a PC card with the computer, your operating system must be configured to recognize the device.  PCMCIA utility software is provided with the computer, containing the latest available drivers and a file containing instructions on how to use them.  The PCMCIA utilities provide the following features:

- an installation program for simplified installation and configuration of PCMCIA drivers and utilities.

- support for the Microsoft Flash File System II (FFSHII).

- a PC Format utility which can format a SRAM PC card as floppy disk drive D: or E:.

- an easy-to-use information utility for managing PC cards formatted with PC Format.

- support for hot insertion and removal of PC cards under DOS without the need to restart the computer.

- support for Power Management.

- support for booting from a PC card designated as a hard disk drive or floppy disk drive.

The following instructions should improve the relationship between the computer hardware and the supplied PCMCIA utility software under DOS.

### Installation

To install computer PCMCIA support, insert the Phoenix PCMCIA driver utility and type A:Install.exe at the operating system command prompt.  Select the PCMCIA installation option from the setup menu and follow the instructions on the screen.

### Configuration

To configure or reconfigure PCMCIA software support after installation, change directions to the directory where you installed the PCMCIA software and type PCMSETUP at the prompt.  Follow the instructions on the screen.

### Documentation

To install the driver, insert the Phoenix PCMCIA driver utility and type A:Install.exe at the operating system command prompt.  Select the appropriate computer documentation option from the Setup menu and follow the instructions on the screen.  To view PC card documentation, copy the USERMAN.ZIP file from drive A to drive C, decompress the file, then view the documentation in MS Word 6.0.

### Windows Installation

To install the PCMCIA driver for Windows 95, follow the steps listed below:

Insert the PCMCIA driver utility diskette into drive A.

- Double click Control Panel / Add New Hardware / PCMCIA socket.

- Click “Have Disk” to change the driver.

- Follow the instructions on the screen to complete the installation process.

# Using the Audio System

The audio capabilities of the computer bring high quality stereo sound to your software applications, including features like an FM synthesizer, and digital recording and playback.  The computer’s integrated audio system is fully compatible with the Windows Sound System, and includes a number of applications on diskette which allow you to record, compress, store, and playback voice, sound, and music under the Windows environment.  The audio system is also Sound Blaster compatible, so you can hear all the digital sound effects and voice recordings used by the latest entertainment software.

The computer’s audio capabilities include the following features:

- Digitized audio playback capability (up to 44KHz), which faithfully plays back and reproduces all kinds of digitized sounds with a 16-bit Digital-to-Analog Converter(DAC).  The audio system works with a number of applications that utilize the digital voice channel to deliver realistic human speech and sound effects.

- Digitized audio recording capability (up to 44KHz) allowing digitization and recording of any kind of sound through the computer’s built-in microphone or an external source.

- High quality sound, dynamically filtered for low noise digital recording and playback.  Input from the computer’s built-in microphone uses Automatic Gain Control (AGC) to compress sound input to adapt dynamically to different recording conditions.

- Audio compression which offers real-time compression of digital audio at rates of 2:1, and real-time decompression at rates of 8:1.  Compression turns large sound files into smaller files that can easily be stored and transferred to a diskette or even sent over a network.

- Built-in amplifier and speaker, including volume control, and a sound-out port for connection to an external pair of headphones with power amplified speakers.

## Adjusting Volume

Computer sound volume can be adjusted using the speaker hot keys (Fn + <Pgup>) or (Fn + <Pgdn>).

## Audio Software

The audio system supports a large library of third party software packages include music/sound drivers.  All you have to do is select the correct music/sound driver when you install the package, or follow the package’s instructions for selecting the driver.  You may select the Windows Sound System, or Sound Blaster Pro.

The computer also includes software which includes a special driver and accessories that are designed especially for the Microsoft Windows environment.  These audio tools require Microsoft Windows 3.1 or a later version.

To install the audio driver for Windows 3.1, follow the steps below:

- Insert the ESS1878 audio driver utility diskette into drive A.

- Run Setup.exe on the diskette from the Windows file manager.

- Follow the instructions on the screen to complete the installation process.

When using the optional audio system with the computer’s save-to-file feature enabled, it may occasionally be necessary to restart Microsoft Windows after a resume event in order to reinitialize the audio device drivers.

To install the audio driver for Windows 95, follow the steps below:

- Insert the ESS1878 audio driver utility diskette into drive A.

- Double click “Control Panel / System / Device Manager”.

- Remove the “?” device.

- Double click  “Control Panel / Add New Hardware / Sound”.

- Follow the instructions on the screen to complete the installation process.

### Using the Video System

To install the NM2093 VGA driver for Windows 3.1, follow the steps below:

- Insert the NM2093 VGA driver utility diskette into drive A.

- Run Setup.exe on the diskette from the Windows file manager.

- Follow the instructions on the screen to complete the installation process.

To install the VGA driver for Windows 95, follow the steps below:

- Insert the NM2093 VGA driver utility diskette into drive A.

- Double click “Control Panel / Add New Hardware / Display Adapters”.

- Click “Have Disk” and select “NeoMagic MagicGraph 128ZV” to change the driver.

- Follow instructions on the screen to complete the installation process.

# Using the Microphone

Your computer has a built-in microphone located in the middle left corner of the Palm Rest cover.

To use the microphone, face your computer from a normal distance and speak in a normal voice.  You do not need to bend down to speak directly into the microphone.