Using Software

5.

Using Supplied Software				O

Guidelines for Installing Applications		O

Using System Passwords				O

Getting Help						O

# Using Supplied Software

Your computer is shipped with the following software:

- Power-saving utilities

- VGA Drivers

- Audio Drivers

- Glide Pad Drivers

- PCMCIA Drivers

- Modem Utilities

- USB Drivers

# Guidelines for Installing Applications

If you want to customize the Save-to-Disk partition size or reinstall MS-DOS, please refer to the following section.

### Save-to-File

If this feature is enabled, when the computer enters Suspend mode the contents of memory is saved to your hard disk drive for retrieval when a resume event occurs.  The advantage of this state is that the battery power is further conserved because current can also be cut to memory when the computer enters the Suspend mode.  However, if Save-to-File is enabled, the resume process takes somewhat longer because a hard disk drive takes longer to power up to an operating condition.

### Save-to-RAM

If the computer has not been configured for Save-to-File, when the system enters Suspend mode the computer becomes inactive, but memory continues to receive current so that stored information can be retrieved instantly whenever a resume event occurs.  If Suspend-to-RAM satisfies your portable computing requirements, you can skip the next section and proceed to the section on Advanced Power Management.

# Installing an operating System

If you wish to enable Save-to-File Suspend mode on a new hard disk drive, you must first prepare the hard disk drive for this feature before installing an operating system.  You should also be aware that enabling Save-to-File mode subtracts from the formatted capacity of your hard disk drive and may require a full backup and restore of your hard disk drive’s data in the event of a computer memory upgrade.

To enable the Save-to-File feature on a new hard disk drive, follow these steps:

- If you have not already done so, insert a system diskette (a diskette with an operating system installed on it) into the floppy disk drive.

- Power on your computer or reset the system by pressing the CTRL+ALT+DEL keys simultaneously.  The computer will automatically load the operating system from the diskette.

- After the operating system loads into memory, replace your system disk with the supplied utility disk, type A:\PHDISK\PHDISK /CREATE /FILE at the operating system command prompt.

# Advanced Power Management (APM)

In addition to the power saving features designed into the computer’s hardware and ROM BIOS firmware, your computer is also compliant with the Advanced Power Management (APM) specification.  Through an APM device driver supplied with your operating system, the operating system is able to notify the computer’s ROM BIOS when system resources, such as the keyboard, I/O ports, display panel, and microprocessor, are not in use.  The computer’s APM-aware ROM BIOS is then able to selectively power down system components that are not in use.

Windows provides you with APM which gives you effective power management while using the Windows environment.  From the Windows start button, point to Settings, then Control Panel.  In this window you will see an icon of a battery and an AC plug labeled Power.  Double-click this icon to display the Power Properties window.  From this window you can select advanced or standard power management, or disable the power management.  The window also displays a battery meter. You can also select whether the battery meter icon is displayed beside the clock on the taskbar.

# Installing Computer Drivers and Utilities

After copying your application software to the hard disk drive, you may want to install the enhanced drivers and utilities software included with your computer.  The following driver and utility diskettes are provided:

Disk 1 — 	Contains VGA drivers for Windows 3.1

Disk 2 —	Contains Audio drivers for Windows 3.1 and Windows 95

Disk 3,4 — 	Contains PCMCIA drivers for Windows 95

Disk 5 — 	Contains Glide Pad  drivers for Windows 95

Disk 6 — 	Contains PHDISK.EXE, USB drivers, and

Modem Utility for Windows 95

Disk 7,8 —	FIR Puma drivers

Disk 9 —	CD-ROM Drivers for 10X CD-ROM

(Comes with CD-ROM Option only)

# Using System Passwords

The computer has a USER password security system.

If the User password is set, the password must be entered to gain access to the Setup Security options: USER PASSWORD, DISKETTE ACCESS, AND FIXED DISK BOOT SECTOR.

The password prevents unauthorized access to the computer at system startup or when the computer resumes from a  Suspend mode if the PASSWORD CHECK DURING RESUME parameter in Setup is enabled

## Setting a System Password

- Caution: If you forget the system password, you will not be able to use your computer.  To regain access, you will need to return the computer to your dealer.  This service is not covered by warranty.

To set a password in Setup, follow these steps:

- Press F2 during boot to enter Setup.

- Press the arrow keys to move to the Security Screen.

- Move down the screen to a Password parameter, then press Enter.

## Window pops up requesting a new password.

Enter the new password (up to seven(7) printable text characters) and press Enter, then retype the password for verification and press Enter.

- The password becomes active after you save and exit Setup.

If you set a password, you are prompted for the new password before starting your computer or entering Setup.

## Disabling or Changing a System Password

To disable or change a system password:

- Press F2 during boot to enter Setup.

- Enter your password when prompted.

- Press the arrow keys to move to the Security Screen.

- Move down the screen to a Password parameter, then press Enter.

- Note: you will only be able to change the Password if you used the password to enter Setup.

Enter a new password to set the  password, or press Enter with no entries to clear the password.  Verify your entry, then press Enter.

- Note: Save the changes then exit Setup.