Options

4.

AC Adapter					O

PC Cards						O

PS/2 Devices					O

External Monitor				O

External Mouse					O

Memory						O

Audio Options					O

Miscellaneous Options				O

Fax/Modem					O

Port Replicator					O

Battery Charger					O

Car Adapter					O

# AC Adapter

Your computer uses a small, lightweight external AC adapter to charge and power the computer.  The AC adapter can be operated anywhere between 100 - 240 volts AC and has a detachable AC power cord.

You can order power cords with specific plugs for the region where you will be operating the computer.

- Caution: Use only the AC adapter that came with your computer.  Another adapter could damage your computer.

# PC Cards

Your computer supports all PCMCIA PC Cards.  These PC cards are used to add functionality to your computer, such as communicating over a telephone or connecting to a network.

The computer has built-in slots that support one Type III or two Type I/II PC Cards.  Type III PC Cards must be inserted into the lower slot.

Your computer is designed to support additional functionality on top of the standard interface.  These functions include the Zoomed Video Port interface required by MPEG cards (on the top PC Card socket).

# PS/2 Devices

Your computer supports several external devices that connect to the PS/2 port.  Such devices include an external numeric keypad, external keyboards, and external mouse’s.

# External Monitors

Your computer can support standard analog external monitors with resolutions up to 1024 x 768 modes.  The computer also supports DDC compatible monitors allowing you to use the “Green” monitors (those monitors that meet the Energy Star standards).

When the computer is connected to an external monitor, you can use the computer with the display closed.

You can also display images on the external and internal display at the same time.  This feature is called “SimulSCAN™.  To enable this feature, set the DISPLAY parameter on Setup to “Both”.  You can also easily switch between the displays LCD, CRT, or to both the LCD and CRT together by using the Fn+F12 hot key.

# External Mouse

Your computer comes with a pointing device already installed, but you can use an external PS/2 or a serial mouse if that is your preference.

To connect a PS/2 mouse, insert the connector into the PS/2 port in the rear of the computer.

To connect a serial mouse, attach the connector to the 9-pin serial port on the rear of the computer.

- Note: A serial mouse is not a Plug and Play device.  For Windows 95 to detect a serial mouse, use the Add New Hardware icon in the Control Panel.

# Memory

Your computer comes with 16 MB of Dynamic Random Access Memory (DRAM).  Your computer also has one memory upgrade slot.  You can increase memory by installing an upgrade module into this slot.  You can expand RAM from 16 MB to

32 MB, 48 MB, or 80 MB.

Refer to Chapter 2 for information on how to upgrade the memory modules for your computer.

# Audio Options

Your computer comes with a set of built-in stereo speakers and monaural microphone, but if you wish to use other stereo equipment, the computer comes with three audio jacks — Line-out, Line-in, Headphone-out, and Microphone-in.

Line-out allows you to use amplified external speakers.  The computer’s internal speakers will be disabled when a set of external speakers are plugged into the computer.

Line-in connects to an external stereo source.  This allows you to play an external radio, stereo, or CD through the computer speakers or you can make a recording, if you have the correct software.

Headphone-out allows you to use a headset for listening to an external radio, stereoCD’s and/or other sound processing equipment that are connected to your computer.  The computer’s internal speakers will be disabled when a headset is plugged into the computer.

Microphone-in allows you to record input into the computer through an external microphone.

# Fax/Modem

The Fax/Modem allows you to send and receive telefax messages directly to your computer.  The Fax/Modem has a speed of 33.6mbs.

# Port Replicator

The notebook computer has an expansion port where you can attach an optional Port Replicator.

# External Battery Charger

There is an External Battery Charger available as an option that can be purchased from your local dealer.

# Car Adapter

There is a Car Adapter available as an option from your local dealer.

# Miscellaneous Options

Your computer can accept many additional options.  Some of the additional options include:

Batteries	You can purchase spare batteries — Compatible NiMH or

Li-Ion — from your local computer store.

For information on these batteries, refer to Chapter 3, Using Battery Power.

Carrying Case	Helps to protect the computer and accessories during transport.

Several carrying cases are available including leather 	cases, accessory cases, portfolios, and backpacks.

Printers		You can connect a parallel printer to the parallel port or a serial

printer to the serial port connectors.