Using Battery Power

3.

Removing and Installing the Battery			O

Guidelines for Battery Use					O

Switching to Battery Power				O

Recharging the Battery					O

Calibrating the Battery					O

Responding to Low Battery Conditions			O

Conserving Battery Power					O

# Removing and Installing the Battery Pack

The rechargeable battery pack allows you to operate the computer without an external power source and with the advanced power management features activated, you could use it for an extended period of time.  At a low battery power level, the computer will alert you by a blinking battery status icon and an intermittent beep sound.  Take action immediately to avoid losing data.

If you plan to make frequent and prolonged use of the battery pack while traveling, you may want to consider the purchase of an optional secondary battery pack.  You can purchase optional battery packs from your dealer.

To remove the battery pack, follow the steps below:

- Power off the computer, being sure to save your data first.

- Open the battery cover on the right rear side of the computer.

- Locate the battery pull-out tab on the battery and extend it as far as it will go.

- Using this tab, pull the battery out of the computer.

You can also warm swap the battery when using the AC adapter that came with your computer.

- Insert the AC adapter into the notebook computer and connect the AC adapter to an electrical outlet (correct voltage), leaving the computer powered on.

- Remove the battery using the battery pull-out tab, as indicated above.

- Insert a new battery into the battery compartment and continue working.

To insert or replace the battery pack, follow the steps below:

- Insert a new or recharged battery pack into the battery compartment.  Make sure 	that the contacts are facing down and to the rear of the compartment.  Check the 	label (facing down when inserted) showing the Smart indicator facing up on the 	battery.

Battery Power Level Indicator

When the battery pack is securely in place press the battery tab back into the  	battery and close the battery cover.

Battery Pull-Out Tab

Insert

Remove

Battery

When cold swapping batteries (turning the computer off), it may be necessary to restart the computer by using the hardware reset switch.

- Warning: There is danger of an explosion if the battery is incorrectly replaced.  Replace the battery only with the same or an equivalent type recommended by the manufacturer.  Discard used batteries according to the manufacturer’s instructions.

# Guidelines for Battery Use

## When You First Buy Your Notebook

Your notebook is shipped to you with the battery not installed in the unit.  Upon unpacking your notebook, you should first charge the battery for at least three hours. And then run the machine on battery power until it shuts off automatically.  If you notice that the battery only lasts a short time (less then one hour), you should repeat this process.  This process conditions the battery, removing any negative effects of long term storage, and optimizing the battery’s performance.

During this first charge and discharge process, you may notice that the Smart indicator on the battery does not read correctly.  This will change after the conditioning process.

## Normal Care and Operation

To get the best results from your Smart battery:

- Keep the battery terminals clean

- Do not heat the battery

- Store the battery at room temperature

Once a month you should fully charge your battery, and then fully discharge it until your notebook shuts of automatically.  This will ensure that you continue to get optimum performance from your Smart battery.

If you notice that the Smart indicator on the battery is not reading correctly, by following the above charge and discharge pattern the battery should return it’s operation to normal.

## Reading the Smart Battery Fuel Gauge

The Smart battery fuel gauge is designed to make your life easier.  The Smart indicator shows, in 25% steps, how much energy is left in your battery.  It can be used as an approximate indicator for the run-time of your notebook.

As you use the battery, it is possible that the PowerSmart fuel gauge will show less than full energy, even though you have fully charged the battery.  This could mean one of two things:

- You need to fully charge and then fully discharge your battery, as described above.

- You have used your battery a lot, and it is starting to wear out.  In this case, the fuel gauge is a good indicator of how much the battery is worn, and will help you decide when to purchase a new battery.

## When you are not Using your Notebook

If you are not going to use your notebook for more than a week (for example, if you go on vacation), you should remove your battery from the notebook and store it in a cool place (normal room temperature is O.K.).

## Do’s and Don’ts of Battery Care

- Don’t drop the battery or subject it to shocks.

- Don’t expose the battery to direct sunlight, moisture, chemicals, or temperature extremes.

- Don’t short the battery leads or insert the battery upside down.

- Charge the battery after several days of disuse to keep it fully charged.  If your computer is idle for an extended period of time, charge the battery every three (3) months.

- Never use the battery to power other products.

- The battery has thermal fuses to prevent unsafe computer operation.  The computer may not operate on battery power after storage in a very warm place until the thermal fuses cool.

- Keep the Duracell Smart battery properly calibrated to maintain a maximum charge by following the conditioning instructions mentioned earlier in this chapter.

THIS COMPUTER IS DESIGNED TO WORK WITH THE DURACELL COMPATIBLE SMART BATTERY PACKS ONLY.  USE ONLY AUTHORIZED LITHIUM-ION DR201 COMPATIBLE OR NICKEL METAL HYDRIDE DR36 COMPATIBLE BATTERY PACKS WITH THIS COMPUTER.

# Switching to Battery Power

To use battery power, install the battery pack as described in Installing the Battery Pack mentioned earlier in this section.

As long as the battery has a charge remaining, you can switch to battery power by removing the connection form the AC adapter, even if the computer is already on.

Your computer switches back to AC power when the AC adapter is plugged back into the computer, even if the computer is already on.

To maintain a full charge on your battery, always reconnect the computer to the AC adapter whenever possible.

# Recharging the Battery

The following procedure is acceptable under most circumstances:

- Install the battery pack into your computer (if not already installed).

- Connect the AC adapter.

To maintain a full charge, leave the computer connected to the AC adapter except when transporting the computer.

The computer charges the battery when it is on as well as when it is off, as long as the AC is plugged in.

- Caution: Never recharge the battery differently then from the procedures described in this manual.

# Calibrating the Battery

The computer uses either a Duracell compatible Nickel-Metal-Hydride (NiMH) DR36 or a compatible Lithium-Ion (Li-Ion) DR201 rechargeable battery.  Your new Duracell smart battery keeps extremely accurate track of the battery capacity.  However, all rechargeable batteries lose capacity over time due to different charging and discharging conditions, as well as general wear and usage.  In order to keep accurate track of the true capacity, the battery must be calibrated every once in a while.

- Caution: If the battery loses track of the true capacity, it will not correctly predict low battery conditions and can cause the system to shut off prematurely and without warning.

The battery itself keeps track of when it needs to be calibrated.  The computer will post a warning when you boot up the system as to when it is time to re-calibrate.  To calibrate the battery, you must first fully charge the battery, then completely discharge the battery.

Follow the instructions below to calibrate your battery:

- Charge the battery by leaving the battery in the computer and having the AC 	adapter plugged into the computer.  The battery is fully charged when the charging 	LED turns off.

- Disconnect the AC adapter from the computer.

- Turn the computer power ON, if it is not already on.

- Allow the computer to run down completely until it powers off on its own.

- Re-connect the AC power.

- Recharge the battery.

# Responding to Low Battery Conditions

There are two battery low stages in this computer.  When the battery capacity reaches 15% capacity (about 15 minutes remaining), the computer enters the first stage of battery-low warning.  At this time the battery low LED found on the LCD panel flashes and an audible warning sounds.

When the battery capacity reaches 8% capacity (about 8 minutes remaining), a second audible warning sounds.

The following actions can maximize the time before the battery is depleted and minimize the effects of losing power:

- Set the screen brightness and contrast control to the lowest possible setting.

- Save your work in progress to minimize the danger of losing data.

- If you are using a RAM disk, save the contents of the RAM disk to the hard disk drive .

- Turn off the computer if it does not need to be active.

Once your computer shuts down, you can insert a new or fully charged battery and then resume operation.  AC power can be connected or reconnected at anytime.

# Conserving Battery Power

The following tips can help you to prolong the life of a battery charge:

- Keep the display at the lowest comfortable brightness and contrast level.  Reducing brightness and contrast even a small amount can significantly reduce power consumption and increase operating time.

- Set the power management settings in Setup to optimize the time-outs.

- You can minimize the number of times the computer needs to access the hard disk drive by using disk caches or RAM Disks.

- Disconnect or turn off external options that you are not using.