Port Replicator

(Port 52)

Installation Instructions

Introduction

The “Port 52” makes it easier for you to connect all of your external devices to the Notebook Computer.  You may find it so convenient, that you will need one for the office and one for the home!

+Note: For Windows 95 users only, the first time you power on after connecting the Port 52 you will see “New Hardware Found” messages.  Your system is running through a hardware detect cycle.  Just wait for the cycle to complete before proceeding.

Read the following sections for instructions on the Port 52 installation and usage.

Port 52 Installation

Follow these steps to install the Port 52.

- Plug the Port 52 into the expansion connector on the back of your notebook computer.

- Press the two Lock Knobs on both sides to connect the Port 52 to the notebook computer.

- Press up on the Lock Latch to secure the lock mechanism.

Lock Latch

Lock Knob

Expansion Port

Lock Latch

Parallel Port

Game/MIDI Port

PS/2 Port

Ext. Keyboard

DC in Plug

VGA Port

TV Port

Lock Knob

Serial Port

Port Descriptions

- PS/2 (Mini-DIN K/B) Port

Connects to a 6-pin standard PS/2 101 keyboard.

- TV Connector

Connects to a NTSC/PAL television.  Follow the instructions in this sheet to set up your computer for TV mode.

- VGA Port

Connects an external VGA/SVGA monitor to your notebook computer.

- Game/MIDI Port

Connects a joystick or MIDI device to your notebook computer.

- Parallel Port

Connects a printer to your notebook computer.  You can change the LPT Mode in the Setup program.

- Serial Port

Connects a serial device to your notebook computer, such as an external modem or mouse.

- PS/2 (Mini-DIN Mouse) Port

Connects to a PS/2 mouse.

Using TV Output

To use your TV or VCR as an output device with your Notebook Computer and Port 52, you must first complete the following:

- connect the notebook computer to the TV

- set the notebook computer system BIOS to TV mode.

These steps are described in detail next.

Connecting Your Notebook Computer to a TV

Use these steps to connect your notebook computer to a TV or VCR.  You will need a RCA-to-RCA type cable to complete this procedure.  You can purchase one of these cables at a local video supply store.

- Power off your notebook computer.

- Connect the Port 52 to the computer via the expansion connector on the back of the system.

- Use the RCA-to-RCA type cable and connect one end to the Port 52’s TV connector.

- Connect the other end of the RCA-to-RCA cable the “Video Input” connector on the TV or VCR.

Go on to the next set of instructions to set up the notebook computer’s system BIOS to display in TV mode.

Setting the System BIOS to TV Mode

Complete these steps to set the notebook computer’s system BIOS to display in TV mode using the notebook computer’s Setup utility.  If you’d like more information on Setup, see your Notebook Computer’s User’s Guide.

- Power on the Notebook Computer.

- When you see the “System BIOS Shadowed” screen, press “F2” to enter Setup.

- Use the right arrow key and highlight “Peripherals”.

- Use the down arrow key and highlight “Display to TV”.

- Use the “-“ key to change the settings from “Disabled” to “NTSC”, or “PAL” if in Europe.

- Press “Esc”.  Press “Enter” to “Save Changes and Exit”.

- Press “Enter” to continue.  The system restarts in the TV display mode.

- When you need to return to using the notebook computer’s LCD as your output device, remember to disable the TV mode setting in Setup before disconnecting the TV and the Port 52.

Disabling TV Mode

With the TV and the Port 52 still connected, use the Setup utility to disable TV mode as follows:

- Follow steps 1 through 4 in “Setting the System BIOS to TV Mode” above.

- Use the “-“ to change the setting to “Disabled”.

- Press Esc.  Press Enter to “Save Changes and Exit”.

- Press Enter to continue.  The system automatically restarts in the Normal display mode.

After disabling TV mode in Setup, disconnect the RCA-to-RCA cable from the TV and reset the video settings for use with the notebook computer’s LCD.

- If your notebook computer is accidentally started in the TV mode without a TV/VCR connection, the LCD blanks immediately following the Power On Self Test (POST).  However, if you press the “F2” key during POST, you can access Setup and disable TV mode as described previously.