Introduction to the

1.

ALL IN ONE

NOTEBOOK COMPUTER

Introducing the Computer				O

Features of the Computer				O

Unpacking the Computer				O

Starting the Computer					O

Using the AC Adapter					O

Using the Indicator Lights				O

Environmental Specifications			O

Safety Precautions					O

Using the Computer					O

Power Saving Modes					O

Securing the Computer				O

# Introducing the Computer

This computer is a lightweight, compact, and fully IBM compatible computer, featuring the latest in portable computing technology.  Designed for a wide range of general business and personal productivity applications, this computer makes an ideal choice for use in the office, the schoolroom, at home, and/or on the road.  Standard features include a powerful Pentium microprocessor, fast PCI-bus VGA graphics, a high performance hard disk drive, a high density floppy disk drive, a powerful 10X speed CD-ROM disk drive, support for PCMCIA cards and devices, a built-in Glidepad, and advanced power management capabilities to prolong battery power.

Bottom Assembly

LCD Assembly

CD-ROM

Battery

Palm Rest and Glide Pad Assembly

Figure 1 - 1.   Your Notebook Computer

# Features of the Computer

This notebook computer includes a variety of innovative features designed to meet the most demanding computer requirements:

A powerful Intel Pentium microprocessor, with 256 KB on board cache and math coprocessor, supporting suspend and resume capabilities under several different operating systems.

A flexible and field-upgradeable memory architecture supporting up to 80 MB of onboard EDO RAM.

Two PCMCIA expansion sockets which can accommodate one Type III or two Type I/II PC cards.

A detachable and rechargeable Nickel-Metal Hydride (NiMH) smart battery pack, Lithium-Ion (Li-Ion) battery pack as an option.

A PCI-bus video accelerator driving a color LCD display panel, with a wide viewing area for high resolution VGA compatibility.

A wide range of I/O (Input/Output) ports including: a serial and parallel device, an external keyboard and PS/2 mouse port, an analog monitor, wireless IR (Infrared) communication port, jacks for a microphone and stereo audio input, headphone output, an expansion bus port, game port, and TV output jack.

A large integrated Glidepad, ergonomically placed to facilitate left or right handed use with today’s graphical user interfaces.

Advanced power management capabilities which conserve 90-95% of battery power by automatically shutting down inactive peripheral devices and system components.

A CD-ROM drive, compatible with all major data formats and music CD’s.

An integrated audio system, compatible with Sound Blaster, with audio input and output ports, a built-in microphone and stereo speakers.

# Unpacking the Computer

Your computer comes securely packaged in a sturdy cardboard shipping carton.  Upon receiving your computer, open the carton and carefully remove the contents.  In addition to this User’s Manual, the shipping carton should also contain the following items:

- The Notebook Computer

- An AC Adapter and AC Power Cord

- A NiMH Battery Pack(s)

- Utility Diskettes

- User’s Manual

Carefully inspect each component to make sure that nothing is missing and/or damaged.  If any of these items are missing or damaged, notify your dealer immediately.  Be sure to save the shipping materials and the carton in case you  need to ship or if you plan to store the computer away sometime in the future.

# Using the AC Adapter

The computer can be powered by either the rechargeable battery pack or the included AC adapter.  You will find detailed instructions on using both power sources in the next chapter.  Follow the steps below to prepare the battery pack and to attach the AC adapter.

Although the battery is fully charged when shipped from the factory, transit and shelf time will likely have reduced its charge.  Before using the computer for the first time, you should recharge the battery pack by attaching the AC adapter, as follows:

- Connect the DC power cable from the AC adapter to the AC adapter plug on the rear of the computer, as illustrated below.  Be sure to always handle the adapter cords by the plugs only.

- Connect the female end of the AC adapter to the AC adapter.

- Connect the male end of the AC power cord to an electrical outlet.

Warning: Use only the AC adapter that came with your computer.  Adapters for other electronic devices (including other notebook computers) may look similar, but could damage your computer if used.

AC Plug

Figure 1 - 2.   The AC Plug

When you have connected the AC adapter to the computer and to an electrical outlet, this outlet will now supply all power to the computer and will also recharge the battery.  To preserve the battery’s charge under normal use, you should use the AC adapter to power the computer whenever you have access to a proper voltage electrical outlet.

# Starting the Computer

Once you have connected the AC adapter, you are now ready to turn on the computer.  Follow the steps below to begin using your new computer:

- If you have not done so, slide and release the display lock latch on the front of the computer, and raise the LCD panel to a comfortable viewing angle.

- To power on the system, press the power on switch firmly.  You will then see the power indicator LED light up.

After a few seconds, the computer will begin to execute an internal diagnostic program which is automatically run whenever the computer is turned on.  The Power-On-Self-Test (or POST) checks the memory, the keyboard, the system board, and other components of your computer before the computer begins normal operation.

Warning: Never turn off or reset the computer while a disk drive is in use; doing so can result in the loss of or in the destruction of all data.  Also, always wait at least five (5) seconds after turning off the computer before turning it back on; turning the power on and off in rapid succession could damage the computer’s electrical circuitry.  If the computer does not power on, press the hardware reset switch and then press the power switch again.  This additional step may sometimes be necessary with fresh battery packs or if the system has been idle for a very long period of time.

Figure 1 - 3.   The On/Off Switch

On/Off Switch

# Using the Indicator Lights

The computer is equipped with a set of LED indicators located above the keyboard, which inform you of the current operating status of your computer at a glance.  The status indicators are illustrated and described below:

## Power On

This LED informs you  that the computer is powered on when lit.  When the battery is running critically low, the icon will blink and the computer will emit an intermittent audible beep.  You should consider this condition as a warning to save your data immediately and prepare an alternative power source (use an AC outlet or install a fresh battery pack).

## Battery Charge

This LED informs you that the computer is charging the battery when lit.  If off, the battery has been fully charged.

- Note: If the computer has been left unused for a prolonged period of time, it is possible for the battery to become completely discharged.  In such cases, it is necessary to connect the computer to the AC adapter for approximately thirty (30) minutes before the battery will begin to recharge normally.

## Media Access

This LED informs you that the computer is reading or writing to one of the systems disk drives (Hard, Floppy, or CD-ROM).

## PMU

This LED informs you that the computer’s Power Management Unit (PMU) features are enabled when lit.

## NumLock

This LED informs you that the keyboard number lock function is engaged when lit.  When lit, the keyboard’s embedded keypad is engaged.

## Cap Lock

This LED informs you that the keyboard caps lock function is engaged when lit.  When lit, the characters are entered in the upper case.

## Scroll Lock

This LED informs you that the keyboard Scroll Lock function is engaged when lit.  When lit, the text scrolls without altering the cursor position on the screen.

# Environmental Specifications

This section provides information on the optimum operating environment for your computer.

## Temperature

Operating:	32° to 104°F

(0° to 40°C)

Storage:		14° to 140°F

(-10° to 60°C)

## Relative Humidity (Noncondensing)

Operating:	20% to 80%

Storage:		10% to 90%

## Shock

Operating:	Maximum 5g pulse in X, Y, and Z orientations

Storage:		Maximum 50g pulse in X, Y, and Z orientations

## Vibration

Operating:	Sinusoidal 5 to 35.6 Hz limited to 0.015 inch peak to-peak

maximum displacement

0.5g, 25.6 to 250 Hz

Storage:		Sinusoidal 5 to 27.1 Hz limited to 0.016 inch peak-to-peak

maximum displacement

2.0g, 27.1 to 500 Hz

# Safety Precautions

Follow all warnings and instructions which may be marked on the computer.

Except as described elsewhere in this manual, refer all servicing to qualified personnel.  Immediately shut off the computer and refer to servicing if any of the following conditions occur:

- when the power cord or plug has become damaged or frayed

- if liquid has been spilled onto the computer

- if the computer has been dropped or if the cabinet has been damaged

Never push objects of any kind into cabinet openings.  They may contact dangerous voltage points or will short out parts that could result in fire or cause electrical shock.

Keep all liquids away from the computer and its accessories.

Turn off the computer before installing or removing a peripheral device.

Turn off the computer and disconnect the AC adapter before cleaning.

# Using the Computer

Never pick up or carry the computer by the display.

Never use the computer in harsh environments where it could be subjected to rapid temperature changes or excessive dust.

Never expose the computer to excessive vibration.

Never expose the hard disk drive or floppy disk(s) to strong magnetic fields, such as those generated by audio system speakers, telephone handsets, or hand-held metal detectors.

To avoid overheating the computer, never place anything over the top of the computer when it is operating or recharging the battery.

Before moving an active computer, press the Suspend hot key (Fn+ESC ) to put the computer into Suspend mode and close the display (refer to the Suspend Mode in the next chapter).

Do not try to force the display beyond its fully opened position - about 180 degrees.

# Power Saving Modes

Your computer offers several options for you to extend your battery life while you are not actively using your computer.  These options turn off different sections of the computer that is not currently being used.

These options are listed in the POWER MANAGEMENT page of the System Setup screen (accessed during boot up).  They are described below:

## Power Management

This option sets when you want the time outs to take effect.

ENABLED: Enables the time out settings regardless of whether the computer is on AC or battery power.

DISABLE ON AC: Only enables the time outs when the computer is running off of the battery power.

DISABLED: Disables all power time out settings.

## Standby Timeout

The amount of time the computer needs to be idle before the computer enters Standby mode.

## Suspend Timeout

The amount of time the computer needs to be idle before the computer enters Suspend mode.

## Hard Disk Timeout

The amount of time the computer needs to be idle before the hard disk will spin down the motor.

The computer will not enter any of the power management time out modes if the hard disk drive, CD-ROM, or floppy disk drive is active.

- Note: If the “Auto Insert Notification” option is enabled in the CD-ROM setting, the computer will not time out on any power management time-outs because Windows 95 will constantly be actively looking for a new CD to be inserted. This option is set to a default of “disabled” by the factory.

# Suspend Mode

Suspend mode is the lowest power consumption level that your computer can sustain.  There are two different suspend modes available for your computer.  One is the Suspend To RAM and the other is Suspend To Disk.  As a default your system is set to Suspend To RAM at the factory.  You can change the program for the computer by accessing the Suspend Mode utility in the Setup Program.

## Suspend To RAM

When the computer Suspends To RAM, an audible beep will sound, and the present state of the computer is stored in RAM.  All but a few essential components of the computer will then power down.  Pressing Fn + Esc will allow the computer to return to exactly the same position it was in before suspend mode was activated.  You can use Suspend To RAM when you want to take a break from your computing, but plan to resume after a short interval.  Operation will resume almost immediately after  pressing any key.

## Suspend To Disk

When the computer Suspends To Disk, an audible beep will sound, and the system will save all running applications to the suspend-to-disk partition file on the hard disk drive.  The computer will then automatically turn off.  When you next turn on the computer, it will read the suspend-to-disk partition and return all data to memory, so that your computer is returned to the exact state it was in before the suspend mode.

Suspend To Disk is a very useful feature as people frequently open many applications and then have them iconized on the Windows screen.  It will take some time for all of these applications to be opened and to begin running, and normally they will all have to be closed before the system can be turned off.  If you use the Suspend To Disk feature, you won’t need to close applications because the system status will all be saved to disk.  When you next turn on the computer, your Windows screen, with all applications open, will be recreated in just a few minutes.

## Suspend-to-Disk Partition

59 MB of hard disk space has been reserved as the Suspend-to-Disk Partition when your computer was shipped from the factory.  This means that you can safely suspend to disk if the computer has up to 48 MB of installed memory.

## Entering Suspend Mode

Suspend mode can be activated in any of the following ways:

- By pressing Fn + ESC

- By setting the Auto Suspend option in the Setup Program to automatically suspend after a certain period of computer inactivity.

- By closing the cover.

- By low battery voltage.

- Note: Your computer will not enter the Suspend To Disk mode if the Suspend-to-disk partition file is missing or is the incorrect size.  Refer to Chapter 2, Adding Memory section for further information.

## Resuming from Suspend Mode

Normally, resuming from a suspend mode is triggered as follows:

- By turning on the power switch.

- If Suspend To RAM is active, by pressing Fn + Esc.

- If the suspend state was entered by closing the cover, operation resumes when the cover is opened (as long as battery voltage is sufficient).

- If the suspend state was caused by the battery capacity dropping below a certain level, operation can be resumed only after AC power is connected or a charged battery installed, and then only by the power switch.

When a resume event occurs, the system returns to where it was when suspend mode was entered, e.g., Software applications open, the display reappears, etc.

# Securing the Computer

Your computer comes with a Kensington® Lock notch.  To secure your computer, follow the steps listed below:

Wrap the cable of a portable computer Kensington security lock around a table, desk, drawer handle, or any immovable object.

Locate the Kensington lock icon at the rear of the left side of the computer and insert the lock into the notch.

Turn the key to secure the lock.

Remove the key from the lock.

Kensington Lock Tab

Figure 1 - 4.   Kensington Lock Tab location.