Getting Started

2.

Running the BIOS Setup Program					O

Using the Brightness and Contrast Controls		O

Using the Glidepad									O

Using the Keyboard									O

Using Disk Drives									O

Using the Internal Numeric Keypad				O

Using the Connectors and Ports					O

Adding Memory										O

Using PC Cards										O

Turning the Speakers On and Off					O

Using a Microphone									O

# Running the BIOS Setup Program

Your computer is likely to have been properly setup and configured by your dealer prior to delivery.  However, you may find it necessary to use the computer’s BIOS (Basic Input-Output System) Setup program to change system configuration information, such as the current date and time, or your hard disk drive type.  The Setup program is stored in the Read-Only Memory (ROM), and can be accessed when you boot up the system.  The Setup program is accessed by pressing the F2 function key when powering on the Computer.

The settings that you specify within the Setup program are recorded in a special area of memory called the CMOS RAM.  This memory is backed up by a battery so that it will not be erased when you turn off or reset the system.  Whenever you turn on the computer, the system will read the settings stored in the CMOS RAM and compares them to the equipment check conducted during the Power On Self Test (POST).  If an error occurs, an error message will be displayed on the screen, and you will then be prompted to run the Setup Program.

The Setup program allows you easy access to a variety of configurable options.  At this point, however, you need only be concerned with the standard configuration (“bootup”) options on page 1 of the Setup program.  Once you have the computer up and running, you will want to refer to the next chapter for a discussion of the Setup program’s power management and other configuration options.

To enter the Setup program, press the F2 function key when the following prompt appears:

Press <F2> to enter SETUP

This will display the first page of the Setup menu,  which is illustrated below.

| Main Peripherals Security Power Savings Exit |  |
| --- | --- |
|  | Item Specific Help |
| System Time: [09:24:07] System Date: [07/07/1996] Diskette A: [1.44 MB, 3 ½] 4IDE Adapter 0 Master (C: 1.2 GB) 4Boot Options 4Keyboard Features System Memory: 640 KB Extended Memory: 48128 KB | Configure the IDE adapter |
| F1 Help éê Select Item - / + Change Values ESC Exit çè Select Menu Enter Select 4Sub-Menu | F9 Setup Defaults F10 Previous Values |

- Note: If you do not press these keys at the correct time and the system attempts to boot, an error message may be displayed ( such as “Missing operating system”).  There is no cause for alarm.  Simply, press CTRL+ALT+DEL to reset the system, and then, at the appropriate time, press F2 to enter BIOS Setup.

- Item Specific Help

- The right side of the screen.  This area describes each parameter and its available settings.

- Menu Bar

- The top line of the screen.  Each of the five selections displays its own screen.

- Parameters

- The left side of the screen.  This area lists the parameters and their current settings.

- Key Status Bar

- The bottom part of the screen.  These lines display the keys available to move the cursor, select a particular function, and so forth.

## Changing Bootup Settings

The bootup settings page of the Setup program will allow you to change the following information:

### System Time

This option allows you to change the system time using the format hour:minute:second.  You can change the system date here and/or from your operating system command prompt.

### System Date

This option allows you to change the system date using the format month, day, year.  You can change the system date here and/or from your operating system’s command prompt

### Diskette A

This option allows you to specify the type of floppy disk drive mounted inside your computer.  The selections are: [1.44 MB, 3 ½”], [Not Installed].  The computer comes equipped with a 3.5”, 1.44 MB floppy disk drive.

### IDE Adapter 0 Master

This option will automatically detect the hard disk drive type for drives that comply with ANSI specifications.

### Boot Options

This option allows you to determine in what order the system searches the drives for the proper boot sequence.  The selections are: [FDD], [HDD], or [CD-ROM].

### Keyboard Features

This option allows you to select “On” or “Off” state for Numlock.

### System Memory

This field reports the amount of base (or conventional) memory found by the BIOS during its POST.  The value will not exceed 640 KB.

### Extended Memory

This field reports the amount of extended memory found by the BIOS during its POST.  The value displayed is the amount of memory located (above 1 MB) in the microprocessor’s memory address map.

## Main Menu / IDE Adapter 0 Master (None)

### Autotype Fixed Disk

Attempts to automatically detect the hard disk drive type for drives that comply with ANSI specifications.  Press Enter to continue on to the next field.

### Type

This field selects the drive type corresponding to the fixed hard disk drive installed in your system.  The selections for this option are: [User] / [Auto] / [None].  If type [User] is selected, Cylinder, Heads, Sectors/Track, and Write precomp fields will need to be edited directly.  A Write precomp of None can be specified by pressing N.

## Main Menu / Boot Options

### Boot From

This option allows you to determine in what order the system searches the drives for the proper boot sequence.  Place the following selections in the order that you want the computer to search the computer for a boot drive.  The selections are: [FDD], [HDD], or [CD-ROM].

### Floppy Check

Selecting “Enabled” will cause the computer to verify the floppy disk drive type on boot, and selecting “Disabled” will speed up the boot process.

## Main Menu / Keyboard Features

### Numlock

This selection allows you to select the “On” or “Off” state for Numlock.

### Keyboard auto-repeat rate

This selection sets the selected key repeat rate.

### Keyboard Auto-repeat delay

This selection sets the delay time before the selected key will begin to repeat.

# Peripherals Menu

## COM1 Port

This selection will set the serial port address.  The selections are: [Disabled] / [3F8, IRQ4] / 2F8, IRQ3] / [3E8, IRQ4] / [2E8, IRQ3].  The default is [3F8. IRQ4].

## COM2 Port

This selection will set the parallel port address.  The selections are: [Disabled] / [3F8, IRQ4] / [2F8, IRQ3] / [3E8, IRQ4] / [2E8, IRQ3].  The default is [2F8, IRQ3].

## COM2 Mode

This selection sets the parallel mode.  The selections are: [FIR] / [Modem].  The default is [FIR].

## LPT Port

This selection will set the LPT port address.  The selections are: [Disabled] / [378, IRQ5] / [3BC, IRQ7] / 278, IRQ5] / [378, IRQ7].  The default is [378, IRQ7].

## LPT Mode

This selection sets the LP T mode.  The selections are: [Unidirectional] /

Bi-Directional] / [Enhanced] / [ECP].  The default is [Bi-Directional].

## On Board Audio

This selection “Enables” or “Disables” the  ESS1788 sound chip.  The default is [Enabled].

## I/O Channel

This selection allows you to choose either; [220h] / [230h] / [240h] or [250h].  The default is [220h].

## IRQ Channel

This selection allows you to choose either; [IRQ5] / [IRQ7] / [IRQ9] or IRQ10.  The default is [IRQ5].

## DMA Channel

This selection allows you to choose either; [DMA 0,1] / [DMA 1,3] or [DMA 3,0].  The default is [DMA 3,0].

## FM I/O

This selection allows you to choose either; [388h] / [390h] / [398h] or [3A0h].

The default is [388h].

## MPU I/O

This selection allows you to choose either; [300h] / [310h] / [320h] or [330h].  The default is [300h].

## Game Port

This selection “Enables” or “Disables” the interface to the “Game Port”.  The default is “Enabled”.

## Display to TV

This selection allows you to choose either; [Disabled] / [NTSC] or [PAL] for the interface to the “TV Display”.  The default is “Disabled”.

## Large Disk Access

If you have DOS, select [DOS].  If you have UNIX, Mode, Novell Netware, or other operating systems, select [Other].  If you are installing new software and the drive fails, change this selection and try again.  Different operating systems may require different representations of the drives geometry.

# Security Menu

## Set Password

Press <Enter> to set a password.  Type the desired password, press Enter and type it again.  The system will now require the password to enter the Setup Utility.  To remove the password when user password is enabled, press Enter twice without typing any password in the blanks.  After setting a password the “User Password” will become enabled.

## Password on Boot

This selection allows you to choose whether the system will require a password to be entered on boot.  You will choose “Enabled” if you want a password, and “Disabled” if you  do not want a password.  The default is “Disabled”.

## Password on Resume

This selection allows you to choose if the system will require a password  to be entered upon resuming from a suspend mode.  You will choose “Enabled” if you want a password, and “Disabled” if you do not want a password.  The CapsLock and ScrollLock LED’s will flash in sequence for 30 seconds to indicate that a password is required to resume.  The LED’s will start flashing again, as soon as any key is hit, until the correct password is entered.  The default is “Disabled”.

# Power Saving Menu

## Power Savings

This selection allows you to select the Power Management Mode.

Choose “Disabled” to turn off Power Management.

Choose “Disabled ON AC” to turn off Power Management only while connected to AC power, but will turn on Power Management while on battery power.

Choose “Enabled” to turn on Power Management all the time.

The default is “Disabled ON AC”.

### Standby Timeout

This selection allows you to choose the length of  time the system will be idle before entering the Standby Mode.  Standby Mode will place the Serial Ports, the Infrared Port, the Parallel Port, the Floppy Disk Drive, and the Hard Disk Drive into the power savings node.  The default is “2 min”.

### Suspend Mode

This selection allows you to choose how the system will save your data when entering the Suspend Mode.  Selecting “Save to Disk” will save the system’s state to disk and will then power off the entire system.  Selecting ‘Save To RAM” preserves the system’s state and will then enter the low power mode.  The default is “Save To RAM”.

### Auto Suspend Timeout

The length of time the system is in the Standby Mode before entering Save To RAM Mode.  The default is “2min”.

### Hard Disk Timeout

The length of time the hard disk is inactive before the hard disk is put into the Power Savings Mode.  The default is “2 min”.

### Cover Switch Control

Closing the LCD Cover will make the system either turn off the LCD or enter into the Suspend mode, depending on the system configuration indicated in the BIOS Setup program.  The default is “LCD Off”.

### Ring Resume

Turning this feature “On” will wake up the system when an incoming call is received from a modem or fax.  The default is “Off”.

# Exit Menu

## Save Changes & Exit

Pressing this will cause the program to Exit after writing all changed SETUP item values to CMOS.

## Discard Changes &Exit

Pressing this will cause the program to Exit without saving changed SETUP item values.

- Warning - Any changes in password will still take effect whether you Save Changes or Do Not Save Changes..

## Set Default Values

Pressing this will Load the Default values for all SETUP items.

# Using the Brightness and Contrast Controls

You can adjust the screen brightness and contrast levels using your computer hot keys.

- Note: The hot keys for brightness and contrast are continuous keys.  They will adjust as long as you hold them down.

## Brightness Control

Press Fn +  and Fn + ¯ to increase and/or decrease the brightness of the display respectively.  The brighter the screen setting, the more power is used during battery operation.

## Contrast Control

Press Fn + ® and Fn + ¬ to increase and/or decrease the contrast of the display respectively.  The higher the contrast setting, the more power is used during battery operation.

- Note: You can adjust the contrast only on DSTN LCD’s.

# Using the Glidepad

## Moving the Cursor

The embedded glidepad offers a unique and efficient way of pointing and selecting in a Windows environment.  The following figure shows the glidepad.

CD-ROM Drive

Glidepad

Floppy Disk Drive

Figure 2 - 1.   Glidepad Location.

The Glidepad responds to finger movements on its surface.  To move the cursor, move your finger along the Glidepad surface.

Once the cursor is in the proper place, tap once on the surface of the Glidepad or use the left button to click just the same as you would for a mouse.  Tap twice to double-click.

# Using the Disk Drives

Unlike a “floppy” diskette, a hard disk drive is rigid and completely sealed in a protective, dust-free environment.  A hard disk drive works very much the same as a diskette, but can retrieve and record data much faster and has a much larger storage capacity.

The computer is equipped with a 2.5” IDE (Integrated Drive Electronics) drive.  This type of drive embodies the latest in fast, reliable mass storage by integrating all of the control circuitry necessary for its operation directly onto the drive itself. This in turn allows the hard drive manufacturer to carefully optimize drive performance.  Once you have installed an operating system onto your hard drive, you will want to install all of your software applications onto it as well, and perform your work from the hard disk, using diskettes only for backup and archival purposes.

# Caring  for the Hard Disk Drives

The hard disk drive included with the computer is designed for portability, but it is not indestructible. Moreover, while modern hard disk drives are exceptionally reliable, they do fail occasionally.  To avoid damaging your disk drive, and to protect your valuable data, you should take the following precautions:

Never turn off or reset the computer while the HDD status icon is lit.

Make regular backups of your hard disk drive to diskettes, and keep a backup copy on hand.

Keep the AC adapter at least six (6) inches away from your hard disk drive(s).

# Using the CD-ROM Drive

The computer comes with a 10X (or higher) speed CD-ROM drive that is compatible with existing music and data CD’s.

This is a Class 1 Laser product.

## Inserting a CD

Press the eject button on the CD-ROM drive to eject the disk tray.

Place the CD disk onto the tray with the label side up.

Push the tray in by hand to return the tray to the drive.

## Handling CDs

Follow these guidelines to avoid damaging your CDs and to prolong the life of your CD-ROM player:

- When removing a disk from it protective cover or when loading a disk into a drive, hold the disk by its central core hole and an outer edge.  Never touch the disk’s data surface (non-labeled side).

- Never write on a disk or place a label on the disk surface.

- To protect the disk against scratches and dirt when not in use, always keep the disk in its protective case or cover.

- When cleaning disk, always wipe in the direction of the center to the outer edge.  Don’t wipe the CD in a clockwise or counterclockwise direction.

- To remove dust or fingerprints, use a clean, soft and dry cloth.  Never use benzene or anti-static fluids.

- Keep disk away from high temperatures or direct sunlight.

## CD-ROM Drive Guidelines

Failure to observe the following precautions can damage both the CD-ROM drive and the data on the CD:

- Do not open the disk tray except when inserting or removing a disk.

- Never push down on an open disk tray.

# Using the Keyboard

The computer is equipped with either an 87- or 88-key keyboard, which provides all the functionality of a full-sized desktop 101-key keyboard.  Even if you are an experienced PC user, you should take a moment to become familiar with the layout of your computer’s keyboard.  In particular, you should familiarize yourself with the special computer function keystrokes which allow you to quickly and easily control and access the power management hot keys.

The computer has many special keys, most of which depend on an application for their functionality.

## Special Keys

The following keys have special functions at the command level of MS-DOS and within many different programs:

Pause

Stops a command or application; primarily used to stop the screen from scrolling; pressing any other key resumes the execution of the command or application.

Shift+PrtSc

Sends the contents of the screen to the printer port; prints only text characters unless you have run the GRAPHICS.COM utility to enable printing graphics.

Ctrl+Break

Terminates the current command or application.

Ctrl+P

Sets the computer to echo keystrokes to the printer; prints a line when you press Enter; continues until you press Ctrl+P again.

Ctrl+Alt+Del

Terminates all programs, reloads MS-DOS and executes the AUTOEXEC file; also called “warm start” or “warm boot’.

The following keys are the system hot keys for your computer:

F2

Loads the ROM-based Setup when pressed during POST (power on self test) at computer startup.

Fn+F10

Toggles the screen output between the LCD, external CRT, TV set, or to LCD and CRT at the same time (called SIMULSCAN).

Fn+ESC

Enters and/or Exits Suspend mode, or wakes the system up from the Suspend mode.

Fn+®

Adjusts the contrast brigher.  (On DSTN LCD panels only.)

Fn+¬

Adjusts the contrast darker. (On DSTN LCD panels only.)

Fn+

Adjusts the brightness up.

Fn+¯

Adjusts the brightness down.

## Using the Internal Numeric Keypad

The keyboard has an embedded keypad that provides the same functions as the discrete numeric keypad on an AT® enhanced keyboard.

The embedded numeric keypad keys generate AT-keypad characters and functions when pressed in conjunction with Num Lock, Fn and Shift.

The embedded numeric keypad has two modes you can enter by toggling Num Lock as signaled by the Num Lock indicator: ON or OFF.

Num Lock On

When the Num Lock indicator is on, pressing a key generates the characters shown in the following figure.

Pressing Shift with a key generates the characters shown in the following figure.

If you press and hold Fn in this mode, the keypad generates their normal characters.

Num Lock Off

When the Num Lock indicator is OFF, the keyboard acts as normal.

Pressing Fn with a key generates the same characters mentioned in the previous paragraph, Num Lock On (with Shift).

# Using the Connectors and Ports

This section provides a description of the connectors and ports on the rear and left side panels of the computer.

## The Rear Panel

Serial Port

Parallel Port

Game/MIDI Port

VGA Port

PS/2 Mouse/Keyboard Connector

Expansion Port

DC In

Figure 2 - 2.	Rear panel of the Notebook Computer

DC In

Connects the AC adapter output connector to this jack to recharge the battery and to supply power to the computer.

- Caution: Use only the supplied AC adapter with your compute.  Other adapters could cause serious damage to the electronic circuits.

Serial

Connects to external devices such as a serial printer.  16550 UART compatible.

Expansion

Connects to a port replicator.

Parallel

Connects to a parallel printer or other device that uses a standard parallel interface; EPP/ECP compatible; also connects to the floppy disk drive when used externally.

External Monitor

Connects to an external analog monitor.  (15-pin)

PS/2

Connects to an external PS/2 keyboard, numeric keypad, or mouse.

If you are connecting a keyboard with a 5-pin DIN connector, you will need to purchase a 6-pin mini-DIN adapter.

Game/MIDI

## The Left Side Panel

FIR Window

Headphone

Line In

Line Out

Microphone

Volume Control

TV Port

PCMCIA Slots

Figure 2 - 3.	Left side panel of the Notebook Computer

FIR

Connects to any IrDA compliant device (such as another IrDA computer or printer) without the use of a cord or cable.  Transmits up to 4 M bits per second.

Microphone

Connects to an external microphone.

Line Out

Connects to a line out device such as headphones or amplified speakers.

Line In

Connects to a line-in device such as a synthesizer, stereo Walkman or

audio CD player.

Headphone

TV Port

Connects to an external TV.

PC Card

The PC Card slots support one Type III or two Type I/II PC Cards.

# Adding Memory

Your computer has 16 MB of EDO Dynamic Random Access Memory (DRAM) for computer system memory.  The computer also allows you to install and expand system memory via a single onboard SO-DIMM (Small Outline Double In-line Memory Module, 144 Pins) socket.  The SO-DIMM socket is easily accessible and can accept a 16 MB, 32 MB  or 64 MB SO-DIMM module, which are available from your dealer.

Whenever you change your computer memory size, you should run the PHDISK utility to re-size your Suspend to Disk file.  This file is used to store all your computer information onto the hard disk drive for the 0V Suspend function.  If you do not re-size the Suspend to Disk file, you will not be able to use the Suspend to Disk utility.

To add memory, locate the memory

FOLLOW THE INSTRUCTIONS THAT CAME WITH YOUR MEMORY UPGRADE OPTION TO INSTALL THE ADDITIONAL MEMORY.

# Using PC Cards

The computer is equipped with two PCMCIA sockets which can accommodate Type I/II or Type III cards conforming to the standards of the PCMCIA (Personal Computer Memory Card International Association).  These include the following types of devices:

## Flash Memory

A solid-state storage device that works like a floppy drive, with a write-protect switch on the edge of the card.  Unlike conventional memory, a flash memory card does not require an external power source to maintain data when removed from the socket.

## SRAM Memory

A static RAM memory device that can be formatted as a diskette or an IDE hard disk drive, with a write-protect switch on the edge of the card.  An on-card battery maintains the data when it is removed from the computer’s PCMCIA socket.

## Network

A device that connects the computer to a local area network (LAN) such as an Ethernet.

## Fax/Modem

A device that connects the computer to the telephone line for use as a fax and/or a modem.

## ATA Device

An AT Attachment, rotating or solid-state mass-storage device that works like an IDE hard disk drive.  An ATA device does not require an external power source to maintain data when removed from the computer’s PCMCIA socket.

## Inserting and Ejecting PC Cards

PC cards are inserted and ejected in much the same way as diskettes.

- Open the PCMCIA compartment cover on the left side of the computer.  (It is unnecessary to power off the system when handling PC cards and devices.)

- To insert a PC card, align the card with the appropriate socket and slide the card into the socket until it locks into place.  Note that some PC Cards must be prepared by a PCMCIA driver for the computer before you can use them for data storage, see your PC card manual for details.

- To eject a PC card, first ensure that the computer is not accessing the memory card or device, and then press the appropriate eject button on the socket.  When the card pops out of the socket, remove the card and store it properly.

## Configuring and Using PC Cards

To use a PC card with the computer, your operating system must be configured to recognize the device.  PCMCIA utility software is provided with the computer, containing the latest available drivers and a file containing instructions on how to use them.  The PCMCIA utilities provide the following features:

- an installation program for simplified installation and configuration of PCMCIA drivers and utilities.

- support for the Microsoft Flash File System II (FFSHII).

- a PC Format utility which can format a SRAM PC card as floppy disk drive D: or E:.

- an easy-to-use information utility for managing PC cards formatted with PC Format.

- support for hot insertion and removal of PC cards under DOS without the need to restart the computer.

- support for Power Management.

- support for booting from a PC card designated as a hard disk drive or floppy disk drive.

The following instructions should improve the relationship between the computer hardware and the supplied PCMCIA utility software under DOS.

### Installation

To install computer PCMCIA support, insert the Phoenix PCMCIA driver utility and type A:Install.exe at the operating system command prompt.  Select the PCMCIA installation option from the setup menu and follow the instructions on the screen.

### Configuration

To configure or reconfigure PCMCIA software support after installation, change directions to the directory where you installed the PCMCIA software and type PCMSETUP at the prompt.  Follow the instructions on the screen.

### Documentation

To install the driver, insert the Phoenix PCMCIA driver utility and type A:Install.exe at the operating system command prompt.  Select the appropriate computer documentation option from the Setup menu and follow the instructions on the screen.  To view PC card documentation, copy the USERMAN.ZIP file from drive A to drive C, decompress the file, then view the documentation in MS Word 6.0.

### Windows Installation

To install the PCMCIA driver for Windows 95, follow the steps listed below:

- Insert the PCMCIA driver utility diskette into drive A.

- Double click Control Panel / Add New Hardware / PCMCIA socket.

- Click “Have Disk” to change the driver.

- Follow the instructions on the screen to complete the installation process.

# Using the Audio System

The audio capabilities of the computer bring high quality stereo sound to your software applications, including features like an FM synthesizer, and digital recording and playback.  The computer’s integrated audio system is fully compatible with the Windows Sound System, and includes a number of applications on diskette which allow you to record, compress, store, and playback voice, sound, and music under the Windows environment.  The audio system is also Sound Blaster compatible, so you can hear all the digital sound effects and voice recordings used by the latest entertainment software.

The computer’s audio capabilities include the following features:

- Digitized audio playback capability (up to 44KHz), which faithfully plays back and reproduces all kinds of digitized sounds with a 16-bit Digital-to-Analog Converter(DAC).  The audio system works with a number of applications that utilize the digital voice channel to deliver realistic human speech and sound effects.

- Digitized audio recording capability (up to 44KHz) allowing digitization and recording of any kind of sound through the computer’s built-in microphone or an external source.

- High quality sound, dynamically filtered for low noise digital recording and playback.  Input from the computer’s built-in microphone uses Automatic Gain Control (AGC) to compress sound input to adapt dynamically to different recording conditions.

- Audio compression which offers real-time compression of digital audio at rates of 2:1, and real-time decompression at rates of 8:1.  Compression turns large sound files into smaller files that can easily be stored and transferred to a diskette or even sent over a network.

- Built-in amplifier and speaker, including volume control, and a sound-out port for connection to an external pair of headphones with power amplified speakers.

## Adjusting Volume

Computer sound volume can be adjusted through the use of the volume control knob on the left side of the unit.

## Audio Software

The audio system supports a large library of third party software packages include music/sound drivers.  All you have to do is select the correct music/sound driver when you install the package, or follow the package’s instructions for selecting the driver.  You may select the Windows Sound System, or Sound Blaster Pro.

The computer also includes software which includes a special driver and accessories that are designed especially for the Microsoft Windows environment.  These audio tools require Microsoft Windows 3.1 or a later version.

To install the audio driver for Windows 3.1, follow the steps below:

- Insert the ESS1878 audio driver utility diskette into drive A.

- Run Setup.exe on the diskette from the Windows file manager.

- Follow the instructions on the screen to complete the installation process.

When using the optional audio system with the computer’s save-to-disk feature enabled, it may occasionally be necessary to restart Microsoft Windows after a resume event in order to reinitialize the audio device drivers.

To install the audio driver for Windows 95, follow the steps below:

- Insert the ESS1878 audio driver utility diskette into drive A.

- Double click “Control Panel / System / Device Manager”.

- Remove the “?” device.

- Double click  “Control Panel / Add New Hardware / Sound”.

- Follow the instructions on the screen to complete the installation process.

### Using the Video System

To install the NM2093 VGA driver for Windows 3.1, follow the steps below:

- Insert the NM2093 VGA driver utility diskette into drive A.

- Run Setup.exe on the diskette from the Windows file manager.

- Follow the instructions on the screen to complete the installation process.

To install the VGA driver for Windows 95, follow the steps below:

- Insert the NM2093 VGA driver utility diskette into drive A.

- Double click “Control Panel / Add New Hardware / Display Adapters”.

- Click “Have Disk” and select “NeoMagic MagicGraph 128ZV” to change the driver.

- Follow instructions on the screen to complete the installation process.

# Using the Microphone

Your computer has a built-in microphone located in the upper left corner of the LCD cover.

To use the microphone, face your computer from a normal distance and speak in a normal voice.  You do not need to bend down to speak directly into the microphone.