# Chapter 4

Making Upgrades Easy

## Before You Begin

Make sure you have a stable, clean working environment.  Dust and dirt can get into computer components and cause malfunction.  Use containers to keep small components separated.  Many of the screws on the computer are different sizes; if you put them together you may spend a lot of time trying to track down the correct screw when you go to put the computer back together.  Also, putting all small components in a safe container keeps them from getting lost.  Having adequate lighting and using proper tools can prevent you from accidentally damaging the internal components.  Most of the procedures that follow require only a few simple tools, including the following:

- A Philips Screwdriver

- A flat tipped screwdriver

- A set of jeweler’s screwdrivers

- A multimeter (not required)

- A grounding strap

Most of the electrical connections can be disconnected by using your fingers.  It is recommended that you don’t use needle nosed pliers to disconnect connections as these can damage the soft metal or plastic parts of the connectors.  Many connectors have tabs to keep them secure.  After the tabs are disengaged the connector should easily slide out.  You can use a small screwdriver to gently pry some of the connectors loose.  If the connector doesn’t come out easily, make sure that nothing is obstructing it, and gently rock it back and forth in its slot until it loosens enough to come out.  Don’t force it!

Before working on internal components, make sure that the computer’s power is off.  Ground yourself before touching any internal components, by touching a metal object.  Static electricity can damage many of the computer’s electronic components.  Humid environments tend to have less static electricity than dry environments.  A grounding strap is warranted whenever danger of static electricity exists.

### Removing the Chassis Cover

Before you can do any internal upgrades, you must remove the chassis cover.  Refer to the following figure for information on removing the cover:

Figure 4-1: Removing the System Chassis Cover

- Locate the securing screw at the rear of the computer.

- Turn the screw counterclockwise to release the cover.

- Pull the chassis cover towards you and then upwards.  It should slide out easily.  If it doesn’t, move it back and forth until it is clear of the chassis.

### Familiar with  Major Components

Figure 4-2 shows the major components of system.

Figure 4-2: Major Components of System

- Power Supply

- CD-ROM Drive Holder

- HDD/FDD Drive Holder

- Mainboard

- Riser Card for Add-On Card

- LAN or Fax-Modem Module

### Removing the Front Panel Chassis

Before you upgrade the HDD/FDD and CD-ROM holders, you have to remove the front panel chassis. Please refer to Figure 4-3 and follows the steps listed below:

Figure 4-3: Removing the Front Panel Chassis

- Loosen the two screws that affix both sides of the front chssis on the main body of the system.

- Loosen the screw that affixes the front panel chassis on the main body of the system.

- Remove the front panel chassis away from the main body of the system.

- Disconnect all the connecting wires and cables.

### Upgrading the HDD

To upgrade your HDD, please refer to Figure 4-4 and follow the steps listed below:

Figure 4-4: Upgrading the HDD

- Loosen the two screws that affix the HDD/FDD holder on the front panel chassis.

- Remove the HDD/FDD holder from the front panel chassis.

- Place the HDD drive into the HDD/FDD holder.

- Fasten the HDD drive on the holder by two screws on each side of the holder.

### Upgrading the CD-ROM

To upgrade your CD-ROM, please refer to Figure 4-5 and the follow the steps listed below:

Figure 4-5: Upgrading the CD-ROM

- Place the CD-ROM drive into the CD-ROM holder.

- Affix the CD-ROM drive on the holder by securing four screws on the bottom of the front panel chassis.

### Installing the HDD/FDD Holder

Figure 4-6: Affixing the HDD/FDD Holder

Affix the HDD/FDD holder on the front panel chassis by securing the two screws on each side of the holder as shown above.

### Installing an Expansion Card

You can make various upgrades to your system by installing the expansion cards on the riser card as shown in Figure 4-7.

The table below lists the maximum sizes of the add-on cards, please note it before your purchasement.

| Card | Width | Length |
| --- | --- | --- |
| ISA | not exceed 90mm | not exceed 170mm |
| PCI1 | not exceed 90mm | not exceed 160mm |
| PCI2 | not exceed 90mm | not exceed 135mm |

Figure 4-7: Installing an Expansion Card

- Loosen the screw that affixes the braket.

- Insert the expansion card to be firmly seated in it’s slot.

- Affix the expansion card by securing the screw on the bracket.

### Reinstalling the Front Panel Chassis

Figure 4-8: Reinstalling the Front Panel Chassis

To reinstall the front panel chassis follow these steps:

- Slide the front panel chassis onto the main body of the system.

- Connect all the wires and cables to the appropriate drives. To connect the wires to the Audio CD-IN connector on the mainboard with the CD-ROM drive, please refer to Page 55.

Tighten the front panel chassis with the main body of the system by     the three screws that mentioned in Figure 4-3.

— This page intentionally left blank —