# Appendix b

Installation Procedures

The mainboard has several user-adjustable jumpers on the board that allow you to configure your system to suit your requirements. This chapter contains information on the various jumper settings on your mainboard.

To set up your computer, you should follow these installation steps:

- Step 1 -

- Setting system jumpers

- Step 2 -

- Installing RAM modules

- Step 3 -

- Connecting wires and cables

- Step 4 -

- Setting BIOS feature

### Mainboard Layout

## 1). Setting System Jumpers

Jumpers

Jumpers are used to select the operation modes for your system. The type of jumper used in this mainboard are shown below:

|  |  | Users are not encouraged to change the jumper settings not listed in this manual. Changing the jumper settings improperly may adversely affect system performance. |
| --- | --- | --- |

Clear CMOS: CLEAR

This jumper allows you to clear the CMOS user-defined configurations when you set the 2-3 pin pair to be short.

## 2). Installing RAM Modules

### DRAM Memory

The working space of the computer is the Random Access Memory (RAM). The system cannot act upon data unless it is loaded into RAM. The system RAM is comprised of industry-standard 72-pin Single In-line Memory Modules (SIMMs).

Occasionally the system must break apart data files because the entire file can not be fitted into the RAM area. Consequently, when the system needs data that is not in RAM, it must access the disk where the balance of the data is stored. Compared with the lighning speed accessing a RAM, accessing a mechanical disk drive is a much slower process.

When more RAM is added, the working area of the computer is larger, thereby increasing total performance. You should verify the type and speed of the RAM currently installed from your dealer. Mixtures of the RAM types, other than those described in this manual, will have upredicatable results.

The mainboard is able to support standard FPM (Fast Page Mode) and EDO (Extended Data Out) DRAMS; and can accommodate onboard memory from 8 to 64MB using SIMMs. The mainboard has two memory banks, Bank 0 and Bank 1. Each bank has two SIMM sockets which can accept either a pair of  4MB, 8BM, 16MB,  or 32MB SIMM in each socket. Bank 0 and Bank 1 allow different sizes of SIMMs (e.g. 4MB or 16MB); however, you must populate each memory bank with the same size (e.g. 4MB on SIM1, 4MB on SIM2, 16MB on SIM3, 16MB on SIM4) of  SIMM.

### RAM Module Configuration

| TOTAL MEMORY | SIM 1 & 2 (72-PIN X 2) | SIM 3 & 4 (72-PIN X 2) |
| --- | --- | --- |
| 8MB | 4MB & 4MB |  |
| 16MB | 8MB & 8MB |  |
|  | 4MB & 4MB | 4MB & 4MB |
| 24MB | 8MB & 8MB | 4MB & 4MB |
| 32MB | 8MB & 8MB | 8MB & 8MB |
|  | 16MB & 16MB |  |
| 40MB | 16MB & 16MB | 4MB & 4MB |
| 48MB | 16MB & 16MB | 8MB & 8MB |
| 64MB | 16MB & 16MB | 16MB & 16MB |
|  | 32MB & 32MB |  |

The following EDO SIMMs are recommended for this system. Any other type of SIMM may not fit for the system.

| VENDOR | MODEL |  |  |
| --- | --- | --- | --- |
| TOSHIBA | TC5118165BJ-60, 8MB |  |  |
| NEC | UPD4218165LE-60, 8MB |  |  |
| MITSUBISHI | M5M418165CJ-6, 8MB |  |  |
| SAMSUNG | KM416C1204BJ-6, 8MB |  |  |
| MICRON | MT4C1M16E5DJ-60, 8MB |  |  |

### Installing SIMMs

|  | Turn the system power off before installing or removing any device. |
| --- | --- |

Complete the following procedures to install SIMMs:

- Locate the SIMM slots on the mainboard.

- Carefully fit the SIMM at a 22.5 degree angle into an available socket to be populated. All the SIMMs must face the same direction.

3.								  										Press down each SIMM into the locked position.

### Removing SIMMs

Press the retaining latchs on both ends of the socket, then take out the SIMM.

## 3). Connecting Wires and Cables

Audio CD-IN Connector: CD_IN

These 4-pin male connector enables the system to receive the audio output from the CD-ROM.

## 4). Setting BIOS Feature

The mainboard comes with the Phoenix BIOS chip that contains the ROM Setup information of your system. This chip serves as an interface between the processor and the rest of the mainboard's components. This chapter explains the information contained in the Setup program and tells you how to modify the settings according to your system configuration.

A Setup program, built into the system BIOS, is stored in the CMOS RAM. This Setup utility program allows changes to the mainboard configuration settings. It is   executed when user changes system configuration; user changes system backup battery; or the system detects a configuration error and asks the user to run the Setup program. As power-on RAM testing, the message Press F2 to enter Setup. appears. Use the arrow keys to select and press Enter to run the selected program.

## Main CMOS Setup

The main CMOS Setup screen is displayed above. Each item may have one or more option settings. The system BIOS automatically detects memory size, thus no changes are necessary. Use the arrow keys to highlight the item and then use the space bar to select the value you want in each item.

### IDE Adapter Configuration

Autotype Fixed Disk

Set at Press Enter will allow the system BIOS to automatically detect the drive type for drives that comply with ANSI specifications.

Type

Select the drive type corresponding to the fixed disk installed in your system. If Type User is selected, Cylinders, Heads, Sectors/Tracks, and Write Precomp fields can be edited directly.

Cylinders

The cylinder number of the hard disk.

Heads:

The read/write head number of hard disk.

Sectors/Track

The sector number of each track defined on the hard disk.

Write Precomp

The cylinder number at which the disk drive changes the write timing.

Multi-Sector Transfers

Determines the number of sectors per block for multiple sector transfers.

The options are: Disabled (Default), 2, 4, 8, 16 Sectors.

LBA Mode Control

Enabling LAB causes Logical Block Addressing to be used in place of Cylinders, Heads, and Sectors.

The options are: Disabled (Default), Enabled.

32 Bit I/O

Enables 32 bits data transfer.

The options are: Enabled, Disabled (Default).

Transfer Mode

Selects the optimum transfer mode of the hard disk drive.

The options are: Standard (Default), Fast PIO 1, Fast PIO 2, Fast PIO 3, Fast PIO 4.

### Memory Cache

Internal Cache

Allows you to enable or disable the L1 cache.

The options are: Enabled (Default), Disabled.

L1 Cache Policy

Allows you to set the L1 cache write policy to Write Back (write to DRAM are deferred) or Write Through (writes to DRAM are done immediately).

The options are: Write Back (Default),  Write Through.

L1 640K to 1M Cache Policy

Allows you to set the L1 cache write policy for the 640 KB to 1 MB region to Write Back (write to DRAM are deferred) or Write Through (writes to DRAM are done immediately).

The options are: Write Back, Write Through (Default).

### Memory Shadow

System BIOS Shadow

When set at Enabled and Cacheable, it allows the system BIOS to copy the BIOS ROM code to the system and cache memory for faster access.

The options are: Enabled and Cacheable (Default), Enabled.

Video BIOS Shadow RAM

When set at Enabled and Cacheable, allows the BIOS to copy the video ROM code of the add-on video card to the main and cache memory for faster access.

The options are: Enabled and Cacheable (Default), Enabled.

Shadow Memory Region

These options control the location of the contents of the ROM beginning at the specified memory location. If no adapter ROM is using the named ROM area, this area is made available to the local bus.

[Disabled] The video ROM is not copied to RAM. The contents of the video ROM cannot be read from or written to cache memory.

[Enabled] The contents of C800h - DFFFh are written to the same address in system memory (RAM) for faster execution.

[Enabled and Cacheable] The contents of the named ROM area are written to the same address in system memory (RAM) for faster execution, if an adapter ROM will be using the named ROM area. Also, the contents of the RAM area can be read from and written to cache memory.

The settings are Disabled (Default), Enabled and Cacheable, Enabled.

### Boot Options

Summary Screen

When enabled, allows the BIOS to display the system configuration when booting up.

The options are: Enabled (Default), Disabled.

Floppy Check

When enabled, allows the BIOS to verify the floppy disk drives when booting up.

The options are: Enabled (Default), Disabled.

Numlock

When set to Auto, allows the BIOS to automatically enable the Num Lock Function when the system boots up.

The options are: Auto (Default), On, Off.

Typematic Override

Allows you to select the keyboard repeat rate. Normal is 2 character/second. Fast is 30 character/second.

The options are: Fast (Default), Normal.

## Advanced Features Setup

Plug & Play O/S

Select Yes if your are using a Play & Plug capable operating system, such as Windows 95.

The options are: No, Yes (Default).

Large Disk Access Mode

Select DOS if you have DOS. If you use UNIX, Novell's Netware, or any other operating system except DOS, select Other. If you fail in the installation of some new software package, change your selection in this item and try again. Different operating system need different representations of drive geometries.

The options are: DOS (Default), Other.

### Integrated Peripherals

COM port A

Allows you to select COM port A address.

The options are: 3F8, IRQ4 (Default); 2F8, IRQ3; 3E8, IRQ4; 2E8, IRQ3; Disabled; Auto.

COM port B Mode

Allows you to set COM port B interface.

The options are: Standard (Default), IrDA, ASKIR, MIDI.

COM port B Address

Allows you to select COM port B address.

The options are: 2F8, IRQ3 (Default); 3E8, IRQ4; 2E8, IRQ3; Auto; Disabled; 3F8, IRQ4.

LPT Port

Allows you to select LPT port address.

The options are: 378, IRQ7 (Default); 278, IRQ7; Auto; Disabled.

LPT Mode

Allows you to connect with an advanced printer I/O mode.

The options are: Output Only, Bi-Directional (Default), ECP.

LPT ECP Mode DMA

Allows you to select the DMA channel number 3 or 1 for the ECP printer mode.

The options are: NONE (default), 1, 3.

Diskette controller

When enabled, allows you to activate the floppy disk controller (FDC).

The options are: Enabled (Default), Disabled.

Primary IDE Controller

When set at Enable, the primary IDE device will have the I/O address 1F0h-1F7h.

The options are: Enabled (Default), Disabled.

Secondary IDE controller

When set at Enable, the secondary IDE device will be assigned the I/O address at 170h-177h.

The options are:  Enabled (Default), Disabled.

Audio Emulation Mode

When set at SB-16, it enables the integrated SoundBlaster 16 emulation.

The options are:  SB-16 (Default), Disabled.

Audio Base Address

Allows you to select the base I/O address for audio emulation.

The options are:  220h (Default), 240h.

Audio IRQ

Allows you to select the IRQ for audio emulation.

The options are: 5 (Default),  7, 10, Disable.

Audio Low DMA Channel

Allows you to select the DMA channel for 8-bit audio.

The options are: 1 (Default), 3, Disabled, 0.

Audio High DMA Channel

Allows you to select the DMA channel for 16-bit audio.

The options are: 5 (Default), 6, 7, Disabled.

MPU I/O Region

Allows you to select I/O region for MPU

The options are:  330h (Default), 300h.

Integrated VGA

When set at Auto, the system BIOS will automatically detect an external VGA card.

The options are: Auto (Default), Disabled.

Video Resolution

Allows you to select the video resolution. Low: 60x480x8, 800x600x8, 1024x768x8. Medium: all resolutions supported by Low, 640x480x16, 800x600x16. High: all resolutions supported by Medium, 1280x1024x8, 1024x768x16.

The options are:  HIGH (Default), LOW, MEDIUM.

TV output mode

Allows you to select the TV output type.

The options are: PAL (Default), NTSC.

### PCI Devices Configuration

Enable Master

When enabled, the selected device as a PCI bus master.

The options are: Disabled, Enabled (Default).

Default Latency Timer

When set at Yes, the system BIOS will use the minimum latency required by the device.

The options are: Yes (Default), No

Latency Timer

Allows you to select the minimum guranteed time slice which allotted for the bus master in the units of PCI bus clocks.

The default option is 0040 (Default).

## Power Management Setup

Power Management State

When set Enable, the system starts the power-saving mode.

The options are: Disabled (Default), Enable.

Standby Timeout

Allows you to select th eamout of time that the system needs to be idle before enteing the power-saving mode.

The options are: OFF (Default),  1, 2, 5, 10, 20, 30min.

## Boot Configuration

This menu allows you to decide the priority of the boot drive by UP and DOWN arrow keys.

## Exit Setup

Save Changes & Exit

Exit after writing all changed SETUP item values to CMOS.

Exit Without Saving Changes

Exit without saving changed SETUP item values.

Get Default Values

Load Default values for all SETUP items.

Load Previous Values

Read previous values from CMOS for all SETUP items.

Save Changes

Write all SETUP item values to CMOS.

— This page intentionally left blank —