# Appendix C

Troubleshooting

Your system has passed through a series of rigorous quality assurance tests to guarantee reliable performance.  However, your new computer is a complicated piece of equipment and as such may malfunction if used incorrectly or one of its components fails.  This appendix attempts to anticipate potential problems that may crop up during the day-to-day use of your computer.  Included are important tips and information you will need to help locate and solve problems you encounter.

In general, troubleshooting involves an organized system of approach to problem solving.  You should try to isolate the problem and identify the defective device (hardware) or improper setting (software). When you encounter a problem, the first thing you should do is perform a thorough visual inspection of the computer.  If none of the indicators are lit and you cannot hear the fan, then the computer is probably not receiving power.  Make sure the power cord is plugged in.  If you are using a power strip or surge protector, ensure that these devices are turned on.

A problem can be caused by an improperly connected cable.  Make sure all peripherals such as the mouse and keyboard are properly connected to their respective ports.  Ensure that none of the connectors’ pins are bent or broken.  Check all cables connected to your computer.  If any are cut, frayed, or damaged in any way, replace them right away.  Never use a damaged cable.  A damaged cable is not only a fire hazard, it may also cause a short circuit, resulting in irreparable damage to your computer.

The examples that follow provide useful tips and information that will help you isolate and solve some of the more common problems you may encounter.  These examples are presented in a question and answer format. If the problem persists after trying the suggested solutions, contact your dealer or a qualified service technician.

### Keyboard Error

| Question: | When I turn on the computer, a keyboard error message is displayed and they keyboard does not work. Is the keyboard broken? |
| --- | --- |
| Answer: | There is probably nothing wrong with the keyboard. It may not be properly connected to the keyboard port. Turn off the computer and check the keyboard connection. Ensure that the keyboard connector is firmly seated in the keyboard port. |

### The Mouse Does Not Work

| Question: | The mouse does not work; is it broken? |
| --- | --- |
| Answer: | It is doubtful that your mouse is broken. It just may not be connected or configured properly. First, turn off the computer and check the mouse connection. Make sure the mouse connector is firmly seated in the mouse port. If the problem persists, your mouse driver may not be installed or properly configured. The mouse requires a device driver to be installed and loaded during bootup. Two separate device drivers are required for operation under DOS and the Windows environment. |

### The Screen is Blank

| Question: | I know the computer is on but the screen is blank. Is the monitor broken? |
| --- | --- |
| Answer: | Probably not. Most likely the contrast or brightness controls are improperly adjusted. Try turning up the brightness. If after trying this, the monitor is still blank, try rebooting the computer by pressing the Crtl + Alt + Delete keys. If this does not solve the problem, turn the computer off, wait five seconds, and then turn it back on. If the problem persists after trying all of these suggestions, turn off the computer and consult your dealer or a qualified service technician. |

### The Monitor Displays Abnormal Patterns

| Question: | I changed the VGA device driver under Windows. The monitor is now displaying abnormal patterns. What did I do wrong? |
| --- | --- |
| Answer: | You have installed a video device driver that is not supported by your computer’s video subsystem. More than likely the driver your are trying to install requires more video RAM than is currently installed on the computer’s graphics accelerator card. Most of the device drivers that support true color (16.7 million colors) require at least 2MB of video RAM. Video modes that support high color (32K or 64K colors) at resolutions of 800 x 500 or above also require 2MB of Video RAM. Consult your dealer for information on upgrading your computer’s video RAM. |

— This page intentionally left blank —