Appendix F

If a recommended feature is implemented, it must meet the PC 98 requirements for that feature as defined in this document.

PC 98 Master Checklist

# Checklist for Basic PC 98

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

1.  System performance meets PC 98 minimum requirements

| Required | Required | Required |
| --- | --- | --- |

2.  System design meets ACPI 1.0 specification and PC 98 requirements

| Required | Required | Required |
| --- | --- | --- |

3.  Hardware design supports OnNow initiative

| Required | Required | Required |
| --- | --- | --- |

4.  BIOS meets PC 98 requirements for OnNow support

| Required | Required | Required |
| --- | --- | --- |

5.  BIOS meets PC 98 requirements for boot support

| Required | Required | Required |
| --- | --- | --- |

6.  All expansion slots in the system are accessible for users to insert cards

| Required | Required | Required |
| --- | --- | --- |

7.  Audible noise meets PC 98 requirements

| Required | Required | Required |
| --- | --- | --- |

8.  System and component design practices follow accessibility guidelines

| Recommended | Recommended | Recommended |
| --- | --- | --- |

9.  Internal system modification capabilities are not accessible to end users

| Recommended | Recommended | Recommended |
| --- | --- | --- |

10.  System design provides physical security

| Recommended | Recommended | Recommended |
| --- | --- | --- |

11.  Each device and driver meets PC 98 device requirements

| Required | Required | Required |
| --- | --- | --- |

12.  Each bus and device meets Plug and Play specifications

| Required | Required | Required |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

13.  Unique Plug and Play device ID provided for each system device and add-on device

| Required | Required | Required |
| --- | --- | --- |

14.  Option ROMs meet Plug and Play requirements

| Required | Required | Required |
| --- | --- | --- |

15.  “PNP” vendor code used only to define a legacy device’s CompatibleID

| Required | Required | Required |
| --- | --- | --- |

16.  Device driver and installation meet PC 98 requirements

| Required | Required | Required |
| --- | --- | --- |

17.  Minimal user interaction needed to install and configure devices

| Required | Required | Required |
| --- | --- | --- |

18.  Connections use icons plus keyed or shrouded connectors

| Required | Required | Required |
| --- | --- | --- |

19.  Hot-plugging capabilities for buses and devices meet PC 98 requirements

| Required | Required | Required |
| --- | --- | --- |

20.  Device Bay-capable bay and peripherals meet Device Bay specification

| Required | Required | Required |
| --- | --- | --- |

21.  Multifunction add-on devices meet PC 98 device requirements for each device

| Required | Required | Required |
| --- | --- | --- |

22.  All devices support correct 16-bit decoding for I/O port addresses

| Required | Required | Required |
| --- | --- | --- |

23.  System-board devices use ISA-compatible addresses

| Required | Required | Required |
| --- | --- | --- |

24.  Each bus meets written specifications and PC 98 requirements

| Required | Required | Required |
| --- | --- | --- |

25.  System includes USB with one USB port, minimum

| Required | Required | Required, with 2 USB ports |
| --- | --- | --- |

26.  System includes support for other high-speed expansion capabilities

| Recommended | Recommended | Required, with 2 |
| --- | --- | --- |
|  |  | IEEE 1394 ports |

27.  If present, PCI bus meets PCI 2.1 or higher, plus PC 98 requirements

| Required | Required | Required |
| --- | --- | --- |

28.  System does not include ISA expansion devices

| Required | Required | Required, with no ISA slots |
| --- | --- | --- |

29.  System includes keyboard connection and keyboard

| Required | Required | Required; USB or wireless |
| --- | --- | --- |

30.  System includes pointing-device connection and pointing device

| Required | Required | Required; USB or wireless |
| --- | --- | --- |

31.  System includes connection for external parallel devices

| Required | Required | Required |
| --- | --- | --- |

32.  System includes connection for external serial devices

| Required | Required | Required |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

33.  System includes wireless capabilities

| Recommended | Recommended | Recommended |
| --- | --- | --- |

34.  System includes support for operating system installation

| Required | Required | Required |
| --- | --- | --- |

35.  System audio support meets PC 98 requirements

| Recommended | Recommended | Required |
| --- | --- | --- |

36.  System includes communications device

| Required; modem | Required; network adapter | Required; modem |
| --- | --- | --- |

37.  System includes smart card support

| Not applicable | Recommended | Not applicable |
| --- | --- | --- |

38.  Graphics adapter meets PC 98 minimum requirements

| Required | Required | Required |
| --- | --- | --- |

39.  Adapter supports television output if system does not include a large-screen monitor

| Recommended | Recommended | Recommended |
| --- | --- | --- |

40.  Color monitor is DDC-compliant with unique EDID identifier

| Required | Required | Required |
| --- | --- | --- |

41.  System meets PC 98 DVD-Video and MPEG-2 playback requirements

| Required | Required with DVD-Video | Required |
| --- | --- | --- |

42.  System supports PC 98 analog video input and capture capabilities

| Recommended | Recommended | Required |
| --- | --- | --- |

43.  System includes analog television tuner

| Recommended | Recommended | Required |
| --- | --- | --- |

44.  System BIOS and option ROMs support Int 13h Extensions

| Required | Required | Required |
| --- | --- | --- |

45.  Host controller for storage device meets PC 98 requirements

| Required | Required | Required |
| --- | --- | --- |

46.  Host controllers and devices support bus mastering

| Required | Required | Required |
| --- | --- | --- |

47.  Hard drive meets PC 98 requirements

| Required | Required | Required |
| --- | --- | --- |

48.  Removable media support media status notification

| Required | Required | Required |
| --- | --- | --- |

49.  Floppy disk capabilities are provided using an expansion card or external bus

| Recommended | Recommended | Recommended |
| --- | --- | --- |

50.  System supports WHIIG

| Not applicable | Required | Not applicable |
| --- | --- | --- |

51.  System includes driver support for WMI

| Not applicable | Required | Not applicable |
| --- | --- | --- |

52.  Management information service provider enabled by default

| Not applicable | Required | Not applicable |
| --- | --- | --- |

53.  Expansion devices can be remotely managed

| Not applicable | Required | Not applicable |
| --- | --- | --- |

# Checklist for Workstation PC 98

1.  Workstation meets all requirements for Office PC 98

| Required |  |  |
| --- | --- | --- |

2.  Workstation meets requirements for optimal performance

| Required |  |  |
| --- | --- | --- |

3.  Workstation supports multiple processors

| Recommended |  |  |
| --- | --- | --- |

4.  Workstation RAM can be expanded

| Recommended |  |  |
| --- | --- | --- |

5 Workstation system memory includes ECC memory protection

| Required |  |  |
| --- | --- | --- |

6.  Workstation includes APIC support

| Required |  |  |
| --- | --- | --- |

7.  Workstation includes high-performance components

| Recommended |  |  |
| --- | --- | --- |

8.  Workstation supports 64-bit I/O bus architecture

| Recommended |  |  |
| --- | --- | --- |

9.  Workstation does not include ISA expansion slots

| Recommended |  |  |
| --- | --- | --- |

10.  Graphics subsystem supports workstation performance demands

| Required |  |  |
| --- | --- | --- |

11.  Workstation meets PC 98 DVD-Video and MPEG-2 playback requirements

| Required, with DVD-Video |  |  |
| --- | --- | --- |

12.  Storage components rely on SCSI controller

| Recommended |  |  |
| --- | --- | --- |

13.  Workstation includes multiple hard drives

| Recommended |  |  |
| --- | --- | --- |

# Checklist for Entertainment PC 98

1.  Entertainment PC 98 system includes two USB ports, with at least one easily accessible connector

| Required |  |  |
| --- | --- | --- |

2.  Entertainment PC 98 system includes two IEEE 1394 ports, with at least one easily accessible connector

| Required |  |  |
| --- | --- | --- |

3.  Entertainment PC 98 system does not include ISA expansion slots

| Required |  |  |
| --- | --- | --- |

4.  All Entertainment PC 98 input devices meet USB HID specifications

| Required |  |  |
| --- | --- | --- |

5.  Entertainment PC 98 includes remote-control pointing device

| Recommended |  |  |
| --- | --- | --- |

6.  All Entertainment PC 98 pointing devices support DirectInput and work simultaneously

| Required |  |  |
| --- | --- | --- |

7.  Entertainment PC 98 includes data/fax/voice modem that supports V.pcm

| Required |  |  |
| --- | --- | --- |

8.  Entertainment PC 98 includes DVD-ROM drive and DVD-Video playback

| Required |  |  |
| --- | --- | --- |

9.  Audio support meets PC 98 audio and Entertainment PC 98 requirements

| Required |  |  |
| --- | --- | --- |

10.  Graphics adapter uses AGP

| Required |  |  |
| --- | --- | --- |

11.  Entertainment PC 98 graphics subsystem includes PC 98 hardware acceleration for 2-D and 3-D graphics

| Required |  |  |
| --- | --- | --- |

12.  Entertainment PC 98 graphics subsystem includes support for television output if the system doesn’t have a large-screen monitor

| Recommended |  |  |
| --- | --- | --- |

13.  Entertainment PC 98 includes large-screen DDC2B color entertainment monitor

| Recommended |  |  |
| --- | --- | --- |

14.  Entertainment PC 98 DVD-Video and MPEG-2 playback meet PC 98 requirements

| Required |  |  |
| --- | --- | --- |

15.  Entertainment PC 98 supports PC 98 analog video input and capture capabilities

| Required |  |  |
| --- | --- | --- |

16.  Entertainment PC 98 includes analog television tuner

| Required |  |  |
| --- | --- | --- |

17.  Entertainment PC 98 includes digital broadcast satellite subsystem

| Recommended |  |  |
| --- | --- | --- |

18.  Entertainment PC 98 includes DTV support

| Required |  |  |
| --- | --- | --- |

# Checklist for Mobile PC 98

1.  Mobile PC performance meets PC 98 minimum requirements

| Required |  |  |
| --- | --- | --- |

2.  Mobile PC supports Smart Battery or ACPI-specified battery

| Required |  |  |
| --- | --- | --- |

3.  Expansion capabilities of mobile PC are accessible to users

| Required |  |  |
| --- | --- | --- |

4.  Mobile PC connections use icons plus keyed or shrouded connectors

| Required |  |  |
| --- | --- | --- |

5.  Mobile PC includes a USB port

| Required |  |  |
| --- | --- | --- |

6.  Mobile PC includes an IEEE 1394  port

| Recommended |  |  |
| --- | --- | --- |

7.  USB-connected device does not maintain fully on power state

| Required |  |  |
| --- | --- | --- |

8.  Mobile PC includes CardBus

| Required |  |  |
| --- | --- | --- |

9.  Mobile PC keyboard and pointing device meet PC 98 requirements

| Required |  |  |
| --- | --- | --- |

10.  Mobile PC includes wireless capabilities

| Recommended |  |  |
| --- | --- | --- |

11.  Mobile PC includes support for installing the operating system

| Required |  |  |
| --- | --- | --- |

12.  Mobile PC audio meets PC 98 audio requirements

| Recommended |  |  |
| --- | --- | --- |

13.  Mobile PC includes communications device

| Recommended |  |  |
| --- | --- | --- |

14.  Built-in display adapter meets PC 98 minimum requirements

| Required |  |  |
| --- | --- | --- |

15.  Mobile system supports hot pluggable devices and alternative server connections

| Recommended |  |  |
| --- | --- | --- |

16.  Mobile PC/docking station combination meets PC 98 requirements

| Required |  |  |
| --- | --- | --- |

17.  Docking station meets all Basic PC 98 requirements

| Required |  |  |
| --- | --- | --- |

18.  Docking station interface is supported using ACPI-defined mechanisms

| Required |  |  |
| --- | --- | --- |

19.  Mobile PC/docking station combination supports automatic resource assignment and dynamic disable capabilities

| Required |  |  |
| --- | --- | --- |

20.  Docking station supports warm docking

| Required |  |  |
| --- | --- | --- |

21.  Docking system supports fail-safe docking

| Required |  |  |
| --- | --- | --- |

22.  Port replicator supports automatic resource assignment and dynamic disable capabilities for replacement devices

| Required |  |  |
| --- | --- | --- |

23.  Port replicator supports warm docking

| Required |  |  |
| --- | --- | --- |

24.  Mini-notebook performance meets PC 98 minimum requirements

| Required |  |  |
| --- | --- | --- |

# Checklist for USB

1.  USB included on PC 98 system

| Required |  |  |
| --- | --- | --- |

2.  All USB hardware complies with USB 1.0 specifications

| Required |  |  |
| --- | --- | --- |

3.  Connections use USB icon

| Required |  |  |
| --- | --- | --- |

4.  Devices and drivers support maximum flexibility of hardware interface options

| Recommended |  |  |
| --- | --- | --- |

5.  USB host controller meets either OpenHCI or UHCI specification

| Required |  |  |
| --- | --- | --- |

6.  USB host controller can wake the system

| Required |  |  |
| --- | --- | --- |

7.  System and devices comply with USB power management requirements

| Required |  |  |
| --- | --- | --- |

8.  USB devices meet requirements in related USB device class specification

| Required |  |  |
| --- | --- | --- |

# Checklist for IEEE 1394

1.  Controllers and devices support IEEE 1394-1995 standards

| Required |  |  |
| --- | --- | --- |

2.  Controllers comply with OpenHCI for IEEE 1394

| Required |  |  |
| --- | --- | --- |

3.  OpenHCI controllers and devices support advances defined in IEEE 1394A

| Required |  |  |
| --- | --- | --- |

4.  Host supports peak data rate of 400 Mb/s, minimum

| Required |  |  |
| --- | --- | --- |

5.  Design avoids excessive currents resulting from ground-fault potential among devices

| Recommended |  |  |
| --- | --- | --- |

6.  Device command protocols conform to standard device class interfaces

| Required |  |  |
| --- | --- | --- |

7.  Devices support peak data rate of 400 Mb/s, minimum

| Recommended |  |  |
| --- | --- | --- |

8.  Devices requiring support for high-bandwidth data transfer use IEEE 1394

| Recommended |  |  |
| --- | --- | --- |

9.  Plug and Play devices demonstrate interoperability with other devices

| Required |  |  |
| --- | --- | --- |

10.  Topology faults do not cause the bus to fail

| Required |  |  |
| --- | --- | --- |

11.  Removable media devices support media status notification

| Required |  |  |
| --- | --- | --- |

12.  Devices that can initiate peer-to-peer communications also support remote programming

| Required |  |  |
| --- | --- | --- |

13.  Device provides a configuration ROM for unique device identification

| Required |  |  |
| --- | --- | --- |

14.  Device configuration ROM implements general ROM format

| Required |  |  |
| --- | --- | --- |

15.  Bus information block implemented at a base address offset of 0404h

| Required |  |  |
| --- | --- | --- |

16.  Configuration ROM provides globally unique device ID

| Required |  |  |
| --- | --- | --- |

17.  Root directory is located at a fixed address following the bus information block

| Required |  |  |
| --- | --- | --- |

18.  Configuration ROM includes a unit directory for each independent device function

| Required |  |  |
| --- | --- | --- |

19.  Each unit directory provides a valid Unit_Spec_Id and Unit_Sw_Version

| Required |  |  |
| --- | --- | --- |

20.  Each unit directory provides a pointer to a unit-dependent directory

| Required |  |  |
| --- | --- | --- |

21.  Vendor and model leaves support textual descriptor leaf format

| Required |  |  |
| --- | --- | --- |

22.  Unit-dependent directory provides a pointer to the unit’s CSRs

| Required |  |  |
| --- | --- | --- |

23.  Device provides three connector ports

| Recommended |  |  |
| --- | --- | --- |

24.  Device uses standard 6-pin IEEE 1394 connector

| Required |  |  |
| --- | --- | --- |

25.  Self-powered devices propagate the power bus through each connector

| Required |  |  |
| --- | --- | --- |

26.  Only single-port leaf-node devices use 4-pin connectors

| Required |  |  |
| --- | --- | --- |

27.  Device connectors exhibit common speed and power characteristics

| Required |  |  |
| --- | --- | --- |

28.  Standard 400-Mb/s rated IEEE 1394 cable provided with devices

| Required |  |  |
| --- | --- | --- |

29.  Devices power their PHY at all times

| Required |  |  |
| --- | --- | --- |

30.  Devices report power source and cable power consumption in Self_id packet

| Required |  |  |
| --- | --- | --- |

31.  Devices implement link power control

| Required |  |  |
| --- | --- | --- |

32.  Device requiring power increments in excess of Link_on implements unit-power CSRs

| Required |  |  |
| --- | --- | --- |

33.  Devices that source cable power must report this capability

| Required |  |  |
| --- | --- | --- |

34.  IEEE 1394-enabled PC sources cable power

| Required |  |  |
| --- | --- | --- |

35.  Power source supplies a minimum of 20 volts at 15 watts

| Recommended |  |  |
| --- | --- | --- |

36.  Devices notify the power manager of power change requests

| Required |  |  |
| --- | --- | --- |

37.  Devices and controllers comply with Cable Power Distribution specification

| Required |  |  |
| --- | --- | --- |

38.  Devices and controllers comply with IEEE 1394 power specification

| Required |  |  |
| --- | --- | --- |

# Checklist for PCI

1.  All components comply with PCI 2.1

| Required |  |  |
| --- | --- | --- |

2.  System does not contain ghost cards

| Required |  |  |
| --- | --- | --- |

3.  System uses standard method to close BAR windows on nonsubtractive decode PCI bridges

| Required |  |  |
| --- | --- | --- |

4.  System supports PCI docking through a bridge connector

| Recommended |  |  |
| --- | --- | --- |

5.  PCI chip sets support Ultra DMA/33

| Required |  |  |
| --- | --- | --- |

6.  System-board bus complies with PCI 2.1

| Required |  |  |
| --- | --- | --- |

7.  Bus master privileges are supported for all connectors

| Required |  |  |
| --- | --- | --- |

8.  ISA Write Data Port address is propagated to the ISA bus at power up

| Required |  |  |
| --- | --- | --- |

9.  Functions in a multifunction PCI device do not share writable PCI Configuration Space bits

| Required |  |  |
| --- | --- | --- |

10.  Devices use PCI 2.1 Configuration Space register for Plug and Play device ID

| Required |  |  |
| --- | --- | --- |

11.  Device IDs include PCI 2.1 Subsystem IDs

| Required |  |  |
| --- | --- | --- |

12.  Configuration Space is correctly populated

| Required |  |  |
| --- | --- | --- |

13.  Interrupt routing supported using ACPI

| Required |  |  |
| --- | --- | --- |

14.  BIOS does not configure I/O systems to share PCI interrupts

| Recommended |  |  |
| --- | --- | --- |

15.  BIOS configures boot device IRQ and writes to the interrupt line register

| Required |  |  |
| --- | --- | --- |

16.  Hot swapping for any PCI device uses ACPI-based methods

| Required |  |  |
| --- | --- | --- |

17.  All PCI components comply with PCI Bus Power Management Interface specification

| Required |  |  |
| --- | --- | --- |

# Checklist for IDE and ATAPI

1.  Controller complies with ATA-2 specification

| Required |  |  |
| --- | --- | --- |

2.  Bootable IDE controller supports El Torito No Emulation mode

| Required |  |  |
| --- | --- | --- |

3.  System BIOS and option ROMs support Int 13h Extensions

| Required |  |  |
| --- | --- | --- |

4.  Controller and peripherals support media status notification

| Required |  |  |
| --- | --- | --- |

5.  Dual IDE adapters use single FIFO with asynchronous access or dual FIFOs and channels

| Required |  |  |
| --- | --- | --- |

6.  System BIOS and devices support LBA

| Required |  |  |
| --- | --- | --- |

7.  Controller and peripherals support PCI IDE bus mastering

| Required |  |  |
| --- | --- | --- |

8.  Controller and peripheral connections include Pin 1 cable designation with keyed and shrouded connectors

| Required |  |  |
| --- | --- | --- |

9.  Peripherals comply with SFF 8020i, Version 2.5 or higher

| Required |  |  |
| --- | --- | --- |

10.  BIOS enumeration of all ATAPI devices complies with SFF 8020i, Version 2.5 or higher

| Required |  |  |
| --- | --- | --- |

11.  Devices support ATAPI RESET command

| Recommended |  |  |
| --- | --- | --- |

12.  IDE/ATAPI controllers and devices support Ultra DMA/33

| Required |  |  |
| --- | --- | --- |

13.  Operating system recognizes the boot drive in a multiple-drive system

| Required |  |  |
| --- | --- | --- |

14.  Each device has a Plug and Play device ID

| Required |  |  |
| --- | --- | --- |

15.  Dynamic resource configuration supported for all devices

| Required |  |  |
| --- | --- | --- |

16.  Resource configuration meets bus requirements

| Required |  |  |
| --- | --- | --- |

17.  ISA address ranges 3F7h and 377h are not claimed by IDE controllers

| Required |  |  |
| --- | --- | --- |

18.  Device supports ATA STANDBY command

| Required |  |  |
| --- | --- | --- |

19.  Bus and device meet PC 98 power management requirements

| Required |  |  |
| --- | --- | --- |

# Checklist for SCSI

1. Host controller supports bus mastering

| Required |  |  |
| --- | --- | --- |

2. Option ROMs support Int 13h Extensions

| Required |  |  |
| --- | --- | --- |

3. Option ROMs support virtual DMA services

| Required |  |  |
| --- | --- | --- |

4. Bus type is clearly indicated on connectors for all adapters, peripherals, and terminators

| Required |  |  |
| --- | --- | --- |

5. Differential devices support DIFFSENS as defined in SCSI-3

| Required |  |  |
| --- | --- | --- |

6. Automatic termination circuit meets SCSI-3 specification

| Required |  |  |
| --- | --- | --- |

7. SCSI terminator built onto internal cables meets SCSI-3 specification

| Required |  |  |
| --- | --- | --- |

8. Terminator power is supplied to the SCSI bus, with over-current protection

| Required |  |  |
| --- | --- | --- |

9. High-density external connector meets SCSI-2 specification

| Required |  |  |
| --- | --- | --- |

10. Internal terminator is close as possible to the last peripheral on the cable

| Recommended |  |  |
| --- | --- | --- |

11. SCSI bus parity signal meets SCSI-2 specification

| Required |  |  |
| --- | --- | --- |

12. Cables meet SCSI-3 Clause 6 requirements

| Required |  |  |
| --- | --- | --- |

13. User cannot incorrectly plug in cables for internal connections

| Required |  |  |
| --- | --- | --- |

14. Internal SCSI peripherals do not terminate the SCSI bus

| Recommended |  |  |
| --- | --- | --- |

15. External connectors use automatic termination or an accessible on-board termination switch

| Required |  |  |
| --- | --- | --- |

16. High-density, shielded device connector meets SCSI-2 specification

| Recommended |  |  |
| --- | --- | --- |

17. Removable media includes media status notification support

| Recommended |  |  |
| --- | --- | --- |

18. All components comply with Plug and Play SCSI specifications

| Required |  |  |
| --- | --- | --- |

19. Each device has a Plug and Play device identifier

| Required |  |  |
| --- | --- | --- |

20. Automatic resource assignment and dynamic disable capabilities are supported for all devices

| Required |  |  |
| --- | --- | --- |

21. Bus and device meet PC 98 power requirements

| Required |  |  |
| --- | --- | --- |

22. Hardware supports the STOP/START UNIT command as defined in SCSI-2

| Required |  |  |
| --- | --- | --- |

23. STOP/START UNIT command can be used to decrease power consumption

| Recommended |  |  |
| --- | --- | --- |

# Checklist for PC Card

1.  All devices comply with the PC Card standards

| Required |  |  |
| --- | --- | --- |

2.  System and ZV-compatible PC Card 16 cards comply with ZV standard definitions

| Required |  |  |
| --- | --- | --- |

3.  Controller supports industry-standard ExCA register set

| Required |  |  |
| --- | --- | --- |

4.  System maintains mapping of IRQ Routing Register bits to system interrupt vectors

| Required |  |  |
| --- | --- | --- |

5.  IRQ connections can be determined by using the 0805 register

| Required |  |  |
| --- | --- | --- |

6.  CardBus controllers support both ISA and PCI interrupts

| Required |  |  |
| --- | --- | --- |

7.  System supports industry-standard definition for CardBus bridges

| Required |  |  |
| --- | --- | --- |

8.  BIOS initializes CardBus controller in 82365-compatible mode and supports backward compatibility

| Recommended |  |  |
| --- | --- | --- |

9.  CardBus controllers do not share writable PCI Configuration Space bits

| Required |  |  |
| --- | --- | --- |

10.  Each PC Card 16 memory window in CardBus controller has it own page register

| Required |  |  |
| --- | --- | --- |

11.  Card supports required I/O card tuples

| Required |  |  |
| --- | --- | --- |

12.  Configuration table entry tuples listed in priority order

| Required |  |  |
| --- | --- | --- |

13.  Card specifies maximum configuration options

| Required |  |  |
| --- | --- | --- |

14.  Configuration space meets Common Silicon Guidelines

| Required |  |  |
| --- | --- | --- |

15.  RESERVED fields comply with PCI 2.1

| Required |  |  |
| --- | --- | --- |

16.  CardBus card implements required and recommended tuples

| Required |  |  |
| --- | --- | --- |

17.  Socket controller complies with device class power management reference specification

| Required |  |  |
| --- | --- | --- |

18.  PC Card 16 cards implement power-related events using ReqAttn bit and #STSCHG mechanism

| Required |  |  |
| --- | --- | --- |

19.  CardBus controllers and cards implement power management specifications

| Required |  |  |
| --- | --- | --- |

20.  No user intervention required for correctly installing devices

| Required |  |  |
| --- | --- | --- |

21.  Device is immediately functional without restarting the system

| Required |  |  |
| --- | --- | --- |

22.  ZV-compatible PC Card driver uses DirectDraw LVE

| Required |  |  |
| --- | --- | --- |

23.  PC Card 16 card driver supports sharing of level-mode interrupts

| Required |  |  |
| --- | --- | --- |

# Checklist for I/O Ports and Devices

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

1.  System includes connection for external serial devices

| Required | Required | Required |
| --- | --- | --- |

2.  System includes connection for external parallel devices

| Required | Required | Required |
| --- | --- | --- |

3.  System includes external connection for keyboard

| Required | Required | Required |
| --- | --- | --- |

4.  System includes external connection for pointing device

| Required | Required | Required |
| --- | --- | --- |

5.  System includes USB game pad or joystick

| Recommended | Recommended | Recommended; wireless |
| --- | --- | --- |

6.  System includes built-in wireless capabilities

| Recommended | Recommended | Recommended |
| --- | --- | --- |

7.  Devices use USB or external bus connections rather than legacy serial or parallel ports

| Required | Recommended | Required |
| --- | --- | --- |

8.  All devices meet PC 98 general device requirements

| Required | Required | Required |
| --- | --- | --- |

9.  Serial port meets device class specifications for its bus

| Required |  |  |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

10.  Legacy serial port is implemented as 16550A UART or equivalent and supports 115.2K baud

| Required |  |  |
| --- | --- | --- |

11.  Legacy serial port supports flexible resource configuration and dynamic disable capabilities

| Required |  |  |
| --- | --- | --- |

12.  Conflict resolution for legacy serial port ensures availability of at least one serial port

| Required |  |  |
| --- | --- | --- |

13.  Parallel port meets device class specifications for its bus

| Required |  |  |
| --- | --- | --- |

14.  Flexible resource configuration supported for each parallel port

| Required |  |  |
| --- | --- | --- |

15.  EPP support does not use restricted I/O addresses

| Required |  |  |
| --- | --- | --- |

16.  Compatibility, nibble mode, and ECP protocols meet IEEE 1284-1994 specifications

| Required |  |  |
| --- | --- | --- |

17.  Port connectors meet IEEE 1284-I specifications, minimum

| Required |  |  |
| --- | --- | --- |

18.  IEEE 1284 peripherals have Plug and Play device IDs

| Required |  |  |
| --- | --- | --- |

19.  Device identification string provides CompatibleID key

| Recommended |  |  |
| --- | --- | --- |

20.  Pointing-device connection meets requirements for its bus class

| Required |  |  |
| --- | --- | --- |

21.  Remote control provides PC 98 minimum support

| Recommended |  |  |
| --- | --- | --- |

22.  Keyboard connection meets requirements for its bus class

| Required |  |  |
| --- | --- | --- |

23.  No interference occurs between multiple keyboards

| Required |  |  |
| --- | --- | --- |

24.  Keyboard includes Windows and Application logo keys

| Recommended |  |  |
| --- | --- | --- |

25.  Device meets USB HID class specification requirements

| Required |  |  |
| --- | --- | --- |

26.  IR device uses NDIS 5.0 miniport driver

| Required |  |  |
| --- | --- | --- |

27.  IR device meets IrDA specifications

| Required |  |  |
| --- | --- | --- |

28.  IR device meets IrDA Control IR specification

| Required |  |  |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

29.  IR device meets PC 98 bus and port specifications

| Required |  |  |
| --- | --- | --- |

30.  IR device meets USB guidelines for interfacing with IrDA and Control IR devices

| Required |  |  |
| --- | --- | --- |

31.  IR device supports flexible resource configuration and dynamic disable capabilities

| Required |  |  |
| --- | --- | --- |

32.  System supports standard input speeds of 4 Mb/s

| Recommended |  |  |
| --- | --- | --- |

33.  System differentiates command streams if transceiver includes legacy consumer IR support

| Required |  |  |
| --- | --- | --- |

34.  Each device has a unique Plug and Play device ID

| Required |  |  |
| --- | --- | --- |

35.  Automatic resource assignment and dynamic disable capabilities are supported

| Required |  |  |
| --- | --- | --- |

36.  Each device complies with its device class power management reference specification

| Required |  |  |
| --- | --- | --- |

37.  Device supports wake-up events

| Required for wireless input; |  |  |
| --- | --- | --- |
| optional for other devices |  |  |

38.  Device drivers and installation meet PC 98 requirements

| Required |  |  |
| --- | --- | --- |

# Checklist for Graphics Adapters

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

1.  Graphics adapter uses PCI, AGP, or another high-speed bus

| Required | Required | AGP required |
| --- | --- | --- |

2.  System uses WC with higher-performance processors

| Required | Required | Required |
| --- | --- | --- |

3.  Primary graphics adapter works normally with default VGA mode driver

| Required | Required | Required |
| --- | --- | --- |

4.  Adapter and driver support multiple adapters and multiple monitors

| Required | Required | Required |
| --- | --- | --- |

5.  Adapter supports television output if system does not include large-screen monitor

| Recommended | Recommended | Required |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

6.  Adapter meets PC 98 general device requirements

| Required |  |  |
| --- | --- | --- |

7.  Screen resolution and local memory capacity meet PC 98 minimum requirements

| Required | Required | Required |
| --- | --- | --- |

8.  Adapter meets VESA specifications for ergonomic timing rates

| Required |  |  |
| --- | --- | --- |

9.  All supported color depths are enumerated

| Required |  |  |
| --- | --- | --- |

10.  Graphics operations use relocatable registers only

| Required |  |  |
| --- | --- | --- |

11.  Adapter supports downloadable RAMDAC entries for image color matching

| Required |  |  |
| --- | --- | --- |

12.  Adapter supports DDC monitor detection

| Required |  |  |
| --- | --- | --- |

13.  Adapter supports video overlay surface with scaling

| Required | Required | Required |
| --- | --- | --- |

14.  Hardware supports VGA destination color keying for video rectangle

| Required | Required with DVD-Video | Required |
| --- | --- | --- |

15.  Video port meets PC 98 specifications if present on graphics adapter

| Required | Required | Required |
| --- | --- | --- |

16.  Adapter supports MPEG-2 motion compensation acceleration

| Recommended |  |  |
| --- | --- | --- |

17.  Extended resources can be dynamically relocated after system boot

| Required |  |  |
| --- | --- | --- |

18.  VGA resources can be disabled by software

| Required |  |  |
| --- | --- | --- |

19.  Frame buffer can be accessed directly by applications

| Required | Required | Required |
| --- | --- | --- |

20.  Adapter and driver support linear-mapped, low-resolution modes

| Required | Required | Required |
| --- | --- | --- |

21.  Adapter supports transparent blter

| Required | Required | Required |
| --- | --- | --- |

22.  Hardware supports double buffering with no tearing

| Required | Required | Required |
| --- | --- | --- |

23.  Hardware supports programmable blter stride

| Required | Recommended | Required |
| --- | --- | --- |

24.  Hardware supports RGB rasterization

| Required | Required | Required |
| --- | --- | --- |

25.  Hardware supports multi-texturing

| Recommended | Recommended | Required |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

26.  Hardware supports texture formats

| Required | Required | Required |
| --- | --- | --- |

27.  Hardware complies with texture size limitations

| Required | Recommended | Required |
| --- | --- | --- |

28.  Hardware supports destination RGB alpha blending

| Recommended | Recommended | Recommended |
| --- | --- | --- |

29.  Hardware supports Z comparison modes and Direct3D-compatible formats

| Recommended | Recommended | Required |
| --- | --- | --- |

30.  Hardware meets PC 98 3-D accelerator performance requirements

| Recommended | Recommended | Required |
| --- | --- | --- |

31.  Adapter supports both NTSC and PAL output

| Recommended |  |  |
| --- | --- | --- |

32.  Default boot mode supports appropriate locale

| Required | Required | Required |
| --- | --- | --- |

33.  Adapter supports underscan scaling

| Required | Recommended | Required |
| --- | --- | --- |

34.  Adapter supports flicker filter

| Required |  |  |
| --- | --- | --- |

35.  Adapter provides proper termination

| Required |  |  |
| --- | --- | --- |

36.  Adapter supports RCA-style composite video and S-Video connectors

| Recommended | Recommended | Required |
| --- | --- | --- |

37.  Adapter supports both VGA and television output

| Required |  |  |
| --- | --- | --- |

38.  Software supports positioning

| Required | Recommended | Required |
| --- | --- | --- |

39.  Software supports detection of television connection

| Required | Recommended | Required |
| --- | --- | --- |

40.  Each device has a Plug and Play device ID

| Required |  |  |
| --- | --- | --- |

41.  System supports conflict resolution, VGA compatibility, and extended registers

| Required |  |  |
| --- | --- | --- |

42.  Chips support linear packed-pixel frame buffer, relocatable above 16 MB

| Required |  |  |
| --- | --- | --- |

43.  Option ROM supports DDC2B

| Required |  |  |
| --- | --- | --- |

44.  BIOS setup utility provides option to force use of system-board graphics

| Recommended |  |  |
| --- | --- | --- |

45.  BIOS supports large frame buffers for graphics adapters

| Required |  |  |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

46.  AGP meets PC 98 implementation guidelines

| Required |  |  |
| --- | --- | --- |

47.  PCI graphics device supports IRQ and correctly populates PCI BARs

| Required | Recommended | Required |
| --- | --- | --- |

48.  PCI system-board graphics device is not hidden from Plug and Play enumeration

| Required |  |  |
| --- | --- | --- |

49.  Graphics adapter complies with device class power management reference specification

| Required |  |  |
| --- | --- | --- |

50.  Graphics adapter complies with VBE/Core 2.0 extensions for power management

| Required |  |  |
| --- | --- | --- |

51.  Device drivers and installation meet PC 98 requirements

| Required |  |  |
| --- | --- | --- |

52.  Driver does not bypass any Microsoft-provided system components

| Required |  |  |
| --- | --- | --- |

53.  Applications provided with device meet Win32 requirements

| Required |  |  |
| --- | --- | --- |

54.  Driver supports dynamic color bit-depth change

| Required |  |  |
| --- | --- | --- |

# Checklist for Video and Broadcast Components

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

1.  System meets PC 98 requirements for DVD-Video and MPEG-2 playback

| Required | Required with DVD-Video | Required |
| --- | --- | --- |

2.  System supports PC 98 analog video input and capture capabilities

| Recommended | Recommended | Required |
| --- | --- | --- |

3.  System includes analog television tuner

| Recommended | Recommended | Required |
| --- | --- | --- |

4.  System includes digital broadcast or satellite subsystem

| Recommended | Recommended | Recommended |
| --- | --- | --- |

5.  System includes DTV support

| Recommended | Recommended | Required (U.S. only) |
| --- | --- | --- |

6.  Video input, capture, and broadcast device support is based on DirectX foundation class and WDM Stream class

| Required | Required | Required |
| --- | --- | --- |

7.  Hardware MPEG-2 decoder uses video port for video data

| Required | Required | Required |
| --- | --- | --- |

8.  PCI-based tuners and decoders support bus mastering with scatter/gather DMA

| Required |  |  |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

9.  Background tasks do not interfere with MPEG-2 playback

| Required | Recommended | Required |
| --- | --- | --- |

10.  All components meet PC 98 general device requirements

| Required |  |  |
| --- | --- | --- |

11.  System warns users if it cannot play DVD movies

| Required | Required if no DVD-Video | Required |
| --- | --- | --- |

12.  MPEG-2 playback meets PC 98 requirements

| Required | Required with DVD-Video | Required |
| --- | --- | --- |

13.  Retail adapters with hardware MPEG-2 decoders enable a standard video port connection to the graphics adapter

| Required |  |  |
| --- | --- | --- |

14.  MPEG-2 decoder supports pull-down algorithm

| Recommended |  |  |
| --- | --- | --- |

15.  DVD decoder driver correctly handles media types, time discontinuity, and decode-rate adjustment

| Required |  |  |
| --- | --- | --- |

16.  DVD decoder supports subpicture compositing and closed captioning

| Required |  |  |
| --- | --- | --- |

17.  Subpicture decoder correctly handles subpicture properties and other functions

| Required |  |  |
| --- | --- | --- |

18.  System supports seamless DVD-Video 1.0 navigation

| Required |  |  |
| --- | --- | --- |

19.  System provides a licensed CSS copyright protection scheme

| Required |  |  |
| --- | --- | --- |

20.  Video input or capture device supports capture of NTSC/PAL picture quality

| Required |  |  |
| --- | --- | --- |

21.  Analog video capture device outputs video data rate of 3.7 MB per second, minimum

| Required |  |  |
| --- | --- | --- |

22.  Video input or capture device supports time-code reading

| Recommended |  |  |
| --- | --- | --- |

23.  Digital video camera uses external bus support

| Required |  |  |
| --- | --- | --- |

24.  Television tuner supports PC 98 audio and video performance

| Required |  |  |
| --- | --- | --- |

25.  Television tuner includes stereo tuner and supports SAP

| Recommended | Recommended | Required |
| --- | --- | --- |

26.  VBI capture oversamples VBI data at least four times

| Required |  |  |
| --- | --- | --- |

27.  VBI capture detects validity of scan-line data

| Required |  |  |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

28.  VBI capture makes VBI data available to the CPU for processing

| Required |  |  |
| --- | --- | --- |

29.  Digital broadcast card can receive all video, audio, data, and other streams

| Required |  |  |
| --- | --- | --- |

30.  Digital broadcast card can receive full bandwidth from each frequency

| Required |  |  |
| --- | --- | --- |

31.  Digital broadcast card can receive a minimum of eight simultaneous streams

| Required |  |  |
| --- | --- | --- |

32.  System includes multiple digital broadcast tuner cards

| Recommended |  |  |
| --- | --- | --- |

33.  Digital broadcast card provides support for legacy conditional access

| Required |  |  |
| --- | --- | --- |

34.  Digital broadcast card provides signal quality and other diagnostic information

| Required |  |  |
| --- | --- | --- |

35.  Digital broadcast card supports general-purpose data cryptography

| Recommended |  |  |
| --- | --- | --- |

36.  Digital broadcast card supports substream filtering

| Required |  |  |
| --- | --- | --- |

37.  ATSC DTV tuner is fully implemented

| Required |  |  |
| --- | --- | --- |

38.  Stream splitting is supported using DirectShow filters

| Recommended |  |  |
| --- | --- | --- |

39.  MPEG-2 decoder and video port support ATSC DTV standard

| Required |  |  |
| --- | --- | --- |

40.  Each device has a Plug and Play device ID

| Required |  |  |
| --- | --- | --- |

41.  Conflict resolution and dynamic disable capabilities are supported

| Required |  |  |
| --- | --- | --- |

42.  Dependent video device is not independently enumerated

| Required |  |  |
| --- | --- | --- |

43.  Device drivers and installation meet PC 98 requirements

| Required |  |  |
| --- | --- | --- |

44.  Software drivers are installed during hardware driver installation

| Required |  |  |
| --- | --- | --- |

45.  Applications provided with device meet Win32 requirements

| Required |  |  |
| --- | --- | --- |

46.  NDIS 5.0 driver provided for digital broadcast receiver

| Required |  |  |
| --- | --- | --- |

# Checklist for Monitors

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

1. Color monitor is DDC2B-compliant with unique EDID identifier

| Required | Required | Required |
| --- | --- | --- |

2. Monitor supports ICC color matching

| Required |  |  |
| --- | --- | --- |

3. Monitor meets all PC 98 general device and driver requirements

| Required |  |  |
| --- | --- | --- |

4. Monitor meets minimum graphics resolution, based on monitor size

| Required |  |  |
| --- | --- | --- |

5. Monitor supports ergonomic timing standards

| Required |  |  |
| --- | --- | --- |

6. Large-screen monitor is 20 inches or larger, if included with an Entertainment PC system

| Required |  |  |
| --- | --- | --- |

7. Large-screen monitor is 16:9, if included with PC system

| Recommended |  |  |
| --- | --- | --- |

8. Entertainment monitor supports 800 × 600 at 60 Hz refresh rate

| Required |  |  |
| --- | --- | --- |

9. Entertainment monitor’s host control is DDC2B-compliant, with digitally controlled geometry

| Recommended |  |  |
| --- | --- | --- |

10. External monitor meets DDC2B and EDID standards

| Required |  |  |
| --- | --- | --- |

11. Monitor complies with device class power management reference specification

| Required |  |  |
| --- | --- | --- |

# Checklist for Audio Components

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

1.  PC system includes PC 98 audio capabilities

| Recommended | Recommended | Required |
| --- | --- | --- |

2.  Audio device does not connect to ISA bus

| Required | Required | Required |
| --- | --- | --- |

3.  Audio performance meets PC 98 requirements

| Required |  |  |
| --- | --- | --- |

4.  Audio system provides support for basic data formats

| Required |  |  |
| --- | --- | --- |

5.  Audio system reports sample position for stream synchronization

| Required |  |  |
| --- | --- | --- |

6.  Audio system provides sufficient externally accessible inputs and outputs

| Required |  |  |
| --- | --- | --- |

7.  Audio system connectors are labeled with icons as defined for PC 98

| Required |  |  |
| --- | --- | --- |

8.  Audio performance meets PC 98 advanced recommendations

| Recommended |  |  |
| --- | --- | --- |

9.  Audio system supports full-duplex operation at independent sampling rates

| Recommended |  |  |
| --- | --- | --- |

10.  Audio system provides hardware or software support for the Downloadable Samples specification

| Recommended |  |  |
| --- | --- | --- |

11.  Audio system supports AEC reference inputs

| Recommended |  |  |
| --- | --- | --- |

12.  Audio system provides hardware filtering of HRTF 3-D filters

| Optional |  |  |
| --- | --- | --- |

13.  CD, DVD, and broadcast audio playback meet PC 98 requirements

| Required with DVD Video |  |  |
| --- | --- | --- |

14.  Audio system provides consistent volume levels for different devices

| Optional |  |  |
| --- | --- | --- |

15.  Each device has a unique Plug and Play device ID

| Required |  |  |
| --- | --- | --- |

16.  Automatic resource assignment and dynamic disable capabilities are supported

| Required |  |  |
| --- | --- | --- |

17.  PCI device conforms to PCI 2.1 and additional PC 98 requirements

| Required |  |  |
| --- | --- | --- |

18.  PCI device supports initiator, target, and block transfer

| Required |  |  |
| --- | --- | --- |

19.  PCI audio components use a suitable configuration scheme if using ISA resources

| Required |  |  |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

20.  PCI device is digital ready

| Required |  |  |
| --- | --- | --- |

21.  Audio meets USB specification and USB audio device class specification

| Required |  |  |
| --- | --- | --- |

22.  Audio meets PC 98 requirements for IEEE 1394

| Required |  |  |
| --- | --- | --- |

23.  System and device comply with PCI bus power management specification

| Required |  |  |
| --- | --- | --- |

24.  Audio device complies with device class power management reference specification

| Required |  |  |
| --- | --- | --- |

25.  Device supports wake-up events

| Optional |  |  |
| --- | --- | --- |

26.  Device drivers and installation meet PC 98 requirements

| Required |  |  |
| --- | --- | --- |

27.  Audio meets PC 98 requirements for WDM driver support

| Required |  |  |
| --- | --- | --- |

28.  Applications provided with device meet Win32 requirements

| Required |  |  |
| --- | --- | --- |

# Checklist for Storage and Related Peripherals

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

1.  Storage device and controller support bus master capabilities

| Required |  |  |
| --- | --- | --- |

2.  Removable media includes media status notification support

| Required |  |  |
| --- | --- | --- |

3.  Option ROMs support Int 13h Extensions

| Required |  |  |
| --- | --- | --- |

4.  Device meets PC 98 general device requirements

| Required |  |  |
| --- | --- | --- |

5.  Device meets PC 98 requirements for ports or buses

| Required |  |  |
| --- | --- | --- |

6.  Device Bay storage device meets PC 98 requirements

| Required |  |  |
| --- | --- | --- |

7.  IDE/ATAPI devices supported on IEEE 1394

| Recommended |  |  |
| --- | --- | --- |

8.  IDE/ATAPI devices and controllers support Ultra DMA/33

| Required |  |  |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

9.  USB-based mass storage device meets PC 98 requirements for USB

| Required |  |  |
| --- | --- | --- |

10.  System BIOS or option ROM supports El Torito No Emulation mode

| Required |  |  |
| --- | --- | --- |

11.  Floppy disk capabilities provided through expansion card or external bus

| Recommended |  |  |
| --- | --- | --- |

12.  IDE floppy drive complies with SFF 8070

| Required |  |  |
| --- | --- | --- |

13.  Legacy FDC built into system

| Optional |  |  |
| --- | --- | --- |

14.  Legacy FDC device meets resource configuration requirements

| Required |  |  |
| --- | --- | --- |

15.  System supports conflict resolution and dynamic disable capabilities for legacy FDC

| Required |  |  |
| --- | --- | --- |

16.  IDE hard drive is SMART-compliant and uses SMART IOCTL API

| Required |  |  |
| --- | --- | --- |

17.  IDE hard drive spin-up time supports OnNow capabilities

| Recommended |  |  |
| --- | --- | --- |

18.  CD-ROM drive provides 8x or higher performance

| Required |  |  |
| --- | --- | --- |

19.  CD-ROM drive is CD-Enhanced–compatible

| Required |  |  |
| --- | --- | --- |

20.  CD-ROM drive supports specified logical and physical CD formats

| Required |  |  |
| --- | --- | --- |

21.  IDE/ATAPI CD-ROM drive complies with SFF 8020i, Version 1.2

| Required |  |  |
| --- | --- | --- |

22.  CD-ROM drive supports multisession and compatibility forms of the READ_TOC command

| Required |  |  |
| --- | --- | --- |

23.  IDE/ATAPI CD changer meets SFF 8070 specification

| Required |  |  |
| --- | --- | --- |

24.  ATAPI rewritable device meets SFF 8070i specification

| Required |  |  |
| --- | --- | --- |

25.  DVD drive supports bus master DMA transfers

| Required |  |  |
| --- | --- | --- |

26.  DVD drive meets minimum compatibility requirements

| Required |  |  |
| --- | --- | --- |

27.  Device and driver support DVD command sets

| Required |  |  |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

28.  DVD device meets SFF 8090 specification

| Required |  |  |
| --- | --- | --- |

29.  DVD device uses high-speed expansion bus

| Required |  |  |
| --- | --- | --- |

30.  DVD drive supports UDF

| Required |  |  |
| --- | --- | --- |

31.  DVD device uses push-to-close design

| Recommended |  |  |
| --- | --- | --- |

32.  DVD device supports defect management

| Required |  |  |
| --- | --- | --- |

33.  DVD device supports copyright protection

| Required |  |  |
| --- | --- | --- |

34.  Each device has a Plug and Play device ID

| Required |  |  |
| --- | --- | --- |

35.  Conflict resolution and dynamic disable capabilities supported for all devices

| Required |  |  |
| --- | --- | --- |

36.  3F7h and 377h are unclaimed by devices

| Required |  |  |
| --- | --- | --- |

37.  Physical security is provided for storage devices

| Recommended | Recommended | Recommended |
| --- | --- | --- |

38.  Device and controller comply with device class power management reference specification

| Required |  |  |
| --- | --- | --- |

39.  Device supports wake-up events

| Optional |  |  |
| --- | --- | --- |

40.  Device drivers and installation meet PC 98 requirements

| Required |  |  |
| --- | --- | --- |

41.  Device and file system run in protected mode following installation

| Required |  |  |
| --- | --- | --- |

42.  Applications provided with the device meet Win32 requirements

| Required |  |  |
| --- | --- | --- |

43.  Driver for partitioned media supports all Windows and Windows NT partition types

| Required |  |  |
| --- | --- | --- |

44.  Driver for block-mode device supports extended BPBs

| Required |  |  |
| --- | --- | --- |

# Checklist for Modems

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

1.  Modem device is provided with PC system

| Required | Required, if no | PCM modem required, |
| --- | --- | --- |
|  | network adapter | upgradable to V.pcm |

2.  Modem supports TIA-602 Hayes-compatible command set

| Required |  |  |
| --- | --- | --- |

3.  Data modem supports 33.6 Kbps (V.34-1996) with V.42 and V.42bis protocol

| Required |  |  |
| --- | --- | --- |

4.  Data modem supporting speeds faster than 33.6 Kbps can be upgraded to V.pcm

| Required |  |  |
| --- | --- | --- |

5.  Fax modem supports 14.4 Kbps (V.17) with Class 1 (TIA-578-A) command set

| Required |  |  |
| --- | --- | --- |

6.  Data modem supports V. 80 for synchronous access

| Recommended |  |  |
| --- | --- | --- |

7.  Modem supports adaptive connection, V.25, V.8, and V.8bis call control signaling with V.25ter Annex A modem commands

| Recommended |  |  |
| --- | --- | --- |

8.  Modem supports delayed and blacklisted number clearing

| Recommended |  |  |
| --- | --- | --- |

9.  Modem supports TDD, meeting V.18-1996 with V.25ter AT commands

| Recommended |  |  |
| --- | --- | --- |

10.  PCM modem supports ITU-T V.pcm

| Required |  |  |
| --- | --- | --- |

11.  Modem controller meets PC 98 requirements

| Required |  |  |
| --- | --- | --- |

12.  Voice modem supports TIA-695 (AT+V)

| Recommended | Recommended | Required |
| --- | --- | --- |

13.  Voice modem support includes PC 98 recommendations

| Recommended |  |  |
| --- | --- | --- |

14.  Voice modem supports local telset interfaces

| Recommended |  |  |
| --- | --- | --- |

15.  Voice modem supports simultaneous voice/data integration capabilities

| Recommended |  |  |
| --- | --- | --- |

16.  Voice modem supports speakerphone

| Recommended |  |  |
| --- | --- | --- |

17.  Voice modem supports full-duplex voice I/O

| Recommended |  |  |
| --- | --- | --- |

18.  Wireless support implemented for modems

| Recommended |  |  |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

19.  Digital cellular phone support is implemented for modems

| Recommended |  |  |
| --- | --- | --- |

20.  ISDN modem supports required command set

| Required |  |  |
| --- | --- | --- |

21.  ISDN modem supports auto-SPID detection algorithms and standard SPID format

| Required |  |  |
| --- | --- | --- |

22.  ISDN modem supports CHAP in firmware if B channels are not exposed

| Required |  |  |
| --- | --- | --- |

23.  ISDN modem exposes both B channels

| Recommended |  |  |
| --- | --- | --- |

24.  ISDN modem supports multilink PPP

| Recommended |  |  |
| --- | --- | --- |

25.  ISDN modem supports asynchronous-to-synchronous conversion

| Required |  |  |
| --- | --- | --- |

26.  ISDN modem uses high-speed port

| Recommended |  |  |
| --- | --- | --- |

27.  ISDN driver supports switch detection

| Recommended |  |  |
| --- | --- | --- |

28.  ISDN driver supports unattended installation, with limitations

| Required |  |  |
| --- | --- | --- |

29.  Each device has a unique Plug and Play device ID

| Required |  |  |
| --- | --- | --- |

30.  Each device has a compatible Plug and Play device ID

| Required |  |  |
| --- | --- | --- |

31.  Automatic resource assignment and dynamic disable capabilities are supported

| Required |  |  |
| --- | --- | --- |

32.  PCI modem meets PCI 2.1 requirements

| Required |  |  |
| --- | --- | --- |

33.  USB modem meets USB specifications

| Required |  |  |
| --- | --- | --- |

34.  Device Bay modem meets PC 98 requirements

| Required |  |  |
| --- | --- | --- |

35.  Device complies with device class power management reference specification

| Required |  |  |
| --- | --- | --- |

36.  Device supports wake-up events

| Required |  |  |
| --- | --- | --- |

37.  Device drivers and installation meet PC 98 requirements

| Required |  |  |
| --- | --- | --- |

38.  Driver supports Unimodem

| Required |  |  |
| --- | --- | --- |

39.  Applications provided with device meet Win32 requirements

| Required |  |  |
| --- | --- | --- |

# Checklist for Network Communications

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

1.  PC system includes network adapter

| Recommended | Required, if no modem | Recommended |
| --- | --- | --- |

2.  PC system includes internal or external ISDN device

| Recommended | Recommended | Recommended |
| --- | --- | --- |

3.  PC system includes cable modem

| Optional | Optional | Optional |
| --- | --- | --- |

4.  PC system includes ATM adapter

| Optional | Optional | Optional |
| --- | --- | --- |

5.  PC system includes ADSL adapter

| Optional | Optional | Optional |
| --- | --- | --- |

6.  PC system includes satellite or broadcast receiver with NDIS driver

| Recommended | Recommended | Recommended |
| --- | --- | --- |

7.  Adapter uses NDIS 5.0 miniport driver

| Required |  |  |
| --- | --- | --- |

8.  Full-duplex adapter automatically detects and switches to duplex mode

| Required |  |  |
| --- | --- | --- |

9.  Adapter automatically senses presence of functional network

| Required |  |  |
| --- | --- | --- |

10.  Adapter automatically senses transceiver type

| Required |  |  |
| --- | --- | --- |

11.  Adapter supports quadword buffer alignment for receive and byte buffer alignment for send

| Required |  |  |
| --- | --- | --- |

12.  Adapter communicates with driver across any bridge

| Required |  |  |
| --- | --- | --- |

13.  Adapter supports filtering for 32 multicast addresses, at minimum

| Required |  |  |
| --- | --- | --- |

14.  Adapter is compatible with remote new system setup capabilities if used as boot device

| Required |  |  |
| --- | --- | --- |

15.  Device Bay network adapter meets PC 98 requirements

| Required |  |  |
| --- | --- | --- |

16.  Internal ISDN device meets PC 98 network adapter requirements

| Required |  |  |
| --- | --- | --- |

17.  Internal ISDN device supports synchronous HDLC framing

| Required |  |  |
| --- | --- | --- |

18.  Internal ISDN device uses NDIS WAN miniport driver

| Required |  |  |
| --- | --- | --- |

19.  Internal ISDN device includes connection for analog phone

| Recommended |  |  |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

20.  ISDN device supports auto-SPID detection algorithms and standard SPID format

| Required |  |  |
| --- | --- | --- |

21.  ISDN driver supports switch detection

| Recommended |  |  |
| --- | --- | --- |

22.  ISDN driver supports unattended installation, with limitations

| Required |  |  |
| --- | --- | --- |

23.  ISDN device includes built-in NT-1

| Recommended |  |  |
| --- | --- | --- |

24.  ATM adapter meets PC 98 network adapter requirements

| Required |  |  |
| --- | --- | --- |

25.  ATM adapter supports a minimum number of active connections

| Required |  |  |
| --- | --- | --- |

26.  ATM adapter supports all service types defined by the ATM Forum

| Recommended |  |  |
| --- | --- | --- |

27.  ATM adapter supports a minimum number of simultaneously active rt-VBR/nrt-VBR/CBR connections

| Required |  |  |
| --- | --- | --- |

28.  ATM adapter supports traffic shaping

| Required |  |  |
| --- | --- | --- |

29.  ATM adapter supports external clocking

| Required |  |  |
| --- | --- | --- |

30.  ATM adapter supports OAM

| Recommended |  |  |
| --- | --- | --- |

31.  ATM adapter supports buffer chaining (Tx + Rx)

| Recommended |  |  |
| --- | --- | --- |

32.  ADSL device meets PC 98 network adapter requirements

| Required |  |  |
| --- | --- | --- |

33.  ATM/ADSL solution is implemented

| Recommended |  |  |
| --- | --- | --- |

34.  ADSL device supports RADSL

| Recommended |  |  |
| --- | --- | --- |

35.  Each device has a unique Plug and Play device ID

| Required |  |  |
| --- | --- | --- |

36.  Automatic resource assignment and dynamic disable capabilities are supported

| Required |  |  |
| --- | --- | --- |

37.  Plug and Play capabilities support multiple adapters

| Required |  |  |
| --- | --- | --- |

38.  All resource settings are reported in the user interface

| Required |  |  |
| --- | --- | --- |

39.  Device complies with device class power management reference specification

| Required |  |  |
| --- | --- | --- |

| Consumer PC 98 | Office PC 98 | Entertainment PC 98 |
| --- | --- | --- |

40.  Device supports wake-up events

| Required |  |  |
| --- | --- | --- |

41.  Device drivers and installation meet PC 98 requirements

| Required |  |  |
| --- | --- | --- |

42.  Driver supports promiscuous mode

| Required |  |  |
| --- | --- | --- |

43.  Driver works correctly with Microsoft network clients and protocols

| Required |  |  |
| --- | --- | --- |

44.  NDIS miniport driver does not make operating system–specific kernel calls

| Required |  |  |
| --- | --- | --- |

45.  NDIS 5.0 driver uses new INF format

| Required |  |  |
| --- | --- | --- |

# Checklist for Printers

1.  IEEE 1394 printer meets PC 98 requirements for IEEE 1394

| Required |  |  |
| --- | --- | --- |

2.  USB printer meets PC 98 requirements for USB devices

| Recommended |  |  |
| --- | --- | --- |

3.  IEEE 1284 printer supports compatibility mode, nibble mode, and ECP, compliant with IEEE 1284-I

| Required |  |  |
| --- | --- | --- |

4.  IEEE 1284 printer meets IEEE 1284-II requirements

| Recommended |  |  |
| --- | --- | --- |

5.  ECP printer works correctly when ECP mode is turned off

| Required |  |  |
| --- | --- | --- |

6.  IEEE 1284 hardware supports error notification

| Required |  |  |
| --- | --- | --- |

7.  Implement Plug and Play support for all supported buses

| Required |  |  |
| --- | --- | --- |

8.  Peripheral device meets IEEE 1284 requirements

| Required |  |  |
| --- | --- | --- |

9.  Printer INF file and installation meet PC 98 requirements

| Required |  |  |
| --- | --- | --- |

10.  Driver correctly reports device capabilities

| Required |  |  |
| --- | --- | --- |

11.  Driver supports error notification

| Required |  |  |
| --- | --- | --- |

12.  Driver supports ICC color matching

| Required |  |  |
| --- | --- | --- |

13.  Port monitor software meets DDK guidelines

| Required |  |  |
| --- | --- | --- |

14.  Driver supports point-and-print network installation

| Required |  |  |
| --- | --- | --- |

15.  Device available immediately following installation

| Required |  |  |
| --- | --- | --- |

16.  Device supports accurate printable regions

| Required |  |  |
| --- | --- | --- |

17.  Driver supports required DDIs

| Required |  |  |
| --- | --- | --- |

18.  Driver based on unidriver

| Recommended |  |  |
| --- | --- | --- |

# Checklist for Scanners and Digital Cameras

1.  Device uses PC 98 compatible port connection

| Required |  |  |
| --- | --- | --- |

2.  Icons provided for port and peripheral connectors

| Required |  |  |
| --- | --- | --- |

3.  Device supports ICC color matching

| Required |  |  |
| --- | --- | --- |

4.  IR device meets PC 98 IR requirements

| Required |  |  |
| --- | --- | --- |

5.  SCSI device meets PC 98 SCSI requirements

| Required |  |  |
| --- | --- | --- |

6.  SCSI device attaches to any PC 98-compliant SCSI controller

| Required |  |  |
| --- | --- | --- |

7.  USB device meets PC 98 USB requirements

| Required |  |  |
| --- | --- | --- |

8.  USB device supports string descriptors

| Required |  |  |
| --- | --- | --- |

9.  IEEE 1394 device meets PC 98 requirements for IEEE 1394

| Required |  |  |
| --- | --- | --- |

10.  Plug and Play capabilities implemented for all supported buses

| Required |  |  |
| --- | --- | --- |

11.  Each device has a Plug and Play device ID

| Required |  |  |
| --- | --- | --- |

12.  Device supports power management requirements for its bus

| Required |  |  |
| --- | --- | --- |

13.  Device drivers and installation meet PC 98 requirements

| Required |  |  |
| --- | --- | --- |

14.  Driver support is implemented under Still Image architecture

| Required |  |  |
| --- | --- | --- |

15.  Applications provided with the device meet Win32 specifications

| Required |  |  |
| --- | --- | --- |