TABLE OF CONTENTS

H. Appendix H - CID Support

H.1 Creating A CID Installation Directory

H.2 Specifying CID Parameters At Installation

H.3 Creating A Response File

H.3.1 Overview

H.3.2 Keywords For Basic Installation And Upgrades

H.3.3 Keywords For ADD-ON Diskettes

H.3.4 Keywords For CSD Diskettes

H.4 Sample File Server Directory Structure

H.5 Sample Response File

H. Appendix H - CID Support

The TalkThru  installation   program      supports  the  IBM  Configuration,

Installation, and Distribution facility (CID).  This support includes:

1. The installation of TalkThru from a File Server on a Local Area Network.

2. Support for a Response File which will allow the installation process  to

occur without user interference.

H.1 Creating A CID Installation Directory

In order  to allow TalkThru installation from a File Server, you first  must

create  an image of the TalkThru installation diskettes on the File  Server.

This is accomplished by performing the following steps:

1. Invoke  the  TalkThru  installation  program  as  you  would  to  install

TalkThru to your hard drive.

2. Request "Create CID Disk Image" from this dialog.

ADD-ON  diskettes are provided by SCA and contain support for features  not

in the  base  product.   CSD diskettes are diskettes provided  by  SCA  and

contain  fixes  and minor enhancements implemented in the  product  between

major  releases.  If  you are creating image files for  an  ADD-ON  or  CSD

diskette, place that diskette in the diskette drive and press OK.

If you  are not creating an image file for an ADD-ON or CSD diskette,  just

press OK.

3. From the next dialog, enter the name of the directory on your File Server

to contain the image of the installation files,  for example:

X:\NETWORK\PRODUCT\IMG\TALKTHRU

and press OK..

At this  point,  the installation will proceed normally and  will  create  a

mirror  image,  in  the  specified directory,  of  the  files  required  for

TalkThru installation.

Once  this is completed, TalkThru may be installed by the IBM CID  facility.

If you  use  the  IBM  CID facility, you will  probably  wish  to  create  a

Response  File.   See the section, Creating A Response File  later  in  this

Appendix  for more information on Response Files.  Once it is  created,  the

section  Specifying CID Parameters At Installation will provide  information

on how the name of the Response File can be specified.

If you  wish to install TalkThru from the File Server without utilizing  the

IBM CID facility, you may do so from any PC with access to this File  Server

by:

- initiating an OS/2 Window

- making the directory specified in step 3 above the current directory

- typing install and pressing Enter

The installation  process  will  proceed  exactly   as  it  would  from  the

installation diskettes.

H.2 Specifying CID Parameters At Installation

The TalkThru installation process supports the specification of several  CID

compliant  parameters  on  the command line to  pre-answer  questions  posed

during  the  installation  process.    When  the  TalkThru  installation  is

requested,  any  of  the  parameters specified in the  table  below  may  be

provided on the command line in this format:

����������������������������������������������������������������������Ŀ

� INSTALL parm1 parm2 ... parmn                                        �

������������������������������������������������������������������������

parm  Specify           Remarks

����  ����������������  ����������������������������������������������������

/r:   Response File     This should be the full path.  If it is  specified,,

the user is NEVER prompted  for  information  during

the install process.  Any information not  contained

in this  file  or  provided  in  one  of  the  other

parameters  is  defaulted.<R><R>The  contents  and

format of the Response File is provided in the  next

section,, Creating A Response File.

/s:   Source Directory  This  is  the  directory  you  specified  when  you

created  the  image  of  the  TalkThru  installation

files.  For more  information  on  this  directory,,

refer  to  the  previous  section  Creating  A  CID

Installation Directory.

/t:   Target Directory  This is an optional parameter  specifying  a  target

directory  to  contain  the  TalkThru  directories.

This may be defaulted to  C:\TALKTHRU  or  specified

in the Response File.

/tu:  CONFIG.SYS        This  is  an  optional  parameter  specifying  the

location of the CONFIG.SYS  File.      This  may  be

defaulted  to  C:\CONFIG.SYS  or  specified  in  the

Response File.

/l1:  ERROR LOG         This is an optional parameter  specifying  the  full

path of an ERROR LOG which will  contain  any  error

messages  produced  by  the  TalkThru  installation

process.

/l2:  HISTORY LOG       This is an optional parameter  specifying  the  full

path of a HISTORY  LOG  which  will  contain  status

messages  produced  by  the  TalkThru  installation

process.

H.3 Creating A Response File

����������������Ŀ

� H.3.1 Overview �

������������������

If you do not wish to have the user specify any answers to  the  questions

posed during  TalkThru  installation,  you  may  specify  the  name  of  a

Response  File  on  the  command  line  when  TalkThru  installation  is

requested.  For more information on how this is specified,  refer  to  the

previous section, Specifying CID Parameters At Installation.

The TalkThru Response File is composed  of  KEYWORDS  and  VALUES  in  the

format:

KEYWORD=VALUE

If a keyword is specified  without  a  value,  a  DEFAULT  value  will  be

supplied. The keywords are divided into three topics...

o Basic TalkThru Installation Or Release Upgrade

o ADD-ON TalkThru Installation

o CSD TalkThru Installation

The Response File may contain one or all topics at the same time. If  more

than one topic is used, the order of installation will be..

- Basic Or Upgrade

- ADD-ON

- CSD

There is a special Keyword that allows the user to  run  the  installation

program and VALIDATE the Response File. NO installation will  take  place.

To use this feature you must specify

VALIDATE_RESPONSE_FILE=YES

in the Response File and use the parameters /l1: for the  error  file  and

/l2: for the history file.  For example:

INSTALL /R:E:\TEST.RSP /L1:E:\ERROR.LOG /L2:E:\HISTORY.LOG

If these two parameters are not used, there cannot be any  record  of  the

validation.

Below is a table containing the possible KEYWORDS and VALUES that  may  be

specified in the Response File.

����������������������������������������������������Ŀ

� H.3.2 Keywords For Basic Installation And Upgrades �

������������������������������������������������������

Keyword                      Value Type               Default

��������������������������������������������������������������������������

TALKTHRU_INSTALL             YES/NO                   YES

TALKTHRU_MAIN_DIRECTORY      Directory                C:\TALKTHRU

TALKTHRU_CONFIG_FILENAME     Filename                 TALKTHRU.DEF

PROGRAM_DIRECTORY            (Sub)Directory           PROGRAMS

SESSION_DIRECTORY            (Sub)Directory           SESSIONS

SCRIPT_DIRECTORY             (Sub)Directory           SCRIPTS

ICON_DIRECTORY               (Sub)Directory           ICONS

DATA_DIRECTORY               (Sub)Directory           DATA

MIN_MAIN_DIR_MEMORY          number of bytes          1024

MIN_PROGRAM_DIR_MEMORY       number of bytes          3350000

MIN_SESSION_DIR_MEMORY       number of bytes          200000

MIN_SCRIPT_DIR_MEMORY        number of bytes          300000

MIN_DATA_DIR_MEMORY          number of bytes          0

MIN_ICON_DIR_MEMORY          number of bytes          150000

EHLLAPI                      YES Or NO                YES

EDITOR_NAME                  User's or                E.EXE

LOCAL_PRINTER                LPT1...Other             LPT1

DEFAULT_DEVICE               COM1...COM4              COM1

MODEM_NAME                   Modem from MODEMS.DAT    DEFAULT

RUN_INSTALLATION             HIDE/MIN/NORM            NORM

TALKTHRU_RUNNING             STOP/CONT                STOP

PROCESS_LOCKED_FILES         NO                       NO

OS2CONFIG_DIRECTORY          Directory                C:\

NOTE: If present, this signifies to update CONFIG.SYS.

TALKTHRU_LANGUAGE            US                       Etc..

ADD_TALKTHRU_DESKTOP         YES/NO                   YES

DESKTOP_TITLE                String Name              TalkThru

INSUFFICIENT_MEMORY          STOP/CONT                STOP

COMM_MANAGER_DIRECTORY       Directory                C:\CMLIB

NOTE: If present, this signifies that the Communications Manager is

used.

DISK_PROMPT                  YES/NO                   NO

NOTE: A Response File is to be used, but TalkThru is not in a CID

image format.  This keyword will force the issuing of the disk

swap messages as needed.

�������������������������������������Ŀ

� H.3.3 Keywords For ADD-ON Diskettes �

���������������������������������������

SCA occasionally  provides  ADD-ON  diskettes  which  offer  features  not

available in the base TalkThru products.  The  Installation  program  will

allow UP TO five ADD-ON diskettes during the same installation.  In  order

to perform an  ADD-ON  installation,  the  install  program  requires  two

pieces of information.

o Where to get the ADD-ON Data.

o The location of the TalkThru definition  file.    From  the  definition

file, the other directories/subdirectories are obtained.

Keyword                     Value Type                             Default

��������������������������  �������������������������������������  �������

ADDON_INSTALL               YES/NO                                 NO

ADDON_IMAGE_SUBDIRECTORY_1  IMAGE

Directory/SubDirectory                 NONE

ADDON_TALKTHRU_DEF_PATH_1   Directory & Filename of

TALKTHRU.DEF File                    NONE

ADDON_IMAGE_SUBDIRECTORY_2  IMAGE

Directory/SubDirectory                 NONE

ADDON_TALKTHRU_DEF_PATH_2   Directory & Filename of

TALKTHRU.DEF File                    NONE

ADDON_IMAGE_SUBDIRECTORY_3  IMAGE

Directory/SubDirectory                 NONE

ADDON_TALKTHRU_DEF_PATH_3   Directory & Filename of

TALKTHRU.DEF File                    NONE

ADDON_IMAGE_SUBDIRECTORY_4  IMAGE

Directory/SubDirectory                 NONE

ADDON_TALKTHRU_DEF_PATH_4   Directory & Filename of

TALKTHRU.DEF File                    NONE

ADDON_IMAGE_SUBDIRECTORY_5  IMAGE

Directory/SubDirectory                 NONE

ADDON_TALKTHRU_DEF_PATH_5   Directory & Filename of

TALKTHRU.DEF File                    NONE

����������������������������������Ŀ

� H.3.4 Keywords For CSD Diskettes �

������������������������������������

Between major releases of TalkThru, SCA will produce CSD disks  containing

fixes and minor  enhancements  to  the  prior  release.  The  installation

program will allow UP TO five CSD diskettes during the same  installation.

In order to perform a CSD installation the install  program  requires  two

pieces of information.

o Where to get the CSD Data.

o The location of the TalkThru definition  file.    From  the  definition

file, the other directories/subdirectories are obtained.

Keyword                   Value Type                               Default

������������������������  ���������������������������������������  �������

CSD_INSTALL               YES/NO                                   NO

CSD_IMAGE_SUBDIRECTORY_1  IMAGE

Directory/SubDirectory                   NONE

CSD_TALKTHRU_DEF_PATH_1   Directory & Filename of

TALKTHRU.DEF FILE                     NONE

CSD_IMAGE_SUBDIRECTORY_2  IMAGE

Directory/SubDirectory                   NONE

CSD_TALKTHRU_DEF_PATH_2   Directory & Filename of

TALKTHRU.DEF FILE                     NONE

CSD_IMAGE_SUBDIRECTORY_3  IMAGE

Directory/SubDirectory                   NONE

CSD_TALKTHRU_DEF_PATH_3   Directory & Filename of

TALKTHRU.DEF FILE                     NONE

CSD_IMAGE_SUBDIRECTORY_4  IMAGE

Directory/SubDirectory                   NONE

CSD_TALKTHRU_DEF_PATH_4   Directory & Filename of

TALKTHRU.DEF FILE                     NONE

CSD_IMAGE_SUBDIRECTORY_5  IMAGE

Directory/SubDirectory                   NONE

CSD_TALKTHRU_DEF_PATH_5   Directory & Filename of

TALKTHRU.DEF FILE                     NONE

H.4 Sample File Server Directory Structure

A typical IMAGE structure on a Network might be:

For An Entire TalkThru Release:

NETWORK\IMG\PRODUCT\TALKTHRU\VER22\PROGRAMS

\ICONS

\SCRIPTS

\ICONS

\CID

\MODEMS

\APP_DLLS

\SESSIONS

\CM_DLLS

\SYS_DLLS

\SAMPLES

\LANGUAGE

For An ADD-ON Disk:

NETWORK\IMG\PRODUCT\TALKTHRU\VER22\ADDON340\PROGRAMS

\APP_DLLS

For A CSD Disk:

NETWORK\IMG\PRODUCT\TALKTHRU\VER22\CSD03123\PROGRAMS

\APP_DLLS

H.5 Sample Response File

Below is a sample of what a Response File might look like:

;==========================================

; Software Corporation of America

; 100 Prospect Street

; Stamford CT. 06901 (USA)

; 203 359 2773

;

;    Sample INSTALLR  response file

;

;==========================================

TALKTHRU_INSTALL=YES

;

TALKTHRU_MAIN_DIRECTORY=C:\TALKTHRU

TALKTHRU_CONFIG_FILENAME=TALKTHRU.DEF

PROGRAM_DIRECTORY=C:\TALKTHRU\PROGRAMS

SESSION_DIRECTORY=C:\TALKTHRU\SESSIONS

SCRIPT_DIRECTORY=C:\TALKTHRU\SCRIPTS

DATA_DIRECTORY<T>=C:\TALKTHRU\DATA

ICON_DIRECTORY<T>=C:\TALKTHRU\ICONS

;

MIN_MAIN_DIR_MEMORY=1024

MIN_PROGRAM_DIR_MEMORY=3000000

MIN_SESSION_DIR_MEMORY=200000

MIN_SCRIPT_DIR_MEMORY=300000

MIN_DATA_DIR_MEMORY=0

MIN_ICON_DIR_MEMORY=150000

;

EHLLAPI=YES

EDITOR_NAME=E.EXE

LOCAL_PRINTER=LPT1

DEFAULT_DEVICE=COM1

MODEM_NAME=DEFAULT

;

RUN_INSTALLATION=NORM

TALKTHRU_RUNNING=STOP

DISK_PROMPT=YES

PROCESS_LOCKED_FILES=NO

OS2CONFIG_DIRECTORY=C:\

TALKTHRU_LANGUAGE=US

ADD_TALKTHRU_DESKTOP=YES

DESKTOP_TITLE=TalkThru

INSUFFICIENT_MEMORY=STOP