OPSD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97124-6497

DATE:	June 03, 1999

TO:	OPSD Business Unit

CC:	iPG / OPSD BIOS Engineering

FROM:	iPG / OPSD BIOS Engineering

SUBJECT:	A Standard BL440ZX BIOS

About This Release

- 4B4LZ0XA.86A.0007.P04.9904191150

### Features/Errata Fixed in This Release

#### ***UPDATE THE BOOT BLOCK WITH .BBO FILE BEFORE UPDATING SYSTEM BIOS**

#### P04-0007:

- Fixed the problem where the on-board LAN had resource conflicts with other system devices.

#### P03-0006:

- Fixed the problem where the system might hang at checkpoint 0x2D (dependent on memory type installed) when a Full Screen User Logo is being used.

- Fixed the problem where CMOS defaults were loaded when no video was detected in the system.

- Fixed the problem where a Celeron™  433 MHz processor is reported as a Celeron™  450 MHz processor during POST.

- Removed I20 boot option from First Boot Device under Boot sub menu in BIOS setup.

- Removed ECC Event Logging option from the Event Log Configuration menu on the Advanced tab within the BIOS setup Utility.

- Fixed the problem where if the F10 key was pressed after a CMOS Checksum Bad message during POST caused a system hang.

- Fixed the problem where pressing ESC during POST launches LANDesk Service Agent and then hangs the system.

- Fixed the problem where Small Splash Logo screen is not visible when POST text is enabled.

- Fixed the IRQ resource issue that will allow a user to reserve all but one IRQ listed in under the Advanced Resource Configuration submenu within the BIOS setup Utility.

- Fixed a problem with the BIOS Update API reporting that all slots are full when both slots where full and a new BIOS update is uploaded.

- Added a Fix for the OS/2 Warp 4.0: APM Suspend from Start Menu.

- Updated DMI type 4 structure for MAX Processor Speed.

- Added SCSI selection as a 2nd, 3rd or 4th boot device.

- Added help text so that Setup is translated for foreign languages.

- Fixed a problem with Video corruption in setup Menu when a Custom default configuration was loaded.

- Fixed a problem when PCI cards that have an option ROM whose size is not a multiple of 2K.

- Fixed Screen Menu Header misalignment and appearing / disappearing setup questions.

- Fixed problem where WOL would not work with Power loss.

- Set LPT IRQ default as IRQ7 when Parallel Port has been selected as "Enabled".

- Allow boot from ATAPI ZIP drive when "ARMD-HDD" selected as boot device in Setup.

- Updated the Copyright message to 1999.

- Fixed a problem where only three unique addresses showed up for serial port A.

- Fixed a problem where the BIOS asked for a password upon boot even if the supervisor password was the only password set.

- Fixed a problem where the BIOS screen did not get refreshed correct, when unattended start was enabled.

- Fixed a hang at checkpoint 8Ch caused by DIMMs.

- Added support to force the display of the POST screen during OPROM initialization.

- Fixed a problem where IDE drives would not be auto detected if the ESC key was pressed during drive detection.

- Fixed a problem where the SMBIOS System slot information, was missing the PCI slot information, and reporting incorrect ISA slot information.

#### P02-0004:

- Changed BIOS ID string to Production BIOS.

- Fixed an issue with Legacy ISA Memory Reservation.

- Fixed boot from LCM with onboard Intel 82558 NIC

- WOR, WOL, Wakeup from Kbd/Mouse in ACPI S1 state.

- Fixed issue with setting of Manufacturing Defaults, which caused test issues in evaluation.

- Fixed a problem where the convert and restore files would not work on custom BIOS's.

- Updated ACPI code for additional compliance.

- Fixed a problem where clearing of the Event Log was causing a test hang in VM86 Mode.

- Updated Quiet Boot support to use Product Specific Switches to	    enable usage of multiple logo sets (a splash/corner logo combo).

- Fixed problem where system boots slowly (up to 50 Sec delay) when the floppy cable has been disconnected.

- Fixed Creative Blaster TNT hangs at post code 2D.

- Added help strings for Event Log Configuration menu under            Advanced menu.

- Added Boot device help strings in Boot menu.

- Added a help text string for AGP Aperture Size question. All languages.

- Added help text strings for Advanced menus.

- Fixed a problem where the user installed BIOS updates was not	        being saved into flash.

- Added support for front panel button to function as a user request for standby and wakeup from standby.

- Mask the PIO/UDMA settings when the IDE configuration is set to Auto.

#### P01-0002:

- Initial Release.