MON85

A

Software Debug Monitor

For the 8085/8080

Copyright 1971,1992 Dave Dunfield

All rights reserved

MON85                                                            Page: 1

1. INTRODUCTION

MON85 is a ROMable interactive  debugging  program  for  the  8085

processor, which contains a full complement of  commands  to  monitor

and control the execution of your program. It may also be used on  an

8080 processor, as long as  you  do  not  attempt  to  use  the  8085

specific SIM  and  RIM  instructions,  or  the  '.5'  type  interrupt

vectors.

MON85 must be installed beginning at location $0000  in  the  8085

processor memory map, allowing it to intercept the interrupt vectors.

Provision has been made to re-vector interrupts to  locations  within

the user program.

All functions of MON85 are performed via software without hardware

assist. The  only  hardware  specific  subroutines  required  by  the

monitor are used to communicate with the console  terminal,  and  are

located at the very end of the monitor source code listing:

INIT    - Called to initialize any hardware required for I/O.

OUT     - Write the character in A to the console. No processor

registers should be modified by this routine.

IN      - Test for a character from the console, and return it

in A if one is available. Otherwise clear A to zero.

No other registers should be modified by this routine.

1.1 8085 Restart Interrupts

MON85 reserves two "restart" interrupts: RST 0 is similar to  a

physical RESET,  causes  a  cold  restart  of  the  monitor.  This

instruction should be used with CAUTION during a debugging session

because  any  breakpoints  set  in  the  user  program   will   be

"forgotten", and not properly removed. RST 1 is used by  MON85  to

regain   control   at   breakpoints.   Opcodes   of   breakpointed

instructions are replaced by RST 1 whenever the  user  program  is

executed, and are restored whenever  the  monitor  is  re-entered.

This insures that the operation of breakpoints is  transparent  to

you during the debugging session.

If for any reason a 'RST 1' ($CF) instruction is encountered in

the user program (and is not a breakpoint), command mode  will  be

entered without the '** Breakpoint' message.

All  other  "restart"  interrupts  are   re-vectored   to   the

corresponding locations in the first page of  memory  occupied  by

the user program (as identified by the 'U' command).

MON85                                                            Page: 2

1.2 Breakpoints

MON85 allows you to set breakpoints in your program, such  that

you  will  be  given  control  whenever  the  program  reaches   a

breakpoint, and can examine or change  things  before  proceeding.

MON85 also allows you to TRACE a program, so that you can see each

instruction and register contents as it executes.

MON85 is completely transparent to  the  program  being  tested

(unless timing loops are interrupted with BREAKPOINTS  or  TRACE).

When a breakpoint is encountered, or  an  instruction  is  traced,

MON85 uses one stack entry on the user program stack.  Since  this

consists of a PUSH and a POP, it should not affect any information

stacked by the user program. However, you should be aware of this,

in case you are examining the  stack,  or  your  program  trys  to

reclaim data already popped from  the  stack.  (It  is  very  poor

practice, to write programs which depend  on  the  stack  contents

below the stack pointer).

When a breakpoint is encountered, the message '** Breakpoint n'

is printed where n is the number of the breakpoint (0-7). If TRACE

is ON, no other action is taken, otherwise command mode is entered

(If the 'A' switch (see  below)  is  ON  the  registers  are  also

displayed).

MON85                                                            Page: 3

2. MON85 COMMANDS

The following commands are recognized by MON85, and may be entered

whenever MON85 is in COMMAND MODE, which is indicated by  the  'C>  '

prompt:

2.1 A ON|OFF

This switch turns ON or OFF the automatic register  display  which

occurs whenever a breakpoint is encountered or an  instruction  is

stepped in trace mode (see below). The default value  for  'A'  is

ON.

2.2 B [0-7 address]

Sets one of eight breakpoints [0-7] at the specified address. Once

set, a breakpoint remains in effect until it is removed by setting

it to address ZERO (0). Breakpoints are completely invisible,  and

may be added, removed or  changed  at  any  time  without  adverse

affects. If the 'B'  command  is  issued  with  no  operands,  the

current breakpoints, and the settings of  the  'A',  'S'  and  'T'

flags (see below) are displayed. A  displayed  address  of  '****'

indicates that a breakpoint is not set.

2.3 C <source> <destination> <size>

Copies  <size>  bytes  of  memory  from  the   <source>   to   the

<destination> address.

2.4 D <address>

Displays memory in assembly listing format, starting at <address>,

continuing until an ESCAPE character is  entered.  Output  can  be

temporarily stopped with the SPACE BAR,  and  restarted  with  the

RETURN key (Additional SPACE's  will  output  single  lines).  The

output is displayed in the following form:

'<address> <opcodes> <ascii> <instruction>'

<address>  is  the  current  memory  location,  <opcodes>  is  the

instruction  opcodes  (1,  2  or  3  bytes),  <ascii>   is   ASCII

representation of  <opcodes>  (all  non-printable  characters  are

displayed as dots), and <instruction>  is  the  assembly  language

instruction and operands which <opcodes> represents.

2.5 E <address>

Edits memory, starting at <address>. The address, and its contents

are displayed, followed by a '-' prompt. Sub commands are:

nn [nn ...] - Replace memory contents with HEX data bytes

'text ...   - Replace memory contents with ASCII text

-           - Backup to previous locations

<blank line>- Advance to next location with change

<ESCAPE>    - Return to MON85 command prompt

MON85                                                            Page: 4

2.6 F <start> <end> <value>

Fills memory between the <start> and  <end>  addressses  with  the

specified <value>.

2.7 G [address]

Loads the user program registers,  and  begins  execution  at  the

specified address. If no [address] is given, execution  begins  at

the address contained in the user program program counter (PC).  A

simple 'G' with no operands, is  all  that  is  needed  to  resume

execution after a breakpoint interrupt, or to  resume  trace  mode

exection.

2.8 I <port>

Reads the specified 8085 I/O port, and displays the data.

2.9 L [address]

Downloads code from the console port.  MON85  will  recognize  and

accept either INTEL or MOTOROLA hex format download  records.  The

address in the FIRST download record is recorded  as  the  program

BASE address (See 'U' command). If  an  [address]  is  given,  the

address fields of each record are adjusted so that the code  image

will be loaded into memory beginning at that address, otherwise it

is placed in memory at the  absolute  address  contained  in  each

record.

If you accidently enter this command, you may enter either 'S9' or

':00' to signify a null download file and return  to  the  command

prompt.

2.10 M <address>

Displays memory in HEX/ASCII dump format starting at the specified

address. ESCAPE, SPACE, and CARRIAGE-RETURN can be used to control

the listing, the same is with the 'D' command.

2.11 O <port> <data>

This command writes the <data> byte  to  the  specified  8085  I/O

<port>.

2.12 R [rp value]

Changes the contents of the user program register pair <rp> to the

specified <value>. Valid pairs are BC, DE, HL, SP, PC, and PSW. If

no operands are given, the contents of the user program  registers

are displayed.

MON85                                                            Page: 5

2.13 S ON|OFF

Controls the handling of subroutine calls when in trace mode  (see

below). When 'S' is set ON, subroutines will be be traced  in  the

normal fashion. When set OFF, subroutines calls  are  not  traced,

and trace will resume at the next instruction following  the  CALL

(After the subroutine executes).

WARNING: DO NOT SET BREAKPOINTS IN THE SUBROUTINES WHEN 'S' IS SET

OFF. When 'S' is set off, MON85 considers the subroutine (and  all

of it's embedded instructions) to be one single  operation.  If  a

breakpoint is encountered inside the subroutine, TRACE  will  lose

the address at which to resume following the subroutine call. This

will cause  unpredictable  action  when  the  subroutine  returns.

Otherwise, 'S' may be turned on  and  off  at  any  time  with  no

adverse effects. The default value for 'S' is ON.

2.14 T ON|OFF

Turns TRACE mode ON and OFF. When TRACE is  set  ON,  and  the  GO

command is issued,  MON85  will  first  prompt  with  'T>'  before

beginning program execution.

Entering a space (' ') will display and execute  one  instruction,

and return to the 'T>' prompt.

Entering '?' will display the processor  registers.  (If  the  'A'

switch is set ON, they will always be  displayed  following  every

instruction that is traced).

An ESCAPE charecter will  cause  MON85  to  return  to  the  usual

command prompt (TRACE remains ON, and will resume  with  the  next

'G' command).

Trace may be turned on and off at any time in a program,  with  no

adverse effects. (If you begin execution with TRACE off, you  will

have to hit a breakpoint to get you back to  command  mode  before

you can turn TRACE on).

When a breakpointed instruction is encountered by the tracer,  The

message '** Breakpoint n'  will  be  issued  at  the  end  of  the

previous instruction, indicating that the breakpointed instruction

occurs NEXT.  Pressing  the  SPACE  BAR  would  then  execute  the

breakpointed instruction. The default value for 'T' is OFF.

2.15 U [address]

Identifies the starting address of a user program. MON85 uses this

address to re-map the "restart" interrupt vectors. When  a  'L'oad

command  is  performed,  MON85  initializes  'U'  to  the  address

contained in  the  first  download  record.  If  no  [address]  is

specified, MON85 will display the current  user  program  starting

address.

2.16 ?

This command displays a short summary of the other MON85 commnds.

MON85                                                            Page: 6

3. COMMAND SUMMARY

A ON/OFF                - Enables/Disables auto register display.

B [0-7 address]         - Sets/Removes/Displays breakpoints.

C <src> <dest> <size>   - Copy memory

D <address>             - Displays memory in disassembly format.

E <address>             - Edit memory contents.

F <start> <end> <value> - Fill memory with a value.

G [address]             - Begin/Resume program execution.

I <port>                - Input from port

L [address]             - Download from host

M <address>             - Displays memory in dump format.

O <port> <data>         - Output to port

R [rp value]            - Sets/Displays register contents.

S ON|OFF                - Enables/Disables subroutine traceing.

T ON|OFF                - Enables/Disables TRACE mode.

U [address]             - Set/Displays program base address.

?                       - Display command summary

MON85

TABLE OF CONTENTS

Page

1. INTRODUCTION                                                        1

1.1 8085 Restart Interrupts                                         1

1.2 Breakpoints                                                     2

2. MON85 COMMANDS                                                      3

2.1 A ON|OFF                                                        3

2.2 B [0-7 address]                                                 3

2.3 C <source> <destination> <size>                                 3

2.4 D <address>                                                     3

2.5 E <address>                                                     3

2.6 F <start> <end> <value>                                         4

2.7 G [address]                                                     4

2.8 I <port>                                                        4

2.9 L [address]                                                     4

2.10 M <address>                                                    4

2.11 O <port> <data>                                                4

2.12 R [rp value]                                                   4

2.13 S ON|OFF                                                       5

2.14 T ON|OFF                                                       5

2.15 U [address]                                                    5

2.16 ?                                                              5

3. COMMAND SUMMARY                                                     6