D51 Version 2 USER'S MANUAL

8051/8052 Disassembler

Jeffery L. Post

22726 Benner Ave,

Torrance, CA 90505

Version 2.2

May, 1996

MSDOS is a trademark of Microsoft Corporation

PCDOS is a trademark of International Business Machines

TABLE OF CONTENTS

Introduction .................................................. page 1

Quick start ................................................... page 3

Command line options .......................................... page 4

A option .................................................. page 4

B option .................................................. page 5

D option .................................................. page 5

H option .................................................. page 5

S option .................................................. page 5

X option .................................................. page 6

Control file directives ....................................... page 7

A directive ............................................... page 8

B directive ............................................... page 8

C directive ............................................... page 8

F directive ............................................... page 9

I directive ............................................... page 9

K directive ............................................... page 9

L directive ............................................... page 10

R directive ............................................... page 10

S directive ............................................... page 10

T directive ............................................... page 11

W directive ............................................... page 12

Format of Output File ......................................... page 13

Background .................................................... page 15

Technical info ................................................ page 16

Summary ....................................................... page 19

8051 register maps ............................................ page 21

Page 1

INTRODUCTION

D51 Version 2 is a significant improvement over D51 V1. Version 2

allows the user to specify areas of the code to be disassembled

as something other than executable code; ie: as ascii text, binary

data, address tables, etc (see the section on control file direct-

ives). You may specify such areas by means of an ascii text file

with the same name as the file to be disassembled, but with an

extension of CTL (for example: program.ctl controls the disassembly

of program.hex or program.bin). By means of the control file, you

can, by an iterative process, get a progressively more meaningful

disassembly by modifying the control file based on examination of

the output of previous passes of D51. You can also specify names

for labels and symbols in the control file, resulting in a much

more readable source file. A file named GENERIC.CTL is provided as

a means to get the user started; just copy GENERIC.CTL to FILE.CTL,

where FILE is the name of the file to be disassembled, and make

appropriate changes. Labels and symbols may have as many characters

as will fit in one line of the control file, but some cross-

assemblers (and especially cross linkers) will truncate any more

than some fixed number of characters. Consult your cross development

tools' documentation. Labels and symbols are case sensitive, so that

'Label' and 'LABEL' are two different labels. Your cross assembler

may not consider them such, so be forewarned.

Like V1, D51 V2 still does not analyze the code; the user must do

so and control disassembly via the control file. A code analyzing

disassembler is under development (as of May, 1995). SD51 (for Super

Disassembler 8051) will do static and dynamic analysis of the hex or

binary file, and automatically determine which parts of the file are

executable code, address tables, ascii text, etc. SD51 will likely

not be public domain, but will be distributed either as shareware or

as a commercial product (interested publishers can contact the author

at the address given above).

Unlike D51 V1, which was distributed as a public domain program

without source code, Version 2 is being distributed with source code

under the GNU General Public License (see the file COPYING for

details). The author has benefited greatly from software distributed

under the GNU GPL and wishes to thank those who have contributed so

much by adding his own meager efforts to the large base of freely

distributable software. It is the author's contention that software

patents are an oxymoron; that although the United States Patent Office

recognizes that mathematical algorithms are not patentable, they are

too ignorant to recognize the fact that ANY software is expressable as

a mathematical algorithm. The author specifically refuses to acknowledge

the legitimacy of any software patents whatsoever. Copyrights, trade-

marks and trade secrets are, however, another matter.

In accordance with the spirit of the GNU General Public License, the

following notice is added to this documentation:

Page 2

This program is free software; you can redistribute it and/or modify

it under the terms of the GNU General Public License as published by

the Free Software Foundation; either version 2 of the License, or (at

your option) any later version.

This program is distributed in the hope that it will be useful, but

WITHOUT ANY WARRANTY; without even the implied warranty of MERCHANT-

ABILITY or FITNESS FOR A PARTICULAR PURPOSE. See the GNU General

Public License for more details.

You should have received a copy of the GNU General Public License

along with this program; if not, write to the Free Software

Foundation, Inc., 675 Mass Ave, Cambridge, MA 02139, USA.

The file COPYING contains the full text of the GNU General Public

License, Version 2.

The quick start section of this document gives some minimal informa-

tion for users who are eager to try out D51. For best use of D51,

however, you should read the sections on command line options and

control file directives before proceeding.

The section entitled 'background' describes the purpose of a dis-

assembler, how a disassembler works (in basic terms), and the reasons

you might want to use one. This section is intended for novice users

but might be interesting to more advanced users.

The technical info section covers disassembly techniques and gives a

detailed description of D51 and its source code. This is intended for

more advanced users who might want to modify or expand D51, or even

create a disassembler for a different 8 bit processor. Those who peruse

this section, and the source code, will see that the techniques

employed in D51 are only applicable to processors that are limited to

64K or less of program memory (most 8 bit processors).

Page 3

QUICK START

To disassemble a file called 'code.bin' or 'code.hex', type in (at

the DOS prompt):

copy generic.ctl code.ctl

to create a control file for the disassembly, and then type:

d51 code d

A control file is not necessary for disassembly, but as will be seen,

it will provide for a much more complete disassembly.

If neither the 'b' option nor the 'h' option is specified, D51 will

first search for a hex file and, if it is not found, then search for

a bin file. D51 can be forced to look for only one or the other by

specifying the (b)inary or (h)ex option on the command line:

d51 code b d		for the file code.bin, or

d51 code h d		for the file code.hex

D51 will produce a file named 'code.d51' containing the disassembly of

the 8051/8052 instructions in the original hex or binary file. It is

very useful on the first few disassemblies to use the 'd' option, which

adds a comment field to every line. The comment field contains the

hexadecimal address of each instruction, the hex data for each byte in

the instruction, and the ascii code for each byte of the instruction.

Example:

(opcode) (operands)	(address) (data)   (ascii)

orl	a,#42h		; 0df6   44 42      DB

This makes it easy to spot areas of ascii text or other nonexecutable

code, and then modify the control file to tell D51 that these areas

should be treated as some other kind of data on subsequent disassem-

blies. Please see the section on control file directives for specific

information.

By examining the output file and then modifying the control file based

on the information obtained, you can get a progressively better disas-

sembly of your 8051 program. Generally, about half a dozen iterations

of the disassembly are enough to produce a very readable source file

that can then be modified and reassembled.

Page 4

COMMAND LINE OPTIONS

Command line options modify the operation of D51. They may be

specified on the command line using either Unix style options:

d51 -db filename

or MSDOS style options:

d51 /d /b filename

Options may be entered in a freeform fashion (in any order or format)

provided that any options preceeding the filename must begin with

either a dash (-) or a slash (/). For example:

d51 /db filename -x1000

d51 filename -d b x1000

d51 -d filename bx1000

all mean exactly the same thing and will cause D51 to behave in

exactly the same way.

Command line options in Version 2 are:

A OPTION (ascii macro):

The A option tells D51 to change the db/defb pseudo op to the

ascii macro for areas defined as ascii text. This will cause D51

to include a macro definition in the disassembly output file that

should expand to 'db' for the cross assembler. To use this option,

your cross assembler must be able to handle macros, and in parti-

cular, D51's macro format. If used in conjunction with the S option

(see below), the S option must come first on the command line.

There are some caveats regarding the ascii macro. The intent of

this macro is to make ascii text easier to find in the output file.

Once you're satisfied with the readability of the output file, you

may want to run D51 once more without the A option to produce a

file more digestible by your cross assembler.

Some cross assemblers treat certain characters within a macro as

special meta-characters. The Huntsville Microsystems HMA8051, for

example, regards the characters '#', '.' and '\' as special with-

in a macro. If these characters occur in ascii text using the

ascii macro of D51, the assembler will do strange and not so

wonderful things with them. If this happens, just do not use the

A option (by default, it is off).

A problem that will be common with all cross assemblers is that

D51 does not currently treat the single quote (') character

Page 5

(is this still a problem?)

differently than it does other characters within ascii text.

Since D51 delineates ascii text (whether using the A option or

not) with the single quote character, its presence in ascii text

areas will cause problems. The only solutions to this right now are

to manually edit the output file of D51 or to define the byte of

the single quote as a binary byte in the control file (see the B

control file directive).

B OPTION (binary file):

The B option forces D51 to read a binary file (extention .bin)

instead of an Intel hex file (extention .hex). If neither the

B nor the H option are specified, D51 will first search for a

file with an extention of .hex, and if not found, will then

search for a binary file with an extention of .bin.

D OPTION (data in comment field):

The D option tells D51 to include a comment field in each

disassembled line. The comment field will contain the hexadecimal

address of each instruction, the hex data for each byte in the

instruction, and the ascii code for each byte of the instruction.

Example:

(opcode) (operands)	(address) (data)   (ascii)

orl	a,#42h		; 0df6   44 42      DB

This makes it easy to spot areas of ascii text or other non-

executable code, and then modify the control file to tell D51

that these areas should be treated as some other kind of data on

subsequent disassemblies. Please see the section on control file

directives for specific information.

H OPTION (hexadecimal file):

The H option forces D51 to read an Intel hex file (extention

.hex) instead of a binary file (extention .bin). If neither the

B nor the H option are specified, D51 will first search for a

file with an extention of .hex, and if not found, will then

search for a binary file with an extention of .bin.

S OPTION (string pseudo op):

This option tells D51 to generate the pseudo ops 'defb' and

'defw' instead of 'db' and 'dw' for those cross assemblers that

may require this. Most 8051 cross assemblers understand 'db'

and 'dw', so these are the defaults for D51. Use this option

for cross assemblers that do not recognize 'db' and 'dw'. If

used in conjunction with the A option, the S option must preceed

the A option on the command line.

Page 6

X OPTION (hexadecimal offset):

This option causes D51 to add a hexadecimal offset to the address

of every location in the code file. This may be useful for dis-

assembling code that was read from the second (or subsequent) rom of

a set of roms. For example, suppose that binary data was obtained

from a set of 4K byte eproms (call them rom1 and rom2). Then the

first file should be disassembled with the command line:

d51 -d rom1

and the second should be disassembled with the X option:

d51 -d rom2 x1000

The two files then can be combined into one and all references

within the disassembled code will be to the correct addresses.

Page 7

CONTROL FILE DIRECTIVES

Control directives tell D51 to treat specified areas of the code

to be disassembled as something other than executable code. The

user specifies control directives by editing (with any text editor)

a control file with the same name as the file to be disassembled, but

with a file extention of .ctl (eg: program.ctl to control the dis-

assembly of the file program.hex or program.bin). A generic control

file called 'generic.ctl' is provided to get the user started. Copy

'generic.ctl' to 'file.ctl' where 'file' is the name of the binary

or Intel hex file to be disassembled, and modify as required based

on the output of D51. Then run D51 again to get a more readable

disassembly of the code.

All directives consist of a character that specifies the type of data,

followed by a hexadecimal number specifying the value or range of

values of the data. In the case of a label or symbol directive, the

hexadecimal number is followed by ascii text defining the label or

symbol. See individual directives for examples. All directives must

be in the first column of the line. Everything following a semi-colon

(;) is a comment and will be ignored by D51.

Label and symbol directives differ in that a label refers to an

address and a symbol refers to any 8 or 16 bit data. Therefore an

opcode such as

mov	r2,#data

will cause D51 to search only the symbol table for a matching value

whereas an opcode such as

mov	dptr,#data

will cause D51 to search the label table first, and if no matching

value is found, to then search the symbol table.

Single values are specified by entering just the value; ranges of

values are specified by a start and stop address separated by a dash

(-), or by a start address and count separated by a plus (+), ie:

a 1000		tells D51 that the data at address 1000H is ascii

data.

a 1000-1010	tells D51 that the data from address 1000H to

(and including) address 1010H is ascii data.

a 1000+6	ascii text starting at 1000H for 6 bytes.

Page 8

Directives are processed in the order in which they are read from

the control file. Specifying an area as ascii data and then later

specifying it as data to be ignored (uninitialized data) will cause

that area to not be disassembled at all. Care must be taken in

specifying areas in the control file.

A DIRECTIVE (address data):

Specifies that the address contains a word value corresponding

to an address for which a label should be generated. For example,

a vector table may be located at address 0x1000 containing four

entries. The user can modify the control file by adding the

following entry:

a 1000-1007

This will cause D51 to generate the following output lines:

dw	vec1

dw	vec2

dw	vec3

dw	vec4

assuming that the control file also contains entries for the

values found at those addresses to generate the labels vec1

through vec4. The addresses referenced will be flagged so

that labels will be generated in the output file at those

addresses.

B DIRECTIVE (byte binary data):

Specifies that the data is to be interpreted as 8 bit binary

data. For example:

b 1000-1007

tells D51 that the data from address 1000H to 1007H should

generate the line:

db	0,1,2,3,4,5,6,7

assuming that the data at address 1000H and up is 00H, 01H, etc.

C DIRECTIVE (code data):

Forces D51 to interpret the data as executable code. This may be

necessary because D51 skips over strings of 00H or ffH bytes that

occur since they are unlikely to be real code. However, sometimes

programmers insert several NOPs for timing purposes.

Page 9

F DIRECTIVE (SFR label definition):

Causes reference to an SFR (special function register) to

generate a label for the specified SFR address. This is useful

for naming SFR's for non-standard members of the 8051 family.

For example, suppose that some manufacturer designs a new 8051

variant and decides to use address efH as a new special function

register called 'NEWREG'. Then by adding:

f ef newreg

to the control file, D51 will disassemble the opcodes f5 ef as:

mov	newreg,a

instead of:

mov	0efh,a

SFR label names are limited to six characters.

I DIRECTIVE (ignore data):

Tells D51 to ignore a range of addresses that may be initialized

by the input file. This is useful when the input file is a binary

file generated by an eprom programmer that dumps the entire eprom

space. The valid data from a 4K eprom might only be, say, 3K in

length. By adding

i c00-fff

to the control file, you tell D51 to not disassemble the data

from address 0c00H to 0fffH.

K DIRECTIVE (SFR bit definition):

As with the F directive, you can give a name to a new SFR bit.

If a new variant of the 8051 family adds a new interrupt, for

instance, its enable bit in the IE register might be bit 6. By

adding:

k ae 6 newint	(ae is the address of IE.6)

you define bit 6 of the interrupt enable register to be

ie.newint

Like SFR label names, SFR bit names are limited to six characters.

Page 10

L DIRECTIVE (label):

Defines a label to D51. Labels are generated in the output

disassembly file whenever a reference to the address is found

and the label exists in the label table. Suppose that address

0000H contains the code 01 43 (ajmp 0043H). Then the entry

l 43 start

in the control file will cause D51 to disassemble this code as:

ajmp	start

rather than:

ajmp	X0043

The code beginning at address 43H will then have the label

'start' in the label field rather than the label 'X0043'. See

the entry for the S directive for an explanation of the diff-

erence between labels and symbols.

R DIRECTIVE (register definition):

This directive specifies a name for an 8051 register. Direct

accesses to registers will normally be disassembled as:

mov	rb2r5,a

in the case of register 5 of register bank 2. If the user

determines that register 5 of bank 2 is being used as a loop

counter, for example, he can give it the name loop by adding:

r 15 loop

to the control file, and the above code will be disassembled as:

mov	loop,a

S DIRECTIVE (symbol):

Defines a symbol to D51. Symbols are output to the disassembly

file whenever the value is encountered in the input file and the

symbol exists in the symbol table. The user can specify a symbol

for a value by a line such as:

s 20 space

Code which uses this value, such as:

mov	a,#20h

Page 11

will then be disassembled as:

mov	a,#space

Labels and symbols differ in that a label applies to an address,

whereas a symbol applies to any 8 or 16 bit data. For code which

can only refer to values, such as

mov	a,#20h

only the symbol table will be searched for a matching value. For

code which could refer to either an address or a value, such as:

mov	dptr,#1234h

the label table will be searched first, and only if no matching

value is found will the symbol table then be searched.

T DIRECTIVE (text [ascii] data):

Forces the data in the range specified to be disassembled as

text (ascii data). Thus the code: 48 69 20 74 68 65 72 65 at

address 1000H will be disassembled as:

db	'Hi there'

or

ascii	'Hi there'	(command line option A used)

by putting the line:

t 1000-1007

in the control file. Otherwise it would be interpreted as:

orl	a,r0		; 1000   48	     H

xrl	a,r1		; 1001   69	     i

jb	2eh.4,X106d	; 1002   20 74 68    th

xrl	a,72h		; 1005   65 72      er

xrl	a,t3		; 1007   65 ff      e.

which is unlikely to be what was intended by the original

programmer. Data specified by this directive is not checked

to verify that it really is ascii data--be careful.

Page 12

W DIRECTIVE (word binary data):

Specifies that the data is to be interpreted as 16 bit binary

data. For example:

w 1000-1007

tells D51 that the data from address 1000H to 1007H should

generate the lines:

dw	0

dw	1

dw	2

dw	3

assuming that the data at address 1000H and up is 00H, 00H,

00H, 01H, 00H, 02H, etc. This differs from the A (address)

directive in that the A directive also causes a reference

to be made to the indirect address. In other words, if the

data at address 1000H is 1234H, the A directive will cause

a label (X1234) to be put in the output file at address

1234H, whereas the W directive will not.

Page 13

FORMAT OF OUTPUT FILE

The output file (extention .d51) contains 8051 instructions and

pseudo ops in a format that should be acceptable to any 8051

cross assembler. When assembled, the source file should produce

an Intel hex file or binary file that is identical to the original.

Identical, in this case, means that the code is the same, not that

it necessarily has the same format. The number of bytes in each

line of the hex file may differ from the original depending on

whether the original was produced by an assembler or was read from

an eprom.

Each output line is in the following format:

LABEL:	OPCODE	OPERANDS	; COMMENT

The label field will be generated only if the address of the code

is referenced by some other code (a call, for instance), or if the

user has specified a label for that address in the control file.

The comment field is only generated if the user specifies the D

option on the command line. It begins with a semicolon and is

followed by the hexadecimal address of the instruction, the hex

value of the byte(s) for the instruction, and finally by the ascii

representation of the instruction byte(s). For example:

X004f:	orl	p3,#0ffh	; 004f   43 b0 ff   C0.

label	opcode	operands	comment

Several things should be noted in this example. The label is 'X'

followed by the address of the instruction in four hex digits.

This type of label is created by D51 when some other code refer-

ences the address. If the user specifies a label name for the

address in the control file ('what' for example), the generated

code becomes:

what:	orl	p3,#0ffh	; 004f   43 b0 ff   C0.

If the user were to specify the symbol 'ones' for the value 0ffH,

the output would then become:

what:	orl	p3,#ones	; 004f   43 b0 ff   C0.

Also note the ascii representation of the instruction bytes. Bit

7 is forced to zero so that ascii with the high bit set can be

viewed (b0H in the example becomes 30H, which is ascii '0'). Any

code which is not printable ascii (a control code, for example)

is output as a period. This is the case for the byte 0ffH above.

Page 14

The comment field starts at a different position in the output

line depending on the type of data in the input file. For normal

executable code, the comment field starts in column 32; for hex

data it starts at column 49, and for ascii data it starts in

column 56. For many programs, this makes for a more readable

source file, but if there is a mix of data types in a short range

of addresses, this can lead to some strange looking formatting.

The user may want to correct this with a text editor in the final

disassembly before modification and reassembly.

Page 15

BACKGROUND

A disassembler is a program that reads machine code from a binary

or hexadecimal file and generates an assembly language source code

file. You may want to do this to recover source code which has been

lost, or to modify the code in equipment which uses an 8051 processor

or just for the educational value of finding out how something works.

The source file generated by the disassembler should be compatible

with a cross assembler that could then be used to create 8051 machine

code identical with the original (except for modifications the user

may wish to make).

D51 reads the entire 8051/8052 program from the hex or binary file into

memory, then reads the control file to flag any data the user specifies

as something other than executable code. When this is done, D51 scans

the data array containing the 8051 code three times. On the first pass,

it looks for code such as calls or jumps that reference other code in

the array. When such code is found, the referenced address is flagged

so that a label can be output to the source file in the second pass.

The second pass creates the output file (extention .d51) and writes

the assembly source code lines. See the section titled 'Technical Info'

for a description of this process. The third pass through the code

generates equate statements at the end of the file for anything that

requires it. This includes labels and symbols defined in the control

file, addresses that were referenced but are not the first byte of an

instruction or are not within the program space read from the input

file, and miscellaneous data values for which no symbol or label has

been specified in the control file.

On the first attempt to disassemble your code, use the generic control

file GENERIC.CTL by copying it to CODE.CTL (where CODE is the name of

the binary or hex file to be disassembled). After examining the source

code generated by D51, modify the control file with any ascii text

editor to force D51 to interpret such areas as executable code (or

tables of addresses or whatever). Then re-run D51 for a more complete

disassembly. Repeat this process as many times as necessary to achieve

an acceptable level of disassembly (please see the documentation for

control file directives).

Page 16

TECHNICAL INFO

D51 Disassembly Techniques:

D51 is a table driven disassembler. The value of each 8 bit opcode to

be disassembled is used as an index into two tables. The mnemonic table

('mnemtbl' in the file D51.TBL) contains ascii text entries for the

mnemonic for each opcode. The option table ('opttbl') contains 8 bit

flags which tell D51 what type of opcode is being disassembled so that

it can generate proper operands.

D51 first analyzes the command line to get the name of the file to be

disassembled and any command line options. After finding and opening

the file, the program allocates two 64K arrays: one to hold the program

to be disassembled and one for disassembly control flags. Both arrays

are initialized with codes that indicate invalid data. The input file

is then read and the data stored in the program array. As the data is

stored, the corresponding byte in the control array is set to indicate

valid program data.

Once the program is in memory, D51 looks for a control file with the

same name as the program file (but with an extension of .ctl). If a

control file is found, it is read and the information is used to modify

the control array flags to indicate what type of data is in the area

of memory specified in the control file directives. The control file

can also specify label and symbol names, among other things.

D51 then executes three passes over the program data. The first pass

looks for executable code that references some other address (calls

and jumps, for example.) The referenced address is then flagged (in

the control flag array) so that a label will be generated in the

output file during the second pass.

The second pass does the real work of the disassembler. Pass 2 scans

the program array and checks to see if the current location contains

executable code or some other type of data (as specified by the

control array flags.) If the current location contains some type of

data that is not executable code, the control flags determine what

action will be taken. If the control flags specify binary byte or

ascii data, the data is temporarily stored in a buffer so that multiple

bytes can be output on one line, rather than having a separate line with

a DB pseudo op for each byte of data. Word and address data, however,

do generate a separate line for each entry.

If the current location contains executable code, D51 outputs the

mnemonic for the instruction (by using the opcode as an index into

the mnemonic table), and then uses the option table flags indexed by

the opcode to determine how to generate appropriate operands. During

this pass, D51 keeps track of any direct accesses to registers and

outputs equates for their names at the end of the pass.

The third and last pass does some clean up work--searching the array

for references to un-initialized data or references to addresses not

at the beginning of multi-byte opcodes so that equate statements can

be generated. This pass also outputs equate statements for the entries

in the symbol table.

Page 17

D51 Source Code:

The source code for D51 is contained in five files:

D51.H		Header file defining constants, program flag

values and data structures.

D51.TBL		Tables for 8051 mnemonics, SFR names, SFR bit

names, register names and option flags.

D51.C		Main function that analyzes the command line,

finds and opens the disassembly file, allocates

memory for data arrays and calls the disassembly

functions in d51p1.c and d51p2.c.

D51P1.C		Contains 'pass0', which is not really a pass but

reads the control file and modifies the flag array

accordingly, and 'pass1' which searches for

address references. This file also has routines

relating to the label and symbol tables.

D51P2.C		Main disassembly routine that generates the output

file (pass 2). Also contains 'pass3' which generates

equates for unresolved references and for the

symbol table entries.

Each of the C files contains prototypes for the routines in the file.

If your compiler doesn't support prototypes, it's time to get a new

compiler.

The header file (D51.H) defines constants for exit codes, program flags

and various limits. It also contains structure definitions for symbol

and label table entries, mnemonic table entries and sfr names. The file

D51.TBL contains the mnemonic table, the option flag table, sfr name and

bit name tables and the register name table.

D51.C is the main disassembler file. The function main() parses the

command line for user options. It then attempts to open the input file

(bin or hex) and allocates memory for program and flag data. The input

file is then read into the program data array.  Main then calls various

routines in other files to perform the disassembly. D51.C also has the

following routines:

atox()		Converts ascii hex to binary hex data.

getcode()	Extracts data from ascii hex file and puts it in

the 64K program array.

find_entry()	Searches for an entry in the symbol or label table.

get_smem()	Allocates memory for a new symbol or label entry

and links it into a singly linked list.

add_entry()	Adds a new entry into the symbol or label list.

D51P1.C contains pass0(), which isn't really a disassembly pass, but

reads the control file (if any) and sets up the program flag array

Page 18

according to the directives found in the control file. The label and

symbol tables are built from data found in the control file. Pass1()

scans through the program array and, if the program flags define the

data as executable code, determines if the code references a program

location (jumps, calls, etc). Any such references set the program

flags for the referenced address. This is necessary so that pass 2 can

generate labels for the referenced addresses. D51P1.C also has routines

to dump accumulated data for db pseudo ops (ascii and binary).

D51P2.C is the real workhorse of the disassembler. Pass2() generates

the output assembly source file by scanning through the program array

and disassembling the code based on the program flag array info. If

the program flag specifies that the data is code, the opcode byte is

used as an index into two arrays: the mnemonic table, which contains

ascii text for the opcode, and the option table, which has bit flags

to tell D51 what type of operands are involved. It is this table

driven feature that makes D51 a fast disassembler. This technique,

however, limits any disassembler based on it to processors that have,

at most, 64K of program space. It would not be applicable to, for

example, a 680x0 processor, which can have up to 4 Gbytes of program

space (unless you have a computer that can handle 4 Gbyte arrays). If

the program flags specify that the data is ascii or binary data, the

data is saved in a separate array so that many bytes can be output

with one db pseudo op. When pass 2 is complete, pass3() rescans the

data array looking for references that require equate pseudo ops. Pass

3 also outputs equates for symbol and label table entries.

Further details of D51's implementation can be found by examing the

source files.

Page 19

SUMMARY

D51 V2 is being distributed under the GNU General Public License.

Complete source code for D51 MUST accompany any distribution of this

software. You are free to modify it in any way you see fit, but you

are NOT free to market it as a commercial product (except for reason-

able fees for media or other reproduction and/or distribution costs).

See the file COPYING for details.

D51 V2 is an 8051/8052 disassembler that allows the user to specify

how code should be disassembled via a control file that defines given

areas of the code as ascii text, binary data or other types of non-

executable code. By an iterative process of disassembly, control file

modification, and further disassembly, the user can obtain a very

readable source file that can then be modified and re-assembled by an

8051 cross assembler. Several public domain 8051 cross assemblers are

available.

I've encountered several problems when reassembling the output of D51.

Almost all are due to the cross assembler not recognizing SFRs of some

of the newer or more exotic 8051 variants. The D51 tables include

names for all SFRs that I'm aware of as of May 1995 (there are likely

some that I'm not aware of). Such errors might be corrected just by

defining a symbol for the SFR in the control file (see the F directive),

but this depends on whether your cross assembler will accept it as a

valid operand. If all else fails, replace the offending instruction

with a 'db' pseudo op and put the mnemonic and operand in a comment.

A similar problem is the assembler not being able to handle a bit

addressable SFR address; the only solution I see to this is to use a

'db' pseudo op. All other problems I've seen (not many) have been due

to disassembling data as code; this is easily corrected by modifying

the control file to specify the correct data type.

D51 V2 was compiled using Borland's Turbo C V2.0 (compact memory model)

and is intended to be run on MSDOS machines. No attempt was made to

make it portable to either other compilers or other operating systems.

The source code for D51 should require little or no modification to be

compatible with other compilers for MSDOS, but may require some

significant changes to run on other operating systems. The source code

was created using Borland's Brief V3.1 and may require reformatting for

other editors (see the file TAB.DOC).

Page 20

Command line options:

A - Change db/defb pseudo op to 'ascii' macro for text

B - Force read of binary file (.bin extention)

D - Add address and data info in comment field

H - Force read of Intel hex file (.hex extention)

S - Change db and dw pseudo ops to defb and defw

X - Add hexadecimal offset to all program locations

Control file directives:

A - Specifies that words in program space are addresses

B - Data is 8 bit binary

C - Program space is executable code

F - Specify SFR label

I - Ignore data (uninitialize program space)

K - Specify label for SFR bit

L - Define label

R - Define register name

S - Define symbol

T - Specify ascii text

W - Data is 16 bit binary

Page 21

8051 REGISTER MAPS

Addresses of 8051 registers:

Register Bank   R0  R1  R2  R3  R4  R5  R6  R7

----------------------------------------------

0           00  01  02  03  04  05  06  07

1           08  09  0A  0B  0C  0D  0E  0F

2           10  11  12  13  14  15  16  17

3           18  19  1A  1B  1C  1D  1E  1F

Addresses of 8051 Special Function Registers:

Address Symbol Name

----------------------------

80   P0     Port 0

81   SP     Stack Pointer

82   DPL    Data Pointer (low byte)

83   DPH    Data Pointer (high byte)

87   PCON   Power Control

88   TCON   Timer/Counter Control

89   TMOD   Timer/Counter Mode

8A   TL0    Timer/Counter 0 (low byte)

8B   TL1    Timer/Counter 1 (low byte)

8C   TH0    Timer/Counter 0 (high byte)

8D   TH1    Timer/Counter 1 (high byte)

90   P1     Port 1

98   SCON   Serial Port Control

99   SBUF   Serial Data Buffer

A0   P2     Port 2

A8   IE     Interrupt Enable

B0   P3     Port 3

B8   IP     Interrupt Priority

D0   PSW    Program Status Word

E0   ACC    Accumulator

F0   B      B Register

Bit Addressable Special Function Registers:

Address Symbol SFR bit Name

-------------------------------

80-87       P0.0-7  Port 0 Bits

88   IT0    TCON.0  External Interrupt 0 Type Control

89   IE0    TCON.1  External Interrupt 0 Edge Control

8A   IT1    TCON.2  External Interrupt 1 Type Control

8B   IE1    TCON.3  External Interrupt 1 Edge Control

8C   TR0    TCON.4  Timer 0 Run Control

8D   TF0    TCON.5  Timer 0 Overflow Flag

8E   TR1    TCON.6  Timer 1 Run Control

8F   TF1    TCON.7  Timer 1 Overflow Flag

90-97       P1.0-7  Port 1 Bits

98   RI     SCON.0  Serial Receive Interrupt Flag

99   TI     SCON.1  Serial Transmit Interrupt Flag

9A   RB8    SCON.2  Serial Receive Bit 8

9B   TB8    SCON.3  Serial Transmit Bit 8

9C   REN    SCON.4  Serial Receive Enable

9D   SM2    SCON.5  Serial Port Mode Specifier

9E   SM1    SCON.6  Serial Port Mode Specifier

9F   SM0    SCON.7  Serial Port Mode Specifier

A0-A7       P2.0-7  Port 2 Bits

A8   EX0    IE.0    Enable External Interrupt 0

A9   ET0    IE.1    Enable Timer 0 Interrupt

AA   EX1    IE.2    Enable External Interrupt 1

AB   ET1    IE.3    Enable Timer 1 Interrupt

AC   ES     IE.4    Enable Serial Port Interrupt

AF   EA     IE.7    Enable Interrupts

B0-B7       P3.0-7  Port 3 Bits

B8   PX0    IP.0    External Interrupt 0 Priority

B9   PT0    IP.1    Timer 0 Interrupt Priority

BA   PX1    IP.2    External Interrupt 1 Priority

BB   PT1    IP.3    Timer 1 Interrupt Priority

BC   PS     IP.4    Serial Port Interrupt Priority

D0   P      PSW.0   Parity

D2   OV     PSW.2   Overflow Flag

D3   RS0    PSW.3   Register Bank Select Bit 0

D4   RS1    PSW.4   Register Bank Select Bit 1

D5   F0     PSW.5   Flag 0 (general purpose flag)

D6   AC     PSW.6   Auxiliary Carry Flag

D7   CY     PSW.7   Carry Flag

E0-E7       ACC.0-7 Accumulator Bits

F0-F7       B.0-7   B Register Bits