Software DMA Implementation

Introduction

A DMA (Direct Memory Access) allows fast data transfers between memories without interrupting the CPU. The transfer is activated by an external event. The control signals and the address and data buses are managed by the DMA, while any external accesses to the CPU are frozen.

The AT91M40400 device does not feature a DMA as a peripheral. However, it can be implemented by software by using the load multiple (ldm) and store multiple (stm) instructions. This implementation avoids a decrease in device performance by reducing the number of instruction fetches.

This application note purposes two different ways to implement a software DMA using the ARM Fast Interrupt (FIQ) as the external event. The code has been designed to reduce the number of instruction fetches as much as possible.

Theory of Operation

The principle of the DMA is that an external event causes a fast memory copy. In this case external event is the Fast Interrupt pin (FIQ) since the FIQ mode has banked registers (r8 to r14) which do not need to be saved before being used in FIQ mode. These registers are initialized before validating the FIQ interrupt, then directly used when the external event occurs.

This application note describes two ways to implement a software DMA. Both implementations make the transfer by splitting the block into packets of 4 words.

Note: In this document 1 packet = 4 words;1 word = 4 bytes

" 	DMA Turbo: Copies one packet (4 words) from a source buffer to a destination buffer at each FIQ interrupt.

" 	DMA Channel: Copies the number of packets (4 words) defined by the user from a source buffer to a destination buffer at each FIQ interrupt, thus freeing the core between packets.

As the FIQ interrupt is used, the DMA must be able to access the banked register. This can only be done by switching to FIQ mode and only if the CPU is in privileged state (any mode except User Mode). This application note assumes that the user function calls are performed while the CPU is in privileged state. It does not describe how to switch from unprivileged to privileged state. This can be done on another level by using the “swi” instruction of the ARM core or the Advanced Interrupt Controller features allowing to force an interrupt entry. In this last case, FIQ or AIC’s software interrupt can be forced to set up software DMA.

Table 1.  	Source Files

| Files | Contents |
| --- | --- |
| dma _turb.s | Code source for DMA Turbo |
| dma_chnl.s | Code source for DMA channel |
| dma.h | DMA prototypes and structures for application use |
| arm.inc | ARM core definition |

DMA Turbo

The DMA Turbo allows 1 packet (4 words) to be transferred at each FIQ interrupt from a source buffer to a destination buffer. Source and destination buffer pointers are post-incremented after each transfer.

The number of packets to be transferred per FIQ is fixed and cannot be dynamically modified. This can cause the memory to be overwritten. The application must take this into account.

The main advantage to this implementation is that no registers need to be saved. It is also a faster way to transfer data than the DMA Channel implementation.

Table 1.  	Registers Used

| Register | Parameter |
| --- | --- |
| r14 | Link register – return address |
| r13 | ‘dest_pt’ = Destination buffer pointer |
| r12 | ‘src_pt’ = Source buffer pointer |
| r8 to r11 | Transfer registers |

User Functions

The functions used to manage the DMA Turbo are defined in dma_turb.c:

Table 1.  	DMA Turbo User Functions

| void DmaTurboSet (int *source, int *destination) | Initialize the FIQ registers for the DMA transfer |
| --- | --- |
| int DmaTurboGet (int *source, int size) | Check the end of the transfer |
| void DmaTurboFiq (void) | FIQ treatment and DMA transfer |

The DmaTurboSet function initializes the FIQ banked registers for the DMA Turbo transfer, then enables the DMA transfer by clearing bit F in the CPSR.

void DmaTurboSet (int *source, int *destination)

source	the source buffer pointer

destination 	the destination buffer pointer

Begin

| Switch to FIQ mode and save entry  mode

| Memorize source buffer pointer (r12_fiq = ‘src_pt’)

| Memorize destination buffer pointer (r13_fiq = ‘dest_pt’)

| Enable DMA (Clear bit F)

| Restore entry  mode

End

The DmaTurboGet function checks that the end of the transfer occurred (‘size’ words received), and, if it did, disables the DMA transfer by setting bit F in the CPSR.

int DmaTurboGet (int * source, int size) returns the number of words already received

source	the source buffer pointer at beginning

size 	the number of words to be copied

Begin

| Switch to FIQ mode and save entry  mode

| Calculate number of transferred words

| Disable DMA (Clear bit F) if number of transferred words >= size

| Restore entry  mode

End

The DmaTurboFiq function is activated by the FIQ (vector address 0x1C), and copies 4 words from source buffer to destination buffer.

void DmaTurboFiq (void)

Begin

| *dest_pt <- *src_pt (multiple load and store 4 words with post increment)

End

Alternate Implementation

The DMA Turbo can be modified to transfer more than 4 words per FIQ. For this, the pair of instructions ‘ldm’ and ‘stm’ (refer to the “ARM Architectural Reference Manual”) must be repeated as many times as necessary. Different types of source and destination buffers can be supported by using the options of post/pre increment/decrement of these instructions.

It can also be modified to make the number of words to transfer defined by the caller. This has the advantage of allowing the dynamic transfer size, but one register must be reserved for the count (e.g. r11). Therefore, only 3 registers are available for the transfer itself.

DMA Channel

Copies a block by splitting it in several set of packets. At each FIQ interrupt, a number of packets (4 words) defined by the user is transfered until the remaining number of bytes is less than 1 packet (4 words). These remaining bytes are transferred at the following FIQ interrupt, and the DMA is stopped by disabling the FIQ.

One inconvenience of this implementation is that non-banked registers are used. Therefore, it is mandatory to save these registers before the transfer, and to restore them afterwards.

The main advantage is that the size of the transfer is controlled, and there is no risk of overwriting the memory.

The block is copied from the source to the destination by boundaries of packets. When no more whole packets can be transferred, the remaining bytes (less than 16) are transferred in a way which optimizes the number of fetches. Each one of following transfers are performed only if the remaining bytes number needs it:

- 2 words (8 to 15 remaining bytes), then

- 1 word (4 to 7 remaining bytes), then

- 1 half-word (2 or 3 remaining bytes), then

- 1 byte (1 byte remaining)

The user can choose to increase, after each FIQ copy, the source pointer, the destination pointer, both pointers or neither.

Table 1.  	Registers Used

| Register | Parameter |
| --- | --- |
| r14 | Link register - return address |
| r13 | Temporary buffer address |
| r12 | Increment mask |
| r11 | Number of packets to copy per FIQ |
| r10 | Block size (byte number) |
| r9 | Destination address |
| r8 | Source address |
| r6 to r7 | Working registers (must be saved and restored) |
| r0 to r3 | Transfer registers (must be saved and restored) |

Type Definition

The DMA channel parameters are set through a structure named ‘dma_descriptor’. This type is defined in dma.h:

typedef struct dma_descriptor

{

u_char		*src;			// Source pointer

u_char		*dest;			// Destination pointer

u_int		nb_bytes;		// Total number of bytes to copy

u_int		nb_packets;	// Maximum number of packets to copy at each FIQ

u_int		dma_mask;		// DMA channel mask (INC_SRC and/or INC_DEST)

} dma_descriptor;

The user can choose whether or not to increase the source/destination pointers using the ‘dma_mask’ field, the value of which is an OR-combination of the constants INC_SRC and INC_DEST (These constants are defined in dma.h).

User Functions

The functions used to manage the DMA Channel are defined in dma_chnl.c:

Table 1.  	DMA Channel User Functions

| void DmaChannelSet (dma_descriptor *dma_desc, int *buf) | Initialize the FIQ registers for the DMA transfer |
| --- | --- |
| int DmaChannelGet (void) | Get the count of remaining bytes to be transferred |
| void DmaChannelFiq (void) | FIQ treatment and DMA transfer |

The DmaChannelSet function sets and enables the DMA channel

void DmaChannelSet (dma_descriptor *dma_desc, int *buf);

dma_desc 	pointer to the DMA descriptor

buf	temporary buffer (7*4 bytes needed) to store non-banked used registers

Begin

| Switch to FIQ mode and save entry mode

| r13_fiq <- temporary buffer pointer

| Load banked registers from the DMA descriptor

| Set default number of packets to transfer (1) if not defined by user

| Enable DMA (Clear bit F) if number of bytes to transfer > 0

| Restore entry mode

End

The DmaChannelGet function returns the remaining number of bytes to be transferred

int DmaChannelGet(void) returns remaining number of bytes

Begin

| Switch to FIQ mode and save entry mode

| Return number of bytes remaining to be transferred

| Restore entry mode

End

The DmaChannelFiq function is activated by the FIQ (vector address 0x1C), and copies r11 x 4 words from the <source> to the <destination> if possible.

void DmaChannelFiq (void)

Begin

| Save used registers in the temporary buffer

| Save source and destination

| While (nb_packets > 0) and (nb_bytes >= PACKET_SIZE)

| | Update nb_bytes by sustracting PACKET_SIZE from it

| | *dest <- *src (multiple load and store 4 words with post increment)

| | Decrement nb_packets

| EndWhile

| If (nb_packets == 0) (i.e. All blocks tranfered)

| Then

| | === Copy last bytes to copy (< 16) ===

| | *dest <- *src

| | If needed, load and store multiple 2 words with post increment

| | If needed, load and store 1 word with post increment

| | If needed, load and store 1 half-word with post increment

| | If needed, load and store 1 byte with post increment

| | Clear nb_bytes

| | Disable DMA (SPSR.F <- 0)

| Endif

| Restore src (if not INC_SRC)

| Restore destination (if not INC_DEST)

| Restore used registers

End

Performances

Table 1.  	DMA Turbo

| Parameter | Value |
| --- | --- |
| Code Size | 20 words |
| Register Usage | R8 to R14 (banked FIQ registers) |
| Peripherals Usage | FIQ Interrupt |

Table 1.  	DMA Channel

| Parameter | Value |
| --- | --- |
| Code Size | 51 words |
| Register Usage | R8 to R14 (banked FIQ registers) |
| Peripherals Usage | FIQ Interrupt |

Tips and Warnings for Both Types of DMA Implementation

Pointer Alignment

Source and destination pointers must always be word-aligned because of the use of load and store multiple instructions. If this is not the case, a word-alignment must be performed, using an “AND” with the existing word in the memory to copy the first non-aligned bytes.

FIQ Rate

Since the DMA described in this application note performs the transfer of the packet(s) at the FIQ interrupt, it is necessary to pace the FIQ at a rate defined by the application.

This application note does not describe how to generate the rate, but it can easily be done by using a timer channel which is cyclically triggered to generate a tick. An FIQ is then generated on each tick. The FIQ can be generated by configuring source 0 to be edge triggered, and by setting bit 0 in the AIC_ISCR register to one (0x00000001). The port P12, multiplexed with FIQ, should be configured as a PIO in order to prevent the FIQ from being generated by an external event.

Another way is to configure the port P12 as a peripheral, and to make the external hardware generate the FIQ which can be configured to be edge or level triggered.

In any case, the source 0 (FIQ) of the AIC must be configured (register AIC_SMR0) and enabled (register AIC_IECR).

Tips and Warnings for DMA Channel Implementation

Temporary Buffer

This application does not implement a stack management to save the registers. For this reason, a temporary buffer has been chosen in order to save and restore the non-banked registers in the function DmaChannelFiq. Of course, this temporary buffer can also be performed by a stack management. In this case, the function DmaChannelSet needs only the DMA descriptor as a call parameter.

Pointer Incrementation

If the source and destination pointers are systematically incremented during the transfer, the command word with the pointer management mask ptrMask is useless. In this case, the saving and conditional restoring of the source and destination pointers is also useless and the registers R6 and R7 are unused in the function DmaChannelFiq. As the mask is not used, the register R12 is also unused in the function DmaChannelFiq and can be used as the packet counter (in place of R11 which must be restored).

This way only registers R0 to R3 in the function DmaChannelFiq must be saved, and the temporary buffer can be only 4- word-sized.

Large Sized Blocks

If the blocks to transfer are significant (more than 15 words), the use of the registers can be optimized by increasing the size of the temporary buffer. R6 and R7 are used only to save R8 and R9. This can be done in the temporary buffer, and whole non-banked registers (R0 and R7) can be used for the transfer in the function DmaChannelFiq (care must be taken however when restoring R8 and R9).

The temporary buffer becomes 11 words (if optional increment feature is used), but the packets transferred are 8-words (PACKET_SIZE).

It is necessary to add a 4-word conditional transfer before the 2-word transfer:

| | | *dest <- *src (load and store multiple 4 words with post

| | |    increment) if needed

This is coded by:

ands				r0, r10, #0x10

ldmneia			r8!, {r0-r3}

stmneia			r9!, {r0-r3}

AT91 
ARM Thumb Microcontrollers

Application Note