| Component | Quantity | Type | Equivalents |  | Component Description |
| --- | --- | --- | --- | --- | --- |
| IC CMOS Logic | 3 | 4015 | 74HC4015 | U6,U11,U14 | Dual 4 bit shift register |
|  | 1 | 4014 | 4021 | U3 | 8 stage shift register |
|  | 1 | 74HC04 | 74HC14 | U1 | Hex inverting buffer |
|  | 2 | 4503 | 74HC367, HEF40097 | U2,U4 | 3-state Hex buffer |
| Transistors | 3 | BC549 | BC548, BC547 | Q3,Q4,Q5 | NPN Epitaxial Switching Transistor |
|  | 1 | BC557 | BC556, | Q1 | PNP Epitaxial Switching Transistor |
|  | 1 | BC327 |  | Q2 | PNP Epitaxial Switching Transistor |
| Diode |  |  |  |  |  |
|  | 6 | 1n4148 |  | D1,D2,D8, D9,D10,D11 | 1N4000 Series Fast Switching Diode |
|  | 1 | 1n5819 | 1n5817-18 | D3 | Schottky barrier diodes |
|  | 1 | Zener 8.2 V 500mW |  | ZD1 | Silicon Z-diode, 8.2 v |
|  | 1 | 1n4002 | 1N4002..1N4007 | D4 – D7 | 1N4000 Series Plastic Rectifier Diode |
|  | 1 | Zener 3,9v |  | ZD2 | Silicon Z-diode, 3,9v |
| LEDs | 1 | 3 mm Red |  |  |  |
|  | 1 | 3 mm Yellow |  |  |  |
|  | 1 | 3 mm Green |  |  |  |
| IC Supply | 1 | 7805 |  | U9 | IC TO-220 voltage reg 5v |
|  | 1 | 7812 |  | U8 | IC TO-220 voltage reg 12v |
| Resistors | 9 | 10K |  | R1,2,5,13,14,18,19,22,23 |  |
|  | 8 | 2K2 |  | R4,6,7,9,10,11,20,21 |  |
|  | 2 | 100K |  | R3,15 |  |
|  | 2 | 1k2 |  | R8,17 |  |
|  | 1 | 4k7 |  | R12 |  |
|  | 1 | 56om |  | R16 |  |
|  | 1 | SIL 10k |  |  | CSC Single-In-Line 9-Pin |
| Capacitors | 6 | 100nF, 50v |  | C5 –C10 | Radial Capacitor, 100 nF, 50 VDC |
|  | 1 | 470uF, 35v | 1000uF, 35v | C4 | Low Voltage Axial d=10mm Electrolytic Capacitor |
|  | 2 | 33uF, 10v |  | C1,C2 |  |
|  | 1 | 47uF, 35v |  | C3 |  |
| IC Sockets - Dip | 6 | 16 pin (solder) |  |  |  |
|  | 1 | 14 pin |  |  |  |
|  | 1 | 32 pin bus | ARIES 32 pin ZIF | U5 |  |
|  | 3 | 8 pin (solder or bus) |  | U7,U10,U12 |  |
|  | 1 | 18 pin (solder or bus) |  | U13 |  |
| Parallel Connector | 1 | 25 pin |  |  | D-Sub, 25 pin ,male |
| Power Jack | 1 | Supply |  |  | power jack, NEB-1R |
| Dip Switch | 1 | 12 pin |  |  |  |
|  | 1 | 4 pin |  |  |  |
| Pin Headers | 1 | 2x4 pin |  |  |  |
|  | 2 | 2x3 pin |  |  |  |
|  | 1 | 3 pin |  |  |  |
|  | 1 | 1x8 pin |  |  |  |
|  | 1 | 1x5 pin |  |  |  |